EXPERIMENT 1

Blinking of LED using ESP32 Microcontroller

| Date: _________________________________ | Page No: ________ |
| --- | --- |

AIM

To write an Arduino program to blink an LED connected to ESP32 GPIO2 at 1 Hz frequency.

COMPONENTS REQUIRED

| Sl. | Component / Item | Specification | Qty |
| --- | --- | --- | --- |
| 1 | ESP32 Development Board | 38-pin/30-pin | 1 |
| 2 | LED (Red / Green) | 5 mm, 2V/20mA | 1 |
| 3 | Resistor | 220 Ohm, 0.25W | 1 |
| 4 | Breadboard | 840 tie-point | 1 |
| 5 | Jumper Wires | Male-Male | 3 |
| 6 | USB Cable | Type-A to Micro-USB | 1 |

THEORY

ESP32 is a dual-core 32-bit microcontroller with built-in Wi-Fi and Bluetooth. GPIO (General Purpose Input/Output) pins can be configured as digital outputs using Arduino IDE. The pinMode() function configures a pin as OUTPUT and digitalWrite() sets it HIGH (3.3 V) or LOW (0 V). A 220 Ohm current-limiting resistor is connected in series with the LED to restrict current to approximately 6 mA, preventing damage to the LED and the GPIO pin. The delay() function pauses program execution for a specified number of milliseconds, creating the 1 Hz blink (500 ms ON, 500 ms OFF).

PROCEDURE

1.  Connect the anode (+) of the LED to GPIO2 of the ESP32 through a 220 Ohm resistor.

2.  Connect the cathode (-) of the LED to the GND pin of the ESP32.

3.  Connect the ESP32 to the computer using a USB cable.

4.  Open Arduino IDE. Go to Tools > Board > select "ESP32 Dev Module". Select the correct COM port.

5.  Type the program code and click the Upload button.

6.  Wait for "Done uploading" message. Observe the LED blinking on the breadboard.

7.  Verify the blink rate is 1 Hz (ON for 500 ms, OFF for 500 ms).

RESULT

The LED connected to GPIO2 of the ESP32 blinked continuously at 1 Hz (500 ms ON, 500 ms OFF), confirming successful GPIO digital output control on the ESP32 microcontroller using Arduino IDE.

EXPERIMENT 2

Interfacing Push Button Switch with ESP32 for LED Control

| Date: _________________________________ | Page No: ________ |
| --- | --- |

AIM

To interface a push-button switch with ESP32 GPIO and control an LED based on switch input using Arduino IDE.

COMPONENTS REQUIRED

| Sl. | Component / Item | Specification | Qty |
| --- | --- | --- | --- |
| 1 | ESP32 Development Board | 38-pin/30-pin | 1 |
| 2 | LED (Red / Green) | 5 mm, 2V/20mA | 1 |
| 3 | Resistor | 220 Ohm, 0.25W | 1 |
| 4 | Push-Button Switch | Tactile, 6 mm | 1 |
| 5 | Breadboard | 840 tie-point | 1 |
| 6 | Jumper Wires | Male-Male | 6 |
| 7 | USB Cable | Type-A to Micro-USB | 1 |

THEORY

A push-button is a momentary digital input device. When pressed it connects the GPIO pin to GND (logic LOW); when released, the internal pull-up resistor keeps the pin at 3.3 V (logic HIGH). INPUT_PULLUP mode in Arduino enables ESP32's built-in pull-up resistor (~45 kOhm), eliminating the need for an external resistor. The digitalRead() function returns LOW when the button is pressed and HIGH when released. The LED output is controlled based on this button state.

PROCEDURE

1.  Connect the LED anode to GPIO2 through a 220 Ohm resistor; cathode to GND.

2.  Connect one terminal of the push-button to GPIO4 of the ESP32.

3.  Connect the other terminal of the push-button to GND.

4.  Connect the ESP32 to the computer using a USB cable.

5.  Open Arduino IDE. Select Board: "ESP32 Dev Module" and correct COM port.

6.  Type and upload the program code.

7.  Press and hold the push-button - observe the LED turning ON.

8.  Release the button - observe the LED turning OFF.

RESULT

The LED turned ON when the push-button was pressed (GPIO4 = LOW) and turned OFF when released (GPIO4 = HIGH), confirming successful GPIO digital input reading with internal pull-up resistor and output control on the ESP32.

EXPERIMENT 3

Interfacing 16x2 LCD Display with ESP32 using I2C Protocol

| Date: _________________________________ | Page No: ________ |
| --- | --- |

AIM

To interface a 16x2 LCD display (I2C) with ESP32 and display text and a live counter on both lines.

COMPONENTS REQUIRED

| Sl. | Component / Item | Specification | Qty |
| --- | --- | --- | --- |
| 1 | ESP32 Development Board | 38-pin/30-pin | 1 |
| 2 | 16x2 LCD with I2C Module | PCF8574 backpack, 0x27 | 1 |
| 3 | Breadboard | 840 tie-point | 1 |
| 4 | Jumper Wires | Male-Male | 4 |
| 5 | USB Cable | Type-A to Micro-USB | 1 |

THEORY

I2C (Inter-Integrated Circuit) is a two-wire synchronous serial protocol using SDA (Serial Data) and SCL (Serial Clock) lines. Multiple devices share the same bus, each identified by a unique 7-bit address. The 16x2 LCD is connected through a PCF8574 I2C expander module (address 0x27 or 0x3F), reducing the 8 data pins of a parallel LCD to just 2 wires. On ESP32, I2C uses GPIO21 (SDA) and GPIO22 (SCL). The LiquidCrystal_I2C library provides high-level functions (lcd.print(), lcd.setCursor()) to control the display without managing low-level I2C bit-banging.

PROCEDURE

1.  Connect VCC of the I2C LCD module to 3.3V (or 5V) pin of ESP32.

2.  Connect GND of the I2C LCD module to GND of ESP32.

3.  Connect SDA of the I2C LCD module to GPIO21 of ESP32.

4.  Connect SCL of the I2C LCD module to GPIO22 of ESP32.

5.  Open Arduino IDE. Go to Sketch > Include Library > Manage Libraries. Search and install "LiquidCrystal I2C" by Frank de Brabander.

6.  Select Board: "ESP32 Dev Module" and correct COM port.

7.  Type and upload the program code.

8.  Observe Line 1 displaying "JECC IoT Lab" and Line 2 showing an incrementing counter.

9.  If only black boxes appear, adjust the contrast potentiometer on the I2C module.

RESULT

The 16x2 LCD displayed "JECC " on Line 1 and an auto-incrementing counter on Line 2, confirming successful I2C communication between ESP32 and the LCD module using the LiquidCrystal_I2C library.

EXPERIMENT 4

Blinking of LED using Raspberry Pi GPIO

| Date: _________________________________ | Page No: ________ |
| --- | --- |

AIM

To write a Python program to blink an LED connected to Raspberry Pi GPIO17 (BCM) at 1 Hz frequency.

COMPONENTS REQUIRED

| Sl. | Component / Item | Specification | Qty |
| --- | --- | --- | --- |
| 1 | Raspberry Pi | 3B / 4B (any model) | 1 |
| 2 | LED (Red / Green) | 5 mm, 2V/20mA | 1 |
| 3 | Resistor | 220 Ohm, 0.25W | 1 |
| 4 | Breadboard | 840 tie-point | 1 |
| 5 | Jumper Wires | Female-Male | 3 |
| 6 | Power Supply | 5V 3A (USB-C/Micro-USB) | 1 |

THEORY

Raspberry Pi is a single-board computer running Raspberry Pi OS (Linux). Its 40-pin GPIO header provides programmable digital I/O pins. The RPi.GPIO Python library enables software control of GPIO pins. BCM (Broadcom) numbering refers to the GPIO channel numbers on the Broadcom chip. GPIO.setmode(GPIO.BCM) selects BCM numbering. GPIO.setup(pin, GPIO.OUT) configures a pin as output. GPIO.output(pin, GPIO.HIGH) sets the pin to 3.3 V (LED ON) and GPIO.LOW sets it to 0 V (LED OFF). time.sleep() introduces delays. GPIO.cleanup() resets all pins on program exit, preventing accidental pin damage during next run.

PROCEDURE

1.  Connect the LED anode (+) to GPIO17 (Physical Pin 11) through a 220 Ohm resistor.

2.  Connect the LED cathode (-) to GND (Physical Pin 6).

3.  Power on the Raspberry Pi and open the Terminal application.

4.  Create a new Python file: type   nano led_blink.py   and press Enter.

5.  Type the program code carefully. Save the file with Ctrl+X, then Y, then Enter.

6.  Run the program: type   python3 led_blink.py   and press Enter.

7.  Observe the LED blinking on the breadboard. Press Ctrl+C to stop the program.

8.  Note that GPIO.cleanup() is called automatically in the finally block on exit.

RESULT

The LED connected to GPIO17 of the Raspberry Pi blinked continuously at 1 Hz (500 ms ON, 500 ms OFF), confirming successful GPIO digital output control using Python RPi.GPIO library on the Raspberry Pi.

EXPERIMENT 5

Interfacing Push Button Switch with Raspberry Pi for LED Control

| Date: _________________________________ | Page No: ________ |
| --- | --- |

AIM

To interface a push-button switch with Raspberry Pi GPIO and control an LED based on switch input using Python.

COMPONENTS REQUIRED

| Sl. | Component / Item | Specification | Qty |
| --- | --- | --- | --- |
| 1 | Raspberry Pi | 3B / 4B (any model) | 1 |
| 2 | LED (Red / Green) | 5 mm, 2V/20mA | 1 |
| 3 | Resistor | 220 Ohm, 0.25W | 1 |
| 4 | Push-Button Switch | Tactile, 6 mm | 1 |
| 5 | Breadboard | 840 tie-point | 1 |
| 6 | Jumper Wires | Female-Male | 6 |
| 7 | Power Supply | 5V 3A (USB-C/Micro-USB) | 1 |

THEORY

GPIO pins on the Raspberry Pi can be configured as digital inputs. The pull_up_down=GPIO.PUD_UP parameter enables the internal pull-up resistor (approximately 50 kOhm), keeping the pin at logic HIGH (3.3 V) when the button is not pressed. When the button is pressed, it connects the pin to GND, pulling it to logic LOW (0 V). GPIO.input() reads the current pin state. This active-low configuration eliminates the need for an external resistor. A 50 ms debounce delay using time.sleep(0.05) prevents false triggering from mechanical switch bounce, which is a rapid fluctuation of the switch contact when it opens or closes.

PROCEDURE

1.  Connect the LED anode to GPIO17 (Physical Pin 11) through a 220 Ohm resistor; cathode to GND (Pin 6).

2.  Connect one terminal of the push-button to GPIO18 (Physical Pin 12).

3.  Connect the other terminal of the push-button to GND (Physical Pin 14).

4.  Power on the Raspberry Pi and open the Terminal application.

5.  Create a new Python file: type   nano switch_led.py   and press Enter.

6.  Type the program code. Save with Ctrl+X, then Y, then Enter.

7.  Run the program: type   python3 switch_led.py   and press Enter.

8.  Press and hold the push-button - observe the LED turning ON.

9.  Release the button - observe the LED turning OFF. Press Ctrl+C to stop.

RESULT

The LED turned ON when the push-button was pressed (GPIO18 = LOW) and turned OFF when released (GPIO18 = HIGH), confirming successful GPIO digital input reading with internal pull-up resistor on the Raspberry Pi using Python.