MODULE 2 — STUDENT STUDY NOTES

Internet of Things & Applications  |  PEMRT525

KTU  |  S5  |  MR 2K24  |  JECC  |  Sources: Bahga & Madisetti Ch.3  +  Cisco Press IoT Fundamentals Ch.3 & 4

How to use: Read each topic in full sentences — understand the idea, look at the figure, then cover and recall. Use the 📖 boxes for quick definitions before exams. Use 📝 EXAM TIP boxes to know exactly how to answer questions.

MODULE 2 SYLLABUS (9 Hours)

IoT and M2M — M2M, Difference between IoT and M2M, SDN and NFV for IoT. Smart Objects — Sensors, Actuators, MEMS, Smart Objects and trends. Sensor Networks — Wireless Sensor Networks (WSNs), Communication Protocols for WSNs, Connecting Smart Objects, Communication Criteria.

TOPIC 1: Machine-to-Machine (M2M) Communication    (Hours 1–2)

Before diving into modern IoT, it is important to understand its predecessor — Machine-to-Machine (M2M) communication. M2M has been connecting machines for decades, long before "IoT" became a common term. Understanding M2M helps you understand where IoT came from and what makes it different.

1.1  What is M2M?

📖  DEFINITION:  M2M (Machine-to-Machine) — The networking of machines (or devices) for the purpose of remote monitoring, control, and data exchange, without direct human intervention.

Think of M2M as machines talking to each other in their own private language, without needing a human in the loop. A classic M2M example is a vending machine that automatically sends an alert to the company's server when it is running low on products — the machine notices the problem and reports it, without any human checking the machine.

In an M2M system, the key components are:

▸  M2M Nodes (Devices): Machines or sensors at the field level — e.g., a temperature sensor on an industrial machine, a vehicle tracking unit, or a utility meter

▸  M2M Gateway: A device that collects data from multiple M2M nodes using local communication protocols (proprietary or non-IP based) and forwards it to the backend network over a WAN (Wide Area Network)

▸  Communication Network: The WAN connection — often cellular (2G/3G GPRS), used to transport data from the gateway to backend servers

▸  Point Solutions (Backend): On-premises applications such as service management systems, remote monitoring dashboards, or enterprise applications that consume and act on the M2M data

M2M System Architecture

⭐  KEY POINT:  M2M = Machines + Local/Proprietary Protocol + Gateway + Cellular WAN + On-premises point solutions. Data stays within a closed system.

1.2  Difference Between IoT and M2M

Although M2M and IoT both involve networking of devices, they differ fundamentally in scope, technology, and architecture. Think of M2M as an older, closed system, and IoT as its evolution into an open, internet-connected world.

| Comparison Aspect | M2M | IoT |
| --- | --- | --- |
| Communication Protocols | Proprietary or non-IP based (e.g., Modbus, MQTT over cellular) | Open, IP-based standards (IPv4/IPv6, HTTP, CoAP, MQTT over internet) |
| Connectivity | Point-to-point or star via dedicated gateway; closed network | End-to-end IP connectivity; open internet architecture |
| Things vs Machines | "Machines" — mechanical/electronic devices with embedded modules; hardware emphasis | "Things" — any physical object with unique ID, sensing and communication; software emphasis |
| Data Collection | Collected in point solutions; on-premises storage; siloed | Collected in the cloud; shared across multiple applications and services |
| Analysis | Limited, application-specific analysis | Big Data analytics, machine learning, real-time dashboards in the cloud |
| Scalability | Limited — proprietary systems are harder to scale | Highly scalable — leverages cloud infrastructure |
| Applications | Specific to one vertical (e.g., fleet management, vending) | Cross-domain applications; smart cities, healthcare, agriculture, industry |
| Integration | Difficult — different vendors use different proprietary systems | Easy — open standards allow integration across devices and platforms |

🧠  TRICK:  M2M vs IoT — Mnemonic: M2M = "Machines talking in private (proprietary, closed, hardware-first)". IoT = "Things talking in public (IP-based, open, software-first, cloud)". The key upgrade: IoT adds internet-scale connectivity, cloud analytics, and open standards.

📝  EXAM TIP:  "Differentiate between IoT and M2M" — Write the comparison table above. Focus on: protocols (non-IP vs IP), data storage (on-premises vs cloud), scalability, and integration. Minimum 5 comparison points for full marks.

TOPIC 2: SDN and NFV for IoT    (Hours 2–3)

As IoT scales to billions of connected devices, traditional networking approaches struggle to keep up. Two emerging technologies — Software Defined Networking (SDN) and Network Function Virtualization (NFV) — are transforming IoT network infrastructure to make it simpler, more scalable, and more flexible.

2.1  Software Defined Networking (SDN)

📖  DEFINITION:  SDN — A networking architecture that separates the control plane (which decides where data flows) from the data plane (which actually forwards data), and centralises network control in a software-based controller.

In a traditional network, every switch and router has its own built-in logic for both forwarding packets AND deciding how to forward them. This makes networks complex, difficult to manage, and hard to scale. SDN solves this by splitting these two functions apart.

▶  Limitations of Conventional Network Architectures

▸  Complex Network Devices: Each device runs many protocols. As networks grow, managing hundreds of individual devices becomes very difficult.

▸  Management Overhead: Network managers must configure each device separately, leading to slow and error-prone changes.

▸  Limited Scalability: Cloud environments create thousands of virtual hosts. Traditional networks cannot adapt fast enough.

Conventional network architecture

SDN architecture

▶  Three Layers of SDN

▸  Infrastructure Layer (Data Plane): Physical or virtual network devices (switches, routers) that forward packets. They do NOT make routing decisions — they just follow instructions from the controller.

▸  Control Layer (Control Plane): The centralised SDN Controller — the "brain" of the network. It knows the entire network topology and tells devices how to forward packets.

▸  Application Layer: SDN applications that communicate with the controller through Northbound APIs (e.g., load balancing, firewalls, traffic engineering applications).

▶  Key Elements of SDN

▸  Centralised Network Controller: Single point of control with a global view of the network. Enables rapid reconfiguration of the entire network from one place.

▸  Programmable Open APIs: Northbound APIs connect SDN applications to the controller. Southbound APIs (OpenFlow) connect the controller to network devices.

▸  OpenFlow: The standard communication protocol between the SDN controller and network switches. An OpenFlow switch contains flow tables that the controller programmes.

[ Figure: Figure 3.9: OpenFlow switch ]

[ Figure: Figure 3.10: OpenFlow flow table ]

For IoT, SDN is particularly powerful because it allows the network to be dynamically reconfigured to handle the heterogeneous, large-scale traffic patterns of IoT systems. For example, an SDN controller can automatically allocate more bandwidth to critical sensor data during emergencies.

⭐  KEY POINT:  SDN = Separation of control plane and data plane + Centralised controller + OpenFlow protocol. The controller sees the whole network; devices just forward packets.

2.2  Network Function Virtualization (NFV)

📖  DEFINITION:  NFV — A technology that leverages virtualisation to run network functions (traditionally implemented in dedicated hardware like firewalls, load balancers, routers) as software on standard commercial off-the-shelf (COTS) servers.

Traditionally, adding a firewall, NAT (Network Address Translator), or WAN optimiser to a network meant buying and installing dedicated, expensive hardware appliances. NFV virtualises these functions — they become software programs that run in virtual machines or containers on standard servers.

▶  Key Elements of NFV Architecture (ETSI Standard)

▸  VNF (Virtualized Network Function): A software implementation of a network function — e.g., a virtual firewall, virtual NAT, virtual WAN optimiser — running over the NFV Infrastructure.

▸  NFVI (NFV Infrastructure): The compute (servers), network, and storage resources that are virtualised to host VNFs. Includes the hypervisor layer.

▸  NFV Management & Orchestration (MANO): Software that manages and orchestrates all virtualisation-specific tasks — deploying VNFs, scaling them, monitoring them.

A practical example: A home network has a physical Home Gateway that provides WAN connectivity, WiFi, NAT, and firewall functions. With NFV, all these functions can be virtualised — the physical box is replaced by software running in a cloud server. The service provider can now push software updates, add new features (e.g., parental controls), and scale services remotely.

| Feature | SDN | NFV |
| --- | --- | --- |
| What it does | Separates control and data planes; centralises control | Virtualises network functions; replaces dedicated hardware with software |
| Primary Goal | Programmable, flexible network management | Reduce hardware costs; speed up service deployment |
| Key Standard | OpenFlow (ONF) | ETSI NFV architecture |
| Dependency | Can work without NFV | Can work without SDN |
| Used together? | Yes — SDN and NFV complement each other | Yes — SDN manages the network; NFV provides the functions |

🧠  TRICK:  SDN vs NFV: SDN = "Who controls the traffic flow?" (Controller decides). NFV = "Who runs the network functions?" (Software, not hardware). SDN separates control. NFV virtualises functions. They are complementary — use both together in modern IoT infrastructure.

📝  EXAM TIP:  "What is SDN? Explain its key elements" — Write definition + 3 SDN layers + 3 key elements (controller, OpenFlow, APIs). Draw the SDN architecture diagram. "What is NFV? Explain with a home network example" — Write definition + 3 key elements (VNF, NFVI, MANO) + home gateway example.

TOPIC 3: Sensors — The Sensing Layer of IoT    (Hours 3–4)

If IoT is a nervous system for the physical world, sensors are its nerve endings. They are the devices that allow the digital world to feel, taste, see, hear, and smell the physical world. Without sensors, IoT devices would be blind and deaf — they would have no way to know what is happening in their environment.

3.1  What is a Sensor?

📖  DEFINITION:  Sensor — A device that measures some physical quantity (temperature, pressure, light, motion, etc.) and converts that measurement into an electrical signal that can be read and processed by a computer or microcontroller.

A sensor does exactly what its name says: it senses. More specifically, it measures a physical quantity and converts that measurement into a readable signal. For example, a thermistor measures temperature by changing its electrical resistance as the temperature changes. A microcontroller reads this resistance, converts it to a temperature value, and can then take action — such as turning on a fan.

3.2  Classification of Sensors

Sensors can be classified in multiple ways depending on what criteria you use. Knowing these classification criteria is important for the exam.

▶  Classification by Relationship with Environment

▸  Invasive sensors: The sensor is physically part of the environment it is measuring. Example: A blood glucose sensor inserted under the skin to continuously measure glucose levels in tissue fluid.

▸  Non-invasive sensors: The sensor is external to the environment. Example: An infrared thermometer that measures skin temperature without touching the patient.

▶  Classification by Physical Contact

▸  Contact sensors: Require physical contact with the object being measured. Example: A thermocouple that must touch a surface to measure its temperature.

▸  Non-contact sensors: Measure without physical contact. Example: A LiDAR sensor measuring the distance to an object using laser pulses, or an infrared motion sensor.

▶  Classification by Measurement Reference

▸  Absolute sensors: Measure on an absolute scale — the reading does not depend on a reference point. Example: A thermometer measuring temperature in Kelvin (0 K is absolute zero).

▸  Relative sensors: Measure relative to a fixed reference point. Example: A barometer that measures atmospheric pressure relative to a reference pressure.

▶  Classification by Physical Quantity Measured (Most Common in Exams)

| Sensor Type | Physical Quantity | IoT Application Example |
| --- | --- | --- |
| Temperature sensor | Temperature (°C, °F, K) | Smart thermostat, cold chain monitoring, industrial cooling |
| Pressure sensor | Force per unit area (Pa, bar) | Smart tyre pressure monitoring, weather stations, industrial pipelines |
| Humidity sensor | Moisture content (%RH) | Smart agriculture, HVAC systems, food storage |
| Light sensor (LDR, photodiode) | Light intensity (lux, lumens) | Automatic street lighting, smartphone display brightness |
| Motion sensor (PIR) | Presence/movement of infrared sources | Security systems, automatic doors, occupancy detection |
| Proximity sensor | Distance to nearby objects | Smartphones (face detection), parking sensors |
| Accelerometer | Acceleration (m/s²) | Fitness trackers, vehicle crash detection, screen orientation |
| Gyroscope | Angular velocity (°/s) | Drones, gaming controllers, navigation systems |
| Gas sensor | Concentration of gases (ppm) | CO₂ monitoring, air quality stations, industrial leak detection |
| Sound sensor (microphone) | Sound pressure (dB) | Smart speakers, noise monitoring, glass-break detectors |
| GPS/GNSS | Geographic position | Asset tracking, vehicle navigation, precision agriculture |

[ Figure: Figure 3-2: Sensors in a Smartphone ]

A modern smartphone contains over a dozen different sensors — accelerometer, gyroscope, barometer, GPS, proximity sensor, ambient light sensor, microphone, camera, fingerprint sensor, and more. This makes a smartphone itself an IoT device!

🧠  TRICK:  Sensor classification in exams — remember 4 criteria: (1) Invasive/Non-invasive, (2) Contact/Non-contact, (3) Absolute/Relative, (4) By physical quantity measured. The 4th is the most important and most likely to appear in Part B.

🔔  REMEMBER:  Sensors convert physical quantities → electrical signals. Actuators do the opposite: convert electrical signals → physical actions. Together, they form the INPUT and OUTPUT of an IoT system.

TOPIC 4: Actuators — The Action Layer of IoT    (Hour 4)

4.1  What is an Actuator?

📖  DEFINITION:  Actuator — A device that converts an electrical signal (or other energy form) into a physical action, movement, or change in the environment.

While sensors are the "input" side of IoT — receiving information from the physical world — actuators are the "output" side — causing a change in the physical world based on that information. Think of sensors as eyes and ears, and actuators as hands and muscles.

The analogy with human biology: Just as humans use their five senses to perceive the environment, and the brain converts sensory information into nerve signals that command muscles and glands, IoT systems use sensors to measure the environment, a processor to analyse the data, and actuators to respond to that environment.

How Sensors and Actuators Interact with the Physical World

Comparison of Sensor and Actuator Functionality with Humans

4.2  Classification of Actuators

▶  By Type of Motion Produced

▸  Linear actuators: Produce straight-line motion. Example: A hydraulic cylinder extending a robotic arm in a factory assembly line.

▸  Rotary actuators: Produce rotational motion. Example: A DC motor spinning a drone propeller.

▸  Multi-axis actuators: Produce motion in 2 or 3 axes simultaneously. Example: A robotic wrist joint.

▶  By Energy Source / Type of Actuation (Most Important Classification)

| Actuator Type | Energy Source | How It Works | IoT Example |
| --- | --- | --- | --- |
| Electrical actuators | Electricity | Convert electrical energy to mechanical motion using electromagnets or motors | DC motor in a smart fan; solenoid valve in a smart irrigation system |
| Pneumatic actuators | Compressed air | Convert compressed air pressure into linear or rotary motion | Pneumatic gripper in robotic arm; air-powered valve in pipeline |
| Hydraulic actuators | Pressurised fluid | Convert fluid pressure into mechanical force | Hydraulic lifts; heavy industrial machinery |
| Thermal actuators | Heat | Materials expand or contract with temperature change | Bimetal strip in a thermostat; shape-memory alloy actuators |
| Magnetic actuators | Magnetic field | Magnets attract or repel to produce motion | Magnetic relays; electromagnetic locks; voice coil actuators in speakers |
| Mechanical actuators | Mechanical energy | Convert rotary motion to linear motion via gears/screws | Lead screw linear actuator; rack-and-pinion systems |

⭐  KEY POINT:  Sensors → Measure → Electrical Signal → Processor → Electrical Signal → Actuators → Physical Action. Sensors are inputs; actuators are outputs. Together they create a closed-loop feedback system in IoT.

📝  EXAM TIP:  "Classify actuators based on energy source with examples" — Write all 6 types (Electrical, Pneumatic, Hydraulic, Thermal, Magnetic, Mechanical) with a real IoT example for each. 3 marks = any 3 types; 9 marks = all types with detailed explanation.

TOPIC 5: MEMS — Micro-Electro-Mechanical Systems    (Hour 4)

📖  DEFINITION:  MEMS (Micro-Electro-Mechanical Systems) — Miniaturised devices that combine mechanical and electrical components on a single chip using semiconductor manufacturing techniques. They can sense, process, and actuate at the microscale.

MEMS are the technology behind the tiny sensors inside your smartphone. A MEMS accelerometer inside a phone is smaller than a grain of rice, yet it can precisely measure acceleration in three axes. MEMS devices are manufactured using the same processes used to make computer chips — which means they can be mass-produced cheaply and integrated directly onto circuit boards alongside processors.

MEMS are important for IoT because they enable the combination of sensing, processing, and sometimes actuation in extremely small, low-cost, low-power packages. Key MEMS examples:

▸  Inkjet printer nozzles: MEMS micropumps precisely control ink droplet size and direction

▸  Smartphones: MEMS accelerometers (screen orientation), MEMS gyroscopes (navigation), MEMS barometers (altitude), MEMS microphones (voice commands)

▸  Automotive airbags: MEMS accelerometers detect sudden deceleration (crash) and trigger airbag deployment in milliseconds

▸  Medical devices: MEMS pressure sensors monitor blood pressure; MEMS pumps control drug delivery in implantable devices

▸  Industrial systems: MEMS vibration sensors monitor machine health; MEMS pressure sensors monitor pipelines

⭐  KEY POINT:  MEMS combine mechanical + electrical functions on a single chip. They are tiny, cheap, low-power, and mass-producible — making them the ideal technology for IoT sensors and actuators.

TOPIC 6: Smart Objects — The "Things" in IoT    (Hour 5)

6.1  What is a Smart Object?

📖  DEFINITION:  Smart Object — A physical object augmented with sensing, processing, communication, and actuation capabilities that allow it to interact with its environment and other objects, and contribute to the IoT ecosystem.

A regular object (a chair, a lamp, a pipe) becomes "smart" when you add these four capabilities. A smart chair can monitor posture. A smart lamp adjusts its brightness automatically. A smart pipe detects leaks. The "smart" in smart object comes from the combination of these four components working together.

6.2  Four Components of a Smart Object

| Component | Function | Examples |
| --- | --- | --- |
| Sensor(s) / Actuator(s) | Interface with the physical world — sense conditions or perform actions | Temperature sensor, humidity sensor, motor, LED, solenoid valve |
| Processor (Microcontroller/SoC) | Process sensor data, run local algorithms, make decisions, control actuators | ARM Cortex-M, ESP32, Arduino ATmega, Raspberry Pi SoC |
| Communication Hardware | Connect to other devices and the network for data exchange | Wi-Fi module, ZigBee radio, BLE, LoRa transceiver, Ethernet |
| Power Source | Provide electrical energy to run the device | Battery, energy harvester (solar, piezoelectric), USB, mains supply |

[ Figure: Figure 3-6: Smart Object Components ]

🧠  TRICK:  Smart Object Components — Remember: S-P-C-P: Sensor/Actuator → Processor → Communication → Power. All 4 must work together for an object to be truly "smart".

6.3  Five Trends in Smart Objects

The field of smart objects is evolving rapidly. Five key trends are driving this evolution:

▸  Decreasing Size: MEMS and semiconductor advances continuously shrink smart objects. Some are now small enough to be implanted inside the human body or embedded in textiles. Smaller devices enable new application areas that were previously impossible.

▸  Decreasing Power Consumption: Advances in semiconductors, ultra-low-power hardware design, sleep modes, and duty cycle optimisation are dramatically reducing energy needs. Some IoT devices now harvest energy from the environment (solar, vibration) and can operate indefinitely without batteries.

▸  Increasing Processing Power: Despite getting smaller and consuming less power, smart objects are becoming more computationally capable. Edge AI (running machine learning models on-device) is now possible even on small microcontrollers — enabling local decision-making without sending data to the cloud.

▸  Improving Communication Capabilities: Wireless communication in smart objects continues to improve in speed, range, reliability, and energy efficiency. New technologies (LoRa, NB-IoT, 5G) are enabling connectivity in places previously impossible.

▸  Increasing Standardisation: The industry is moving toward greater standardisation of hardware interfaces, communication protocols, and software APIs. This enables greater interoperability between devices from different manufacturers, which is essential for building large-scale IoT systems.

📝  EXAM TIP:  "What are the trends in smart objects?" — Write all 5 trends as a numbered list with one sentence of explanation each. For 9 marks, expand each trend with an example and explain why it matters for IoT.

TOPIC 7: Wireless Sensor Networks (WSNs)    (Hours 5–6)

7.1  From SANETs to WSNs

📖  DEFINITION:  SANET (Sensor and Actuator Network) — A network of sensors and actuators that cooperate to monitor and control a physical environment. Sensors provide data; actuators respond to that data.

📖  DEFINITION:  WSN (Wireless Sensor Network) — A network of wirelessly connected, battery-powered sensor nodes (also called "motes") that sense their environment, process data locally, and communicate with each other and a base station.

Sensors and actuators are rarely deployed alone. They work in networks — sharing data, collaborating, and forming a distributed sensing and actuation system. SANETs include both sensors and actuators; WSNs focus specifically on wireless sensor nodes. Typical WSN applications: smart agriculture (soil monitoring), structural health monitoring of bridges, forest fire detection, military surveillance, and smart city air quality monitoring.

7.2  WSN Design Constraints

WSN sensor nodes are fundamentally resource-constrained. Understanding these constraints is critical — they explain why WSN protocols are designed the way they are.

| Design Constraint | What it means | Why it matters |
| --- | --- | --- |
| Limited Energy (Battery) | Nodes run on small batteries; replacement is often impractical (nodes may be in remote or inaccessible locations) | Drives all design decisions — radio duty cycling, sleep modes, data aggregation, low-power protocols |
| Limited Processing Power | Small microcontrollers with slow clocks (8–32 MHz) and small RAM (a few KB) | Complex computations must be offloaded or avoided; local processing is simple |
| Limited Memory | Flash storage: 32–256 KB; RAM: 1–64 KB | Cannot store large data sets; must transmit data promptly; compression is important |
| Limited Communication Bandwidth | Data rates of 20–250 kbps; very low compared to Wi-Fi or cellular | Protocols must minimise data transmitted; data aggregation is essential |
| Small Physical Size | Nodes must be small to be deployable in constrained spaces | Limits battery size, antenna size, and thus communication range |
| Harsh Environment | Deployed outdoors, in industrial environments, or in remote areas | Must be ruggedised, waterproof, and operate across wide temperature ranges |

[ Figure: Figure 3-8: Design Constraints for Wireless Smart Objects ]

⭐  KEY POINT:  All WSN design choices flow from energy constraints. To maximise battery life: sleep most of the time, transmit rarely, aggregate data locally, use low-power radio protocols (IEEE 802.15.4), and use mesh routing to avoid long-range transmissions.

7.3  Communication Patterns in WSNs

WSN nodes use one of two communication patterns to decide when to transmit data:

▸  Event-Driven Communication: A node transmits only when it detects a specific event or threshold crossing. Example: A fire detection sensor transmits only when temperature exceeds 60°C. Very energy-efficient because the radio is OFF most of the time. Best for: alarm/monitoring applications where most of the time "nothing happens".

▸  Periodic Communication: A node transmits at regular fixed intervals (e.g., every 30 seconds), regardless of whether anything notable has been detected. Example: A weather station sending temperature and humidity readings every minute. Predictable behaviour; simpler to design. Best for: continuous monitoring where all data points are needed.

In practice, many systems combine both patterns: Periodic transmissions for routine data + Event-driven transmissions for critical alerts. This gives the best balance of data completeness and energy efficiency.

📝  EXAM TIP:  "Explain event-driven and periodic communication in WSNs with examples" — Define each pattern, give a real-world example for each, and state which is more energy-efficient and why.

7.4  Data Aggregation in WSNs

In a WSN with hundreds or thousands of nodes, transmitting raw data from every single node to the base station would waste enormous amounts of energy and bandwidth. Data aggregation solves this problem by combining data from multiple nodes before forwarding it upstream.

How data aggregation works:

▸  Step 1 — Clustering: Nodes in close proximity with similar readings are grouped into clusters. Each cluster has a designated "cluster head" node.

▸  Step 2 — Aggregation: The cluster head collects readings from all cluster members and applies an aggregation function: averaging, minimum, maximum, compression, or filtering.

▸  Step 3 — Forwarding: The cluster head transmits only the aggregated summary (not individual readings) to the base station or the next cluster head upstream.

[ Figure: Figure 3-9: Data Aggregation in Wireless Sensor Networks ]

Benefits of data aggregation:

▸  Energy saving: Nodes deep in the network transmit far less data; average energy consumption per node drops dramatically

▸  Bandwidth reduction: Network traffic is reduced by an order of magnitude in large WSNs

▸  Latency reduction: Less data to transmit means faster data delivery

▸  Noise reduction: Averaging multiple readings improves accuracy and removes outlier noise

🔔  REMEMBER:  Data aggregation connects to Fog/Edge Computing: Instead of sending all raw data to the cloud, processing happens "at the edge" (near the sensors). This is the same principle as data aggregation — compute locally, send summaries. Fog computing extends data aggregation to a more powerful gateway device.

7.5  Self-Organisation in WSNs

IoT applications are often mission-critical (patient monitoring, industrial safety, military surveillance). In large-scale WSNs, the network must continue to function even if individual nodes fail — batteries die, nodes are physically damaged, or wireless links break. Self-organisation gives WSNs the ability to adapt automatically.

▸  Self-Organisation: Nodes automatically discover their neighbours, form clusters, and establish routing paths without manual configuration. New nodes joining the network integrate themselves automatically.

▸  Self-Healing: When a node fails or a link breaks, the network detects the failure and automatically reconfigures routing paths to work around it. No human intervention is required.

▸  Self-Configuration: New nodes that join the network automatically configure their own parameters (address, routing table, cluster membership) using distributed protocols.

▸  Self-Optimisation: The network continuously adjusts transmission power, duty cycles, and routing paths to maximise energy efficiency and performance as conditions change.

⭐  KEY POINT:  Self-organisation is what makes large-scale WSN deployments practical. Without it, maintaining thousands of sensor nodes across a large farm or factory would require an army of technicians. With self-organisation, the network takes care of itself.

TOPIC 8: Communication Criteria for Connecting Smart Objects    (Hours 6–7)

Not all wireless technologies are equal, and no single technology is perfect for every IoT application. When selecting a wireless technology for an IoT deployment, engineers evaluate eight key criteria. Understanding these criteria is essential for applying IoT knowledge — it is exactly the type of K3 "Apply" question that appears in Part B.

8.1  Range

📖  DEFINITION:  Range — The maximum distance over which two devices can reliably communicate at a specified data rate and error rate.

Different IoT applications have very different range requirements:

| Category | Range | Technologies | IoT Example |
| --- | --- | --- | --- |
| Short range | <1 m to 10 m | NFC, RFID, Bluetooth | Contactless payment, wearable health monitors |
| Medium range | 10 m to 300 m | Bluetooth LE, ZigBee, IEEE 802.15.4, Z-Wave | Smart home, factory floor, building automation |
| Long range | 300 m to 1 km | Wi-Fi (802.11), WirelessHART | Campus-wide monitoring, large warehouses |
| Wide area | 1 km to 50 km | LoRaWAN, Sigfox, NB-IoT, LTE-M | Smart city, precision agriculture, smart metering |

🔔  REMEMBER:  Range specifications in datasheets assume ideal conditions (open field, no obstacles, no interference). Real deployments always achieve shorter range. Walls, furniture, machinery, and other wireless signals can reduce range significantly.

8.2  Frequency Bands

The frequency at which a wireless device operates determines its range, data rate, ability to penetrate obstacles, and regulatory compliance. Most IoT devices use unlicensed ISM (Industrial, Scientific, Medical) frequency bands.

| ISM Band | Region | Advantages | Technologies |
| --- | --- | --- | --- |
| 868 MHz | Europe, India | Better wall penetration, longer range, less crowded than 2.4 GHz | ZigBee, LoRaWAN, IEEE 802.15.4g, Z-Wave |
| 915 MHz | Americas, Australia | Similar to 868 MHz; good range and penetration; more channels | LoRaWAN, ZigBee, IEEE 802.15.4g |
| 2.4 GHz | Worldwide | Available globally; higher data rate; more channels | ZigBee, Wi-Fi, Bluetooth, IEEE 802.15.4 |
| 5 GHz | Worldwide | Very high data rate; less crowded in some areas | Wi-Fi 802.11a/n/ac only |

⭐  KEY POINT:  Sub-GHz bands (868/915 MHz) are generally preferred for IoT because: (1) Better wall/building penetration — signals at lower frequencies bend around and penetrate obstacles more easily. (2) Longer range for the same transmit power. (3) Less crowded than 2.4 GHz (which is shared with Wi-Fi and Bluetooth). Trade-off: lower data rates.

8.3  Power Consumption

Power consumption is often the most critical criterion for IoT smart objects — many are battery-powered and must operate for months or years without battery replacement (nodes may be in inaccessible locations).

▸  Sleep modes: Devices spend most of their time in ultra-low-power sleep states (microamps of current), waking up only to sample sensors and transmit data. Sleep can reduce average current from milliamps to microamps.

▸  Duty cycle minimisation: Duty cycle = fraction of time the radio is transmitting or receiving. A 1% duty cycle means the radio is active for only 36 seconds per hour. Minimising duty cycle dramatically reduces power consumption.

▸  Energy harvesting: Some long-lived deployments use energy harvesting — converting ambient energy (solar light, mechanical vibration, temperature gradients) into electrical power. This enables truly self-powered, maintenance-free sensors.

▸  LPWA technologies: Low-Power Wide-Area (LPWA) technologies like LoRaWAN, Sigfox, and NB-IoT are specifically designed for devices that need to communicate infrequently but need to cover long distances. They achieve ultra-low power consumption by using very narrow bandwidths and long sleep times.

8.4  Topology

| Topology | Description | Advantages | Disadvantages | Example Technologies |
| --- | --- | --- | --- | --- |
| Star | All devices communicate directly with a central hub/gateway. No multi-hop routing. | Simple; low latency; easy management | Limited range (all devices must reach the hub); hub is single point of failure | LoRaWAN, Sigfox, Wi-Fi |
| Mesh | Devices relay data for each other; multiple paths exist between any two nodes; self-healing | Extended range; redundancy; self-healing; no single point of failure | Complex routing; more power needed for relay nodes; latency increases with hops | ZigBee, Thread, Z-Wave, Wi-Fi mesh |
| Point-to-Point | Direct link between two devices only | Simple; very low latency | Only two devices; not scalable | NFC, Bluetooth Classic |
| Point-to-Multipoint | One device (master) communicates with many; slaves do not communicate with each other | Efficient downlink broadcast; moderate complexity | Uplink is limited; slaves cannot share data directly | Bluetooth LE, 802.11 Wi-Fi infrastructure mode |

8.5  Constrained Devices and Networks

RFC 7228 defines three classes of constrained IoT devices based on memory and processing resources:

| Class | RAM | Flash (Program Memory) | Description |
| --- | --- | --- | --- |
| Class 0 | <10 KB | <100 KB | Extremely constrained; cannot run a full IP stack; must use proxies |
| Class 1 | ~10 KB | ~100 KB | Constrained but can run CoAP and 6LoWPAN with care; no full HTTP |
| Class 2 | ~50 KB | ~250 KB | Less constrained; can run most IoT protocols; similar to early smartphones |

Low-power and Lossy Networks (LLNs) are networks composed of constrained nodes with links characterised by high loss rates, low throughput, and instability. Protocols like 6LoWPAN and RPL were specifically designed for LLNs.

8.6  Data Rate and Throughput

IoT access technologies offer data rates from 100 bps (Sigfox) to hundreds of Mbps (LTE/Wi-Fi). For most IoT sensor applications, data rates of a few kbps are more than sufficient — a temperature reading is just a few bytes. However, several factors reduce actual throughput below the stated data rate:

▸  Protocol overhead: Headers, error correction codes, and retransmissions consume part of the available bandwidth

▸  Channel sharing: Multiple devices share the same wireless channel; actual throughput per device decreases as more devices transmit

▸  Duty cycle restrictions: Many IoT technologies impose maximum duty cycle limits (e.g., 1% transmit time per hour) to comply with ISM band regulations

▸  Fragmentation overhead: Protocols like 6LoWPAN fragment IPv6 packets to fit in small 802.15.4 frames, adding overhead

8.7  Latency and Determinism

📖  DEFINITION:  Latency — The delay between sending a data packet and its delivery at the destination. Determinism — The ability to guarantee that a message will be delivered within a specified time bound.

Latency requirements vary enormously across IoT applications:

▸  Industrial control (sub-millisecond): Real-time machine control and safety systems require deterministic, sub-millisecond latency. Use Deterministic Ethernet or IEEE 802.15.4e TSCH mode.

▸  Medical monitoring (seconds): Patient vital signs alerts need near-real-time response for safety.

▸  Smart metering / environmental monitoring (minutes to hours): Meter readings and air quality data can tolerate significant latency.

8.8  Overhead and Payload

In constrained IoT networks, overhead (the size of headers, acknowledgements, and control messages relative to the actual application data payload) is a significant concern:

▸  IPv6 minimum MTU: 1280 bytes — much larger than the 127-byte frame size of IEEE 802.15.4. Fragmentation is needed.

▸  LPWA payload limits: LoRaWAN may limit payloads to 50 bytes per message. Protocol design must be compact.

▸  6LoWPAN solution: Compresses IPv6 headers and provides fragmentation/reassembly to enable IPv6 over constrained links.

📝  EXAM TIP:  "A smart hospital wants to deploy WSN nodes for patient monitoring. Apply five communication criteria to select the most suitable wireless technology." — Apply: Range (within hospital), Power (battery life), Latency (real-time alerts), Topology (mesh for coverage), Data rate (small sensor readings, low rate needed). Then select ZigBee or IEEE 802.15.4-based technology and justify.

TOPIC 9: Communication Protocols for WSNs    (Hours 7–8)

9.1  IEEE 802.15.4

IEEE 802.15.4 is the most widely used standard for WSN communication. It forms the PHY (Physical) and MAC (Medium Access Control) layer foundation for multiple IoT protocol stacks including ZigBee, 6LoWPAN, and Thread.

📖  DEFINITION:  IEEE 802.15.4 — A standard defining PHY and MAC layer characteristics for Low-Rate Wireless Personal Area Networks (LR-WPANs). Designed for low power, low data rate (250 kbps max), and short range (10–100 m).

▶  PHY Layer

The PHY layer handles radio signal transmission. Key parameters: 2.4 GHz band (16 channels, 250 kbps, worldwide); 915 MHz band (10 channels, 40 kbps, Americas); 868 MHz band (1 channel, 20 kbps, Europe). The PHY frame: Preamble (32 bits) + Start of Frame Delimiter (SFD) + PHY Header (PHR) + Payload (max 127 bytes).

[ Figure: Figure 4-5: IEEE 802.15.4 PHY Format ]

▶  MAC Layer

The MAC layer manages channel access (who transmits when) using CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance). Four MAC frame types: Data frames (carry application data), Acknowledgement frames (confirm delivery), Beacon frames (network synchronisation, issued by coordinator), MAC command frames (control — joining, leaving, requesting data).

▸  CSMA/CA: Before transmitting, a node listens to the channel. If the channel is busy (another device transmitting), the node waits a random backoff time, then tries again. This avoids collisions.

▸  Beaconing: The PAN coordinator periodically broadcasts beacon frames that new devices use to discover and join the network.

▸  Security: AES-128 encryption is optionally supported at the MAC layer.

IEEE 802.15.4 MAC Frame Format ]

▶  FFD vs RFD Devices

| Feature | FFD (Full-Function Device) | RFD (Reduced-Function Device) |
| --- | --- | --- |
| Stack implementation | Complete IEEE 802.15.4 stack | Subset of 802.15.4 stack only |
| Roles | PAN Coordinator, Router, or End Device | End Device (leaf node) only |
| Can be coordinator? | Yes | No |
| Can communicate with? | Other FFDs and RFDs | Only its associated FFD coordinator |
| Can relay data? | Yes (router function) | No |
| Power | Usually mains-powered (always on) | Usually battery-powered (sleeps frequently) |

[ Figure: Figure 4-7: 802.15.4 Sample Mesh Network (PAN with FFDs and RFDs) ]

🧠  TRICK:  FFD vs RFD — remember: FFD = "Full-Featured Device" (does everything — coordinator, router, end device). RFD = "Restricted-Function Device" (can only talk to its coordinator; cannot relay; sleeps heavily to save battery).

9.2  ZigBee

ZigBee is a protocol stack built ON TOP of IEEE 802.15.4. While 802.15.4 only defines the PHY and MAC layers, ZigBee adds the Network layer, Security layer, Application Support Sublayer, and Application layer — forming a complete protocol stack ready for application development.

| ZigBee Layer | Function |
| --- | --- |
| Application Layer (APL) | ZigBee Device Objects (ZDO) + pre-defined application profiles (Home Automation, Smart Energy) + user application |
| Application Support Sublayer (APS) | Group addressing, binding, reliable data delivery between applications |
| Network/Security Layer (NWK) | Network start-up, routing (AODV mesh), security (AES-128), neighbour discovery |
| MAC Layer (IEEE 802.15.4) | Channel access, beaconing, association, acknowledgements, optional encryption |
| PHY Layer (IEEE 802.15.4) | 2.4 GHz radio; DSSS modulation; 250 kbps |

ZigBee uses reactive routing (AODV — Ad hoc On-Demand Distance Vector): routes are discovered only when needed. When a device needs to send data to a destination it hasn't communicated with before, it broadcasts a route request. The network discovers the shortest path (fewest hops) and caches the route. This saves energy by not maintaining route tables when no communication is happening.

ZigBee application profiles define standard command sets for specific industries: Home Automation (HA), Smart Energy (SE), ZigBee Light Link (ZLL for LED lighting), Commercial Building Automation (CBA).

9.3  ZigBee IP

📖  DEFINITION:  ZigBee IP — An extension of ZigBee that replaces ZigBee-specific upper layers with IETF open standards (IPv6, 6LoWPAN, RPL, UDP) to enable direct internet connectivity for ZigBee networks.

| Layer | Traditional ZigBee | ZigBee IP |
| --- | --- | --- |
| Application | ZigBee application profiles | ZigBee application profiles (same) |
| Network/Transport | ZigBee NWK (proprietary) | IPv6 + UDP (open IETF standards) |
| Adaptation | None | 6LoWPAN (IPv6 compression + fragmentation) |
| Routing | ZigBee AODV | RPL — Routing Protocol for Low-Power Lossy Networks |
| Security | ZigBee AES-128 | DTLS (Datagram TLS) — same as HTTPS security |
| PHY/MAC | IEEE 802.15.4 | IEEE 802.15.4 (unchanged) |

ZigBee IP was developed specifically for the Smart Energy Profile 2.0 — smart metering and residential energy management. The key advantage: ZigBee IP devices can communicate directly with internet hosts using standard IPv6 — no translation gateway is needed.

🔔  REMEMBER:  6LoWPAN is the "adapter" that makes IPv6 work over IEEE 802.15.4. It does three things: (1) Compresses IPv6 headers from 40 bytes to as few as 2 bytes. (2) Fragments large IPv6 packets to fit in 127-byte 802.15.4 frames. (3) Supports mesh addressing for multi-hop networks.

TOPIC 10: Comparing IoT Connectivity Technologies    (Hour 8–9)

| Technology | Range | Data Rate | Frequency | Topology | Battery Life | Best For |
| --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.15.4 | 10–100 m | 250 kbps | 2.4 GHz / sub-GHz | Star / Mesh | Years | Foundation for ZigBee, 6LoWPAN; WSN base layer |
| ZigBee | 10–100 m (mesh extends further) | 250 kbps | 2.4 GHz (mainly) | Mesh / Star / Tree | Years | Home automation, building automation, smart energy |
| Bluetooth LE | 10–100 m | 1 Mbps (LE) / 2 Mbps (LE 2M) | 2.4 GHz | Star (piconet) | Months–Years | Wearables, healthcare, proximity beacons |
| Wi-Fi (802.11) | 50–300 m | 54 Mbps–1+ Gbps | 2.4 / 5 GHz | Star (AP) | Hours–Days | High data rate devices, cameras, gateways |
| LoRaWAN | 2–15 km | 0.3–50 kbps | 868/915 MHz (ISM) | Star-of-stars | Years | Smart city, agriculture, metering, wide-area IoT |
| NB-IoT | Cell coverage | ~60 kbps | Licensed LTE band | Cellular | 10+ years | Static sensors, smart meters, deep-indoor devices |
| Z-Wave | 30–100 m | 9.6–100 kbps | 868/908 MHz | Mesh | Years | Smart home (dominant in US home automation) |
| RFID/NFC | <1 m (passive) | Low kbps | 13.56 MHz (NFC) | Point-to-point | N/A (passive) | Asset tracking, contactless payments, access control |

📝  EXAM TIP:  "Apply communication criteria to select a wireless technology for a specific IoT application" — Always structure your answer as: (1) State the application requirement for each criterion (range, power, data rate, topology, latency), (2) Identify which criteria are most critical for this application, (3) Select the technology and justify each criterion. This is the standard K3-level Part B approach.

TOPIC REVIEW QUESTIONS: Self-Assessment

PART A — Short Answer (3 Marks each)

Q1. Define M2M. State the role of an M2M gateway using an example.

Q2. Differentiate between IoT and M2M using any three criteria.

Q3. Define SDN. Identify three limitations of conventional network architecture that SDN addresses.

Q4. Define NFV. State its three key components (VNF, NFVI, MANO) and their functions.

Q5. Define a smart object. List its four components and the function of each.

Q6. Classify sensors using any three criteria. Give one example for each criterion.

Q7. Classify actuators based on energy source. Give one IoT example for each type.

Q8. Define a WSN. State any three design constraints of WSN sensor nodes.

PART B — Long Answer (9 Marks each)

Q1. Apply your knowledge of M2M and IoT to compare them using a table with at least five points of difference. Explain the architecture of an M2M system with a labelled diagram and describe the role of each component.

Q2. Apply the concept of Software Defined Networking (SDN) to explain its architecture (three layers), key elements (centralised controller, OpenFlow, open APIs), and how SDN addresses the limitations of conventional networks. Illustrate the SDN architecture with a diagram.

Q3. Apply your understanding of smart objects to describe the four components of a smart object. Explain all five trends shaping the future of smart objects with examples from IoT applications.

Q4. Describe the WSN protocol stack by examining IEEE 802.15.4, ZigBee, and ZigBee IP (6LoWPAN) at each layer. Apply five communication criteria to select the most suitable wireless technology for a smart hospital patient monitoring system, justifying each criterion.

Q5. Explain WSN architecture with a labelled diagram. Compare FFD and RFD nodes. Explain the six design constraints of WSN nodes and how data aggregation and self-organisation address these constraints.

Q6. Apply five communication criteria — Range, Frequency Bands, Power Consumption, Topology, and Data Rate — to evaluate wireless technologies for IoT. Classify the ISM frequency bands used in IoT and justify why sub-GHz bands are preferred for IoT deployments.