JYOTHI ENGINEERING COLLEGE OF COMPUTING

Department of Mechatronics Engineering

Sessional Examination — Model Answer Key

PEMRT525 — IoT and Applications

Batch MR 2K24 | Semester V | Modules 3 & 4

Prepared by: Mr. Jinesh K J

MODULE 3 — IoT Protocols: Modbus | ZigBee | LoRaWAN | NB-IoT

PART A — 3 Mark Questions (Module 3)

Q1.  What is NB-IoT? Compare NB-IoT and LoRaWAN in terms of range and infrastructure requirement.   [CO3]  [K1]  [3 Marks]

Answer:

NB-IoT (Narrowband IoT) is a 3GPP radio technology standard that operates on a licensed cellular frequency band of 180 kHz bandwidth. It provides improved indoor coverage, support for a massive number of low-throughput devices, low power consumption, and low device cost. NB-IoT can operate in three modes: standalone (GSM carrier), in-band (LTE carrier), and guard-band (between LTE/WCDMA bands).

| Feature | NB-IoT | LoRaWAN |
| --- | --- | --- |
| Range | Depends on existing cellular tower placement (1–10 km urban). Coverage limited to where operator has towers. | 10–15 km in rural open areas using a single private gateway. Better for remote deployments. |
| Infrastructure | Requires a cellular operator network and a SIM card per device. Monthly subscription fee applies. | Operates on unlicensed ISM band (865–867 MHz in India). Municipality or company installs its own gateways — no SIM, no subscription cost. |

Q2.  What is Modbus? List its three communication variants and state one use case for each.   [CO3]  [K1]  [3 Marks]

Answer:

Modbus is an industrial communication protocol that uses a master/slave relationship. Only the master initiates transactions (called queries). Slaves (sensors, PLCs, I/O transducers, valves) respond by supplying data or by taking the requested action. Masters can address individual slaves or broadcast to all. Modbus is also used in building management, transportation, and energy applications.

| Variant | How It Works | One Use Case |
| --- | --- | --- |
| Modbus RTU | Serial communication over RS-485. Each byte sent as binary (8 bits). Uses CRC (Cyclical Redundancy Check) for error detection. More compact and faster than ASCII. | Factory SCADA systems where sensors and PLCs are connected on an RS-485 bus. |
| Modbus ASCII | Serial communication over RS-232. Each byte is encoded as 2 ASCII hex characters (7 data bits). Uses LRC (Longitudinal Redundancy Check). Slower but easier to debug. | Legacy industrial equipment or manually operated systems where visual inspection of frames is needed. |
| Modbus TCP | Modbus protocol encapsulated over TCP/IP Ethernet. No additional CRC needed — TCP handles error checking. | Modern BMS (Building Management Systems) and SCADA over Ethernet — enables remote monitoring via IP. |

Q3.  List the four register types in Modbus protocol. State the type of data stored in each register and give one example device that uses each type.   [CO3]  [K1]  [3 Marks]

Answer:

| Register Type | Data Stored | Access | Example Device |
| --- | --- | --- | --- |
| Coil (Output Coil) | 1-bit (0 or 1) | Read / Write | Relay coil (motor ON/OFF), solenoid valve (open/close) |
| Discrete Input | 1-bit (0 or 1) | Read Only | Push button state, limit switch at end of conveyor belt |
| Input Register | 16-bit unsigned word | Read Only | Temperature sensor raw ADC value, pressure transducer output |
| Holding Register | 16-bit unsigned word | Read / Write | Motor speed setpoint (e.g. 1500 RPM), PLC timer configuration |

Q4  What is ZigBee? Name the three device types in a ZigBee network and state the role of each.   [CO3]  [K1]  [3 Marks]

Answer:

ZigBee is a low-power, low-data-rate wireless communication standard built on IEEE 802.15.4 PHY and MAC layers. It uses AES-128 encryption and AODV routing across mesh networks. It is aimed at smart objects and sensors with low bandwidth and low power needs. ZigBee-compliant products from different vendors interoperate through certification by the ZigBee Alliance.

| Device Type | Role in Network |
| --- | --- |
| ZigBee Coordinator (ZC) | Exactly ONE per PAN. Initiates and manages the network. Assigns 16-bit short addresses to joining devices. Stores security keys and network configuration. Must remain powered on at all times. |
| ZigBee Router (ZR) | Full-Function Device (FFD). Routes messages between nodes, extending the network range. Can accept child devices (end devices or other routers). Must remain powered on at all times. Example: smart plug, wall switch. |
| ZigBee End Device (ZED) | Reduced-Function Device (RFD). Communicates only with its parent node (coordinator or router). Cannot route messages for others. Can sleep between readings to save battery. Example: occupancy sensor, temperature sensor, nurse call button. |

PART B — 9 Mark Questions (Module 3)

Q5.  A municipal corporation plans to monitor water levels in 150 open wells spread across a rural district covering 20 km². Sensors transmit water level + GPS hourly. 5-year battery. No cellular or Wi-Fi. Identify the best IoT protocol, explain its working, and compare with two alternatives.   [CO3]  [K3]  [9 Marks]

Answer:

Draw block diagram

(i) Best Protocol: LoRaWAN

| Requirement | How LoRaWAN Satisfies It |
| --- | --- |
| Coverage 20 km² | A single LoRa gateway covers 10–15 km radius in open rural terrain. One gateway completely covers the 20 km² district. |
| 5-year battery life | Class A mode: sensor wakes hourly, transmits a packet (~5 seconds air time at SF12), opens 2 receive windows, then sleeps. Total daily air time is under 2 minutes. Battery life easily exceeds 5 years. |
| Small data packet | Water level (2 bytes) + GPS coordinates (8 bytes) + metadata = ~12 bytes. LoRaWAN payload at SF12 supports up to 51 bytes — easily fits. |
| No cellular or Wi-Fi | LoRaWAN uses unlicensed ISM band (865–867 MHz in India). The municipality installs its own gateways. No SIM card, no monthly subscription. |

(ii) How LoRaWAN Works

Physical Layer: LoRa (Chirp Spread Spectrum) modulation. A chirp is a signal that sweeps linearly in frequency over time. CSS allows signals to be received even below the noise floor, giving a link budget of up to 157 dB. SF12 selected for maximum range.

MAC Layer: Class A device. Sensor wakes hourly, transmits uplink, opens 2 receive windows (RX1 and RX2), then sleeps. ADR (Adaptive Data Rate) algorithm adjusts SF and transmit power to optimise battery and range.

Topology: Star of Stars. 150 End Nodes → 1 LoRa Gateway (installed on a rooftop or water tower) → Network Server (cloud) → Application Server (municipal water dashboard, SMS alerts for low/high water levels). Multiple gateways may receive the same packet; Network Server deduplicates.

(iii) Comparison with Two Alternatives

| Protocol | Why NOT Suitable |
| --- | --- |
| Wi-Fi (IEEE 802.11) | Maximum range 100–200 m per access point. Covering 20 km² requires dozens of APs with mains power — not feasible in a rural area with no infrastructure. Power consumption is 100–200 mA per device; battery would drain in days, not years. |
| ZigBee (IEEE 802.15.4) | Indoor/rural range per hop is 10–100 m. Even with mesh routing across many routers, total effective range is a few hundred metres at most. Wells separated by 500 m–1 km cannot form a ZigBee mesh without hundreds of powered relay nodes deployed across the entire district. |

SET A | Q6 (OR).  Explain ZigBee architecture in detail. Include the three device types, three network topologies, ZigBee protocol stack layers, and one real-world IoT application.   [CO3]  [K2]  [9 Marks]

Answer:

(i) Three Device Types

ZigBee Coordinator (ZC): One per PAN. Initiates the network, assigns addresses, stores AES-128 security keys. Acts as root of the network. Always on.

ZigBee Router (ZR): Full-Function Device. Routes messages between nodes, extends coverage. Accepts child devices. Always on. Multiple routers allowed.

ZigBee End Device (ZED): Reduced-Function Device. Communicates only with its parent. Cannot route. Sleeps to save battery. Used as sensors and actuators.

(ii) Three Network Topologies

| Topology | Description | Advantage | Limitation |
| --- | --- | --- | --- |
| Star | All end devices connect directly to coordinator only. No routers. | Simple, low latency. | Single point of failure; range limited to coordinator coverage. |
| Tree | Coordinator is root. Routers are branches. End devices are leaves. | Extends range beyond star. | Router failure disconnects all its children. |
| Mesh | Routers communicate with any router in range. Multiple paths available. AODV routing. | Self-healing: alternate route found automatically when a router fails. | More complex routing tables. |

(iii) ZigBee Protocol Stack

| Layer | Function |
| --- | --- |
| Application Layer | Hosts ZigBee application profiles (Home Automation, Smart Energy, ZigBee Light Link). Custom vendor profiles also supported. |
| APS (Application Support Sub-layer) | Binds input/output clusters, manages group addressing, provides interface between NWK and Application. |
| NWK (Network Layer) | AODV routing across mesh. Device discovery. PAN association. AES-128 security management. |
| IEEE 802.15.4 MAC | Channel access via CSMA-CA. Beaconing. Reliable frame delivery with acknowledgements. PAN association requests. |
| IEEE 802.15.4 PHY | Wireless transmission at 2.4 GHz (16 channels, 250 kbps), 915 MHz, 868 MHz using DSSS modulation. |

(iv) Real-World Application: Smart Home Lighting

Philips Hue smart lighting uses ZigBee. The Hue Bridge acts as Coordinator. Always-on smart bulbs act as Routers, forming a mesh across rooms. Motion sensors and dimmer switches act as End Devices and sleep between events. The mesh ensures that if one bulb is removed, commands reach other bulbs through alternate router paths. A smartphone app communicates via Wi-Fi to the Bridge, which translates commands into ZigBee frames.

SET B | Q5.  Explain LoRaWAN in detail with one real-world IoT application of LoRaWAN with justification.   [CO3]  [K1]  [9 Marks]

Answer:

(i) What is LoRaWAN?

LoRaWAN (Long Range Wide Area Network) is an unlicensed-band LPWA (Low Power Wide Area) technology optimised for long-range, two-way communications and low power consumption. Published by the LoRa Alliance. LoRa is the physical layer modulation (developed by Cycleo, acquired by Semtech); LoRaWAN refers to the MAC layer specifications and end-to-end architecture.

(ii) Physical Layer — Chirp Spread Spectrum (CSS)

LoRa modulation is based on CSS (Chirp Spread Spectrum). A chirp is a signal that sweeps linearly in frequency over a fixed time period. CSS properties: signals can be demodulated even below the noise floor; a single gateway can simultaneously receive signals from many end nodes on different spreading factors.

| Spreading Factor | Data Rate | Range | Battery Impact |
| --- | --- | --- | --- |
| SF7 (Low SF) | ~5.5 kbps (fastest) | ~2 km | Least air time — lowest energy per packet |
| SF10 | ~1.0 kbps | ~8 km | Moderate |
| SF12 (High SF) | ~250 bps (slowest) | ~15 km+ | Most air time — highest energy per packet |

(iii) MAC Layer — Three Device Classes

| Class | Receive Windows | Power Profile | Best For |
| --- | --- | --- | --- |
| Class A (default) | Two short windows opened immediately after each uplink only. Device sleeps all other times. | Lowest — device sleeps most of the time. | Battery-powered sensors: water meters, agriculture, logistics trackers. |
| Class B | Scheduled downlink windows at beacon-synchronised intervals, in addition to Class A windows. | Medium. | Devices needing regular downlink commands: smart valves. |
| Class C | Receive window always open except during uplink transmission. | Highest — never truly sleeps. | Mains-powered devices: smart street lights. |

(iv) Network Topology — Star of Stars

End Nodes → LoRa Gateways (transparent bridges, relay to IP backbone) → Network Server (deduplication, ADR management) → Application Server (data processing, dashboards). Multiple gateways may receive the same uplink; Network Server selects the best copy. Downlinks go from Application Server → Network Server → one Gateway → End Node.

(v) Real-World Application: Smart Water Metering

Municipalities use LoRaWAN for smart water meter reading. Each meter is a Class A end node that transmits its daily reading (meter ID + reading = ~10 bytes). A single gateway covers an entire urban district (8–10 km radius). Battery life exceeds 10 years because each transmission is only a few seconds of air time at SF10. No on-site personnel needed. Application Server aggregates readings and detects leakage by comparing network input volume vs. billed volume.

Justification: LoRaWAN chosen over NB-IoT (needs cellular operator, SIM cost) and Wi-Fi (100 m range, high power). LoRaWAN provides: long range, low power, unlicensed band, private infrastructure ownership, and low payload size match.

SET B | Q6 (OR).  A private hospital has 300 rooms across 6 floors. Each room has an occupancy sensor for automatic lighting, a temperature sensor for HVAC control, and a nurse call button. The network must keep working even if individual nodes fail and must allow new devices to be added without rewiring. (i) Identify the protocol and justify. (ii) Explain the complete network architecture, topology, self-healing, and adding devices. (iii) Compare with Wi-Fi and Bluetooth.   [CO3]  [K3]  [9 Marks]

Answer:

(i) Protocol: ZigBee — Justification

| Requirement | How ZigBee Satisfies It |
| --- | --- |
| 300 rooms, 900+ devices | ZigBee supports up to 65,000 devices per PAN using 16-bit short addressing. 3 devices × 300 rooms = 900 devices is well within capacity. |
| Self-healing if node fails | ZigBee mesh uses AODV routing. If a router fails, remaining routers automatically broadcast Route Request (RREQ) and discover alternate paths within seconds. |
| No rewiring for new devices | ZigBee is entirely wireless. A new device powers on, broadcasts an association request, and the nearest router or coordinator accepts it into the PAN. |
| Indoor range | ZigBee indoor range per hop: 10–30 m. Routers in corridors relay signals across all 6 floors and all wings. |
| Battery-powered sensors | End Devices (occupancy sensors, temperature sensors) sleep between readings — battery life 2–5 years. |

(ii) Network Architecture

ZigBee Coordinator: One unit installed in the Building Management Room. Manages the PAN for the entire hospital (all 6 floors). Connected via USB to the Hospital Management Software. Assigns addresses to all 900+ devices.

ZigBee Routers: Installed at every corridor junction on each floor. Plugged into mains power (must remain on 24/7). Each router covers rooms in its section (10–30 m range, ~5–8 rooms). They form the mesh backbone.

ZigBee End Devices: Three per room — (a) occupancy sensor (PIR, sleeps, wakes on motion), (b) temperature sensor (wakes every 5 minutes), (c) nurse call button (wakes only on press). All battery-powered.

Self-healing: If Corridor Router C fails, end devices previously routing through it discover Router B or Router D via AODV Route Request broadcast. Network re-establishes within 5–10 seconds automatically, without any staff intervention.

Adding a new device: A new nurse call button is placed in a newly opened room. It powers on, broadcasts a beacon scan, finds the nearest router, sends an association request, and receives a 16-bit address from the coordinator within 30 seconds. Zero rewiring.

(iii) Comparison with Wi-Fi and Bluetooth

| Feature | Wi-Fi Limitations | Bluetooth Limitations |
| --- | --- | --- |
| Device scale | Each AP handles 30–50 active devices. 900 devices require 20+ enterprise APs — very high infrastructure cost. | BT Classic: point-to-point only. BT Mesh: community spec limits to ~100 nodes. Cannot scale to 900 hospital devices. |
| Power consumption | Very high — 100–200 mA active transmit. Battery-powered sensors would need replacement every few weeks. | BT LE lower than Wi-Fi but still 5–10× higher sleep current than ZigBee End Device. Battery life significantly shorter. |
| Self-healing | Wi-Fi does not self-heal. If an AP fails, its coverage zone loses connectivity until IT staff intervene. | BT Mesh has limited self-healing. Not designed for 300-room, 900-device building-wide deployment. |

MODULE 4 — Raspberry Pi Interfacing and Python Programming

PART A — 3 Mark Questions (Module 4)

SET A | Q3.  Why is a pull-up or pull-down resistor needed when a push button is connected to Raspberry Pi GPIO? Give one example with a circuit description.   [CO4]  [K1]  [3 Marks]

Answer:

When a GPIO pin is configured as input and the button is not pressed, the pin is not connected to any defined voltage. It is called a floating pin. A floating pin picks up random electrical noise from nearby signals or the environment, causing it to read HIGH or LOW unpredictably and producing false button detections.

Pull-up resistor: pin connected to 3.3 V via resistor. Button idle → pin reads HIGH. Button pressed (GPIO to GND) → pin reads LOW.

Pull-down resistor: pin connected to GND via resistor. Button idle → pin reads LOW. Button pressed (GPIO to 3.3 V) → pin reads HIGH.

Example — Button with internal pull-up on GPIO25:

Circuit: One leg of button → GPIO25 (Pin 22). Other leg → GND (Pin 6). No external resistor — internal pull-up enabled in software.

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)

GPIO.setup(25, GPIO.IN, pull_up_down=GPIO.PUD_UP)

if GPIO.input(25) == GPIO.LOW:   # button pressed

print("Button Pressed")

GPIO25 reads HIGH when button is open (pulled to 3.3 V internally). GPIO25 reads LOW when button is pressed (connected to GND).

SET A | Q4.  What is pyserial? Write three lines of Python code to open UART on /dev/ttyS0 at 9600 baud and read one line of data.   [CO4]  [K1]  [3 Marks]

Answer:

pyserial is a Python library that provides cross-platform access to the serial port (UART) on Raspberry Pi. It allows Python programs to communicate with external devices (GPS modules, Arduino, sensors) through the TX (GPIO14, Pin 8) and RX (GPIO15, Pin 10) pins using the device file /dev/ttyS0. It supports configuring baud rate, parity, stop bits, and timeout. Install: pip3 install pyserial

import serial

ser = serial.Serial('/dev/ttyS0', 9600, timeout=1)

data = ser.readline()

Line 1: imports the pyserial library. Line 2: opens UART port at 9600 baud with 1-second timeout. Line 3: reads one complete line (waits until newline or timeout), returns raw bytes.

Note: Enable UART first: sudo raspi-config → Interface Options → Serial Port → Disable login shell → Enable hardware UART → Reboot.

SET B | Q3.  Explain how a serial device communicates with Raspberry Pi. State UART GPIO pins, library used, and write two Python lines to send a string over UART.   [CO4]  [K2]  [3 Marks]

Answer:

Serial (UART) communication is asynchronous — no shared clock. Both devices must be configured to the same baud rate (e.g. 9600 bps). Data is framed as: 1 start bit, 8 data bits, optional parity, 1–2 stop bits.

| Signal | GPIO Pin | Header Pin | Direction |
| --- | --- | --- | --- |
| TX (Transmit) | GPIO14 | Pin 8 | RPi sends data → Device receives |
| RX (Receive) | GPIO15 | Pin 10 | Device sends data → RPi receives |
| GND | Any GND | Pin 6 | Shared common ground — mandatory |

Python library: pyserial. Install: pip3 install pyserial. Enable via: raspi-config → Serial Port.

import serial

ser = serial.Serial('/dev/ttyS0', 9600, timeout=1)

ser.write(b'Hello\n')     # b prefix converts string to bytes

SET B | Q4.  What is BCM numbering mode in RPi.GPIO? Write a Python snippet configuring GPIO18 as output and GPIO23 as input with pull-down resistor.   [CO4]  [K3]  [3 Marks]

Answer:

RPi.GPIO supports two pin numbering modes:

BCM mode (GPIO.BCM): uses the Broadcom SoC GPIO numbers (GPIO2, GPIO17, GPIO18, GPIO23...) as labelled in all pinout diagrams and datasheets. This is the standard mode used by most tutorials and libraries.

BOARD mode (GPIO.BOARD): uses the physical pin numbers on the 40-pin header connector (1 to 40). Depends on the physical board layout, not the chip.

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)                           # BCM pin numbering

GPIO.setwarnings(False)

GPIO.setup(18, GPIO.OUT)                         # GPIO18 as output (LED)

GPIO.setup(23, GPIO.IN, pull_up_down=GPIO.PUD_DOWN)  # GPIO23 input + pull-down

With GPIO.PUD_DOWN: GPIO23 reads LOW when nothing is connected and HIGH when signal source drives it to 3.3 V.

PART B — 9 Mark Questions (Module 4)

SET A | Q7.  Write a complete Python program using RPi.GPIO to control an LED with a push button. LED ON when pressed, OFF when released. Include wiring, GPIO setup, main loop, exception handling, and cleanup.   [CO4]  [K3]  [9 Marks]

Answer:

(i) Wiring Description

LED circuit: GPIO17 (Pin 11) → 330 Ω resistor → LED anode (+) → LED cathode (−) → GND (Pin 6). The 330 Ω resistor limits current to ~10 mA, protecting the GPIO pin (max 16 mA).

Button circuit: 3.3 V (Pin 1) → push button → GPIO25 (Pin 22). Internal pull-down enabled in software, so GPIO25 reads LOW when button is open and HIGH when button is pressed (connects GPIO25 to 3.3 V).

(ii) Complete Python Program

import RPi.GPIO as GPIO

import time

# (i) GPIO Setup

GPIO.setmode(GPIO.BCM)           # BCM pin numbering

GPIO.setwarnings(False)

LED_PIN = 17                     # GPIO17 — LED output

BTN_PIN = 25                     # GPIO25 — push button input

GPIO.setup(LED_PIN, GPIO.OUT)    # GPIO17 as output

GPIO.output(LED_PIN, GPIO.LOW)   # LED OFF at startup

GPIO.setup(BTN_PIN, GPIO.IN,

pull_up_down=GPIO.PUD_DOWN)  # internal pull-down on GPIO25

print("Ready. Press button to turn LED ON. Ctrl+C to quit.")

# (ii) Main Loop

try:

while True:

if GPIO.input(BTN_PIN) == GPIO.HIGH:   # button pressed

GPIO.output(LED_PIN, GPIO.HIGH)     # LED ON

else:                                   # button released

GPIO.output(LED_PIN, GPIO.LOW)      # LED OFF

time.sleep(0.05)    # 50 ms debounce

# (iii) Exception Handling

except KeyboardInterrupt:

print("Stopped.")

# (iv) Cleanup

finally:

GPIO.output(LED_PIN, GPIO.LOW)  # LED OFF on exit

GPIO.cleanup()                  # reset all GPIO pins to input/floating

print("GPIO cleaned up.")

SET A | Q8 / SET B 7.  Explain the I2C communication interface of Raspberry Pi. Include SDA/SCL pins, 7-bit addressing, maximum devices, enabling steps, and complete Python code using smbus2.   [CO4]  [K2]  [9 Marks]

Answer:

(i) SDA and SCL GPIO Pins

| Signal | GPIO Pin | Header Pin | Description |
| --- | --- | --- | --- |
| SDA (Serial Data) | GPIO2 | Pin 3 | Bidirectional data line — carries all address and data bytes. |
| SCL (Serial Clock) | GPIO3 | Pin 5 | Clock generated by master (RPi). All devices synchronise on this clock. |

Both lines are open-drain — devices can only pull the line LOW, never drive it HIGH. Pull-up resistors (4.7 kΩ to 3.3 V) are required. Most sensor breakout boards include these.

(ii) 7-bit Addressing and Maximum Devices

Every I2C slave has a unique 7-bit address. With 7 bits, 128 addresses (0x00 to 0x7F) are possible. Addresses 0x00–0x07 and 0x78–0x7F are reserved by the I2C specification. This leaves up to 112 usable device addresses. The master (RPi) begins every transaction by broadcasting the 7-bit address + 1 direction bit (R/W). Only the matching slave responds — all others ignore the frame. This is how up to 112 different sensors, displays, and peripherals share the same two-wire SDA/SCL bus.

(iii) Enabling I2C on Raspberry Pi

Step 1: sudo raspi-config → Interfacing Options → I2C → Yes → Finish → Reboot.

Step 2: Verify: ls /dev/i2c*  (should show /dev/i2c-1)

Step 3: pip3 install smbus2

Step 4: Scan for devices: sudo i2cdetect -y 1

(iv) Complete Python Code Using smbus2

import smbus2

import time

bus  = smbus2.SMBus(1)    # Open I2C bus 1 (/dev/i2c-1)

ADDR = 0x48               # 7-bit I2C address of sensor

def read_temperature():

data = bus.read_i2c_block_data(ADDR, 0x00, 2)   # 2 bytes from reg 0x00

raw = (data[0] << 4) | (data[1] >> 4)            # 12-bit raw value

if raw > 2047:   # two's complement for negative temperature

raw -= 4096

return raw * 0.0625   # LSB = 0.0625 degrees C

try:

while True:

temp = read_temperature()

print(f"Temperature: {temp:.2f} C")

time.sleep(5)

except KeyboardInterrupt:

print("Stopped.")

finally:

bus.close()

Key point: I2C uses only 2 wires for up to 112 devices (vs SPI which needs 4 wires + 1 CS per device). I2C is half-duplex (shared SDA) at up to 400 kbps; SPI is full-duplex at up to 50 Mbps.

SET B | Q8 (OR).  Design a weather monitoring station using Raspberry Pi and SHT45 I2C sensor. Read temperature and humidity every 5 seconds. Activate LED alarm on GPIO17 when temperature exceeds 35°C. Include I2C setup, sensor read function with formula, main loop, exception handling, and cleanup.   [CO4]  [K3]  [9 Marks]

Answer:

(i) Hardware Wiring

| Connection | Detail |
| --- | --- |
| SHT45 SDA → GPIO2 (Pin 3) | I2C data line |
| SHT45 SCL → GPIO3 (Pin 5) | I2C clock line |
| SHT45 VCC → 3.3 V (Pin 1) | Power supply |
| SHT45 GND → GND (Pin 6) | Common ground |
| LED anode (+) → 330 Ω → GPIO17 (Pin 11) | Alarm LED output |
| LED cathode (−) → GND (Pin 9) | LED return |

SHT45 I2C address: 0x44 (factory default). High-precision measurement command: 0xFD. Measurement time: 10 ms.

(ii) Sensor Read Function and Conversion Formula

SHT45 returns 6 bytes: bytes 0–1 = raw temperature, byte 2 = CRC, bytes 3–4 = raw humidity, byte 5 = CRC.

Temperature formula: T (°C) = −45 + 175 × (raw_T / 65535)

Humidity formula:    RH (%) = 100 × (raw_H / 65535)

import smbus2

import RPi.GPIO as GPIO

import time

# Setup

bus         = smbus2.SMBus(1)

SHT45_ADDR  = 0x44

CMD_MEASURE = 0xFD         # high-precision single-shot measurement

ALARM_TEMP  = 35.0         # alarm threshold in degrees C

LED_PIN     = 17

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(LED_PIN, GPIO.OUT)

GPIO.output(LED_PIN, GPIO.LOW)

def read_sht45():

bus.write_byte(SHT45_ADDR, CMD_MEASURE)   # trigger measurement

time.sleep(0.01)                           # 10 ms measurement time

raw = bus.read_i2c_block_data(SHT45_ADDR, 0x00, 6)

raw_t = (raw[0] << 8) | raw[1]

temp  = -45 + 175 * (raw_t / 65535.0)     # temperature formula

raw_h = (raw[3] << 8) | raw[4]

rh    = 100 * (raw_h / 65535.0)            # humidity formula

return round(temp, 2), round(rh, 1)

print("Weather Station Active. Ctrl+C to stop.")

try:

while True:

temperature, humidity = read_sht45()

print(f"Temp: {temperature} C  |  RH: {humidity} %")

if temperature > ALARM_TEMP:

GPIO.output(LED_PIN, GPIO.HIGH)    # LED ON — ALARM

print(f"ALARM: {temperature} C exceeds {ALARM_TEMP} C")

else:

GPIO.output(LED_PIN, GPIO.LOW)     # LED OFF — safe

time.sleep(5)                          # read every 5 seconds

except KeyboardInterrupt:

print("Station stopped.")

finally:

GPIO.output(LED_PIN, GPIO.LOW)

GPIO.cleanup()

bus.close()

print("Cleanup done.")

End of Model Answer Key — PEMRT525 IoT and Applications | MR 2K24 S5 | JECC