Reg No.: ____________________                                              Name : ____________________

Jyothi Engineering College (Autonomous)

DEPARTMENT OF MECHATRONICS ENGINEERING

Vth Semester - B.Tech

First Internal Examination, Aug 2026

13-Aug-2026

PEMRT525 - IOT AND APPLICATIONS

| Total Mark: 30 | CODE: MRU5X130826F | Total Time: 1 Hr 15 Min |
| --- | --- | --- |

| CO1 | Apply IoT fundamentals to model and explain IoT architectural views | K3 |
| --- | --- | --- |
| CO2 | Apply knowledge of smart objects, sensors, actuators, and functional blocks in IoT systems | K3 |

CO  KL  MARK

PART A

| 1. Recall the Publish-Subscribe communication model. Identify the three roles and the protocol that implements it. State one key advantage over Request-Response. | CO1 K1 (3) |
| --- | --- |

| 2. Identify which IoT functional block is responsible for authentication, encryption, and data integrity. Explain why this block is described as a 'cross-cutting concern' across all other blocks. | CO1 K2 (3) |
| --- | --- |

| 3. Define a smart object. List its four components. Describe how combining these components creates the 'smart' in a smart object. | CO2 K1 (3) |
| --- | --- |

| 4. Identify and describe the criteria you would consider when planning a WSN deployment for a smart agriculture field monitoring system. | CO2 K2 (3) |
| --- | --- |

PART B

| 5. Apply your knowledge of three IoT enabling technologies — Wireless Sensor Networks (WSN), Cloud Computing, and Big Data Analytics — to demonstrate how each addresses a specific IoT challenge. Illustrate how they form an integrated IoT pipeline with a suitable example. | CO1 K3 (9) |
| --- | --- |

OR

| 6. Apply the IoT deployment model of Bahga & Madisetti to classify any three deployment levels. Use your understanding to determine the most suitable level for a multi-building campus energy monitoring system and justify your selection. | CO1 K3 (9) |
| --- | --- |

| 7. Apply the WSN protocol stack framework to describe IEEE 802.15.4, ZigBee, and 6LoWPAN at each layer. Use five communication criteria to select and justify the most suitable wireless technology for a smart hospital patient monitoring system. | CO2 K3 (9) |
| --- | --- |

OR

| 8. Illustrate WSN architecture with a labelled diagram. Apply the FFD and RFD classification to describe node capabilities and their differences. Use knowledge of the six design constraints of WSN nodes to explain how data aggregation and self-organisation address these constraints. | CO2 K3 (9) |
| --- | --- |