# MODULE 1 STUDY NOTES

Internet of Things & Applications (PEMRT525)

KTU | S5 | MR 2K24 | JECC

Based on: Bahga & Madisetti — "Internet of Things: A Hands-On Approach"

Topics Covered

1. Introduction to IoT — Definitions & Characteristics

2. IoT Architectural View

3. Physical Design of IoT

4. Logical Design of IoT — Functional Blocks

5. IoT Communication Models

6. IoT Enabling Technologies

7. IoT Levels & Deployment Templates

# TOPIC 1: Introduction to IoT — Definitions & Characteristics

Think of IoT like this: imagine your refrigerator can tell you when milk is finished, your AC switches on before you reach home, and your doctor monitors your heart remotely — all through the Internet. That is the Internet of Things.

IoT (Internet of Things): A dynamic global network infrastructure with self-configuring capabilities based on standard and interoperable communication protocols where physical and virtual "things" have unique identities and are connected to the Internet, and often communicate data associated with users and their environments.

💡 Key Point: IoT = Things + Internet + Intelligence. Physical devices connected to the Internet that can sense, communicate, and act.

## Why is IoT a Revolution?

Traditional internet connected computers and phones. IoT connects everyday objects — thermostats, cars, streetlights, heart monitors — to the Internet. By 2020, over 50 billion devices are expected to be connected.

## IoT Application Domains

| Domain | Examples |
| --- | --- |
| Home | Smart lighting, smart appliances, intrusion detection, smoke detectors |
| Cities | Smart parking, smart roads, surveillance, emergency response |
| Environment | Weather monitoring, air quality, forest fire detection, flood monitoring |
| Energy | Smart grids, renewable energy integration, prognostic health management |
| Retail | Inventory management, smart payments, smart vending machines |
| Agriculture | Smart irrigation, greenhouse control |
| Industry | Machine diagnosis, indoor air quality monitoring |
| Health | Wearable health monitors, fitness trackers |
| Logistics | Fleet tracking, shipment monitoring, route scheduling |

## Characteristics of IoT

Think of the acronym DSIUI — Dynamic, Self-configuring, Interoperable, Unique Identity, Integrated.

### 1. Dynamic & Self-Adapting

IoT devices can adapt their behaviour based on context and changing conditions.

📌 Example: Surveillance cameras switch from normal mode to infrared night mode automatically when it gets dark, and switch to high resolution when motion is detected.

### 2. Self-Configuring

IoT devices can configure themselves, set up networking, and download software updates without manual intervention.

📌 Example: A large weather monitoring network of 500 sensor nodes automatically arranges itself without manual setup.

### 3. Interoperable Communication Protocols

IoT devices support multiple communication protocols and can communicate with other devices and infrastructure.

📌 Example: A smart home device can communicate using both WiFi and Bluetooth.

### 4. Unique Identity

Each IoT device has a unique ID (like an IP address or URI) so it can be identified and addressed individually on the network.

### 5. Integrated into Information Network

IoT devices are part of a larger information network — they can describe themselves, be discovered by other devices, and exchange data.

📌 Example: A weather node tells another node about its temperature sensing capability so they can collaborate.

⚠ Remember: 5 key characteristics of IoT: Dynamic, Self-Configuring, Interoperable, Unique Identity, Network-Integrated.

# TOPIC 2: IoT Architectural View

IoT architecture describes how all components of an IoT system are organized and work together. Think of it like the blueprint of a building — it shows what goes where and how everything connects.

## The Data → Information → Knowledge Flow

Raw data from sensors is meaningless by itself. IoT systems process it into useful knowledge:

| Level | What it is | Example |
| --- | --- | --- |
| Data | Raw sensor readings | (72, 45) ; (84, 56) |
| Context | Meaning added to data | Each tuple = (temperature °F, humidity %) |
| Information | Processed data | Average temperature last 5 min = 78°F |
| Knowledge | Actionable insight | Alert! Temperature > 120°F — fire risk detected |

## IoT System Architecture — 4 Layers

| Layer | Function | Components |
| --- | --- | --- |
| Sensing Layer | Collect data from the physical world | Sensors, actuators, RFID tags, IoT devices |
| Network Layer | Transmit data to the processing layer | WiFi, 4G/5G, ZigBee, Ethernet, gateways |
| Processing Layer | Store, analyze, and process data | Cloud servers, databases, analytics engines |
| Application Layer | Deliver services to end users | Mobile apps, web dashboards, control panels |

💡 Key Point: The architecture moves from physical world (sensors) → network → processing → user applications. Data flows upward; commands flow downward.

## Key Architectural Components

Things/Devices: The physical objects with sensors and connectivity (e.g., a smart thermostat).

Gateway: A bridge that connects IoT devices to the Internet (e.g., a home WiFi router connecting smart bulbs to the cloud).

Cloud/Backend: Where data is stored, processed, and analyzed at scale.

Applications: User-facing software (mobile app, web portal) that presents insights and allows control.

Network: The communication infrastructure connecting devices to gateways and cloud.

📌 Example: Smart irrigation: Soil moisture sensor (Thing) → WiFi router (Gateway) → AWS Cloud (Processing) → Farmer's mobile app (Application).

⚠ Remember: IoT architecture = 4 layers: Sensing → Network → Processing → Application. Data goes up, commands come down.

# TOPIC 3: Physical Design of IoT

Physical design refers to the actual hardware components of an IoT system — the devices and the communication protocols they use to connect and exchange data.

## 3.1 Things in IoT (IoT Devices)

An IoT device is a physical object equipped with sensors, processors, and communication capabilities.

### What an IoT Device Can Do:

Sense: Collect data from environment (temperature, humidity, motion, etc.)

Process: Analyse data locally or send to cloud

Communicate: Send/receive data over network

Actuate: Control physical things (switch on a motor, open a valve)

### Interfaces of an IoT Device:

| Interface Type | Purpose | Example |
| --- | --- | --- |
| I/O for Sensors | Connect sensors and actuators | Temperature sensor, relay switch |
| Internet Connectivity | Connect to network/Internet | WiFi module, Ethernet port |
| Memory & Storage | Store data locally | SD card slot, flash memory |
| Audio/Video | Capture/play media | Camera module, microphone |

📌 Example: A smart home device (like Raspberry Pi) has GPIO pins for sensors, a WiFi chip for Internet, SD card for storage, and a camera port for video.

## 3.2 IoT Protocols — Communication Stack

IoT devices communicate using a layered protocol stack (similar to how the Internet works). Each layer handles a specific part of the communication.

### Layer 1: Link Layer (Physical Communication)

Determines how data is physically transmitted between devices on the same local network.

| Protocol | Type | Speed | Use Case |
| --- | --- | --- | --- |
| 802.3 (Ethernet) | Wired | 10 Mb/s – 40 Gb/s | Wired LAN connections |
| 802.11 (WiFi) | Wireless | 1 Mb/s – 6.75 Gb/s | Home/office wireless networks |
| 802.16 (WiMax) | Wireless Broadband | 1.5 Mb/s – 1 Gb/s | Wide area broadband |
| 802.15.4 (LR-WPAN) | Low-Power Wireless | 40 – 250 Kb/s | ZigBee, sensor networks |
| 2G/3G/4G | Mobile Cellular | 9.6 Kb/s – 100 Mb/s | Mobile IoT devices |

💡 Key Point: For IoT sensor networks, 802.15.4 (ZigBee) is most popular because it is low-power and low-cost.

### Layer 2: Network/Internet Layer (Addressing & Routing)

Handles addressing of devices and routing of data packets from source to destination.

| Protocol | Address Size | Total Addresses | Notes |
| --- | --- | --- | --- |
| IPv4 | 32-bit | ~4.3 billion | Most deployed; addresses exhausted in 2011 |
| IPv6 | 128-bit | 3.4 × 10³⁸ | Successor to IPv4; handles IoT scale |
| 6LoWPAN | 128-bit (IPv6) | Same as IPv6 | IPv6 for low-power devices (works with 802.15.4) |

⚠ Remember: IPv4 has only ~4 billion addresses — not enough for 50 billion IoT devices! That is why IPv6 is needed.

### Layer 3: Transport Layer (End-to-End Communication)

| Protocol | Connection | Reliability | Best For |
| --- | --- | --- | --- |
| TCP | Connection-oriented (stateful) | Guaranteed delivery, ordered | Web browsing, email, file transfer |
| UDP | Connectionless (stateless) | No guarantee, faster | Video streaming, real-time IoT data |

### Layer 4: Application Layer (App-to-App Communication)

| Protocol | Model | Key Feature | IoT Use |
| --- | --- | --- | --- |
| HTTP | Request-Response | Foundation of WWW; GET/POST/PUT/DELETE | REST APIs for IoT web services |
| CoAP | Request-Response | Like HTTP but runs on UDP; for constrained devices | Sensor networks, smart energy |
| WebSocket | Full Duplex | Persistent connection; bi-directional | Real-time IoT dashboards |
| MQTT | Publish-Subscribe | Lightweight; broker-based; low bandwidth | Home automation, telemetry |
| XMPP | Client-Server | Real-time XML messaging | Real-time device communication |
| DDS | Publish-Subscribe | Data-centric; QoS control | Industrial IoT, defense |
| AMQP | Pub-Sub / Point-to-Point | Business messaging; queuing | Enterprise IoT systems |

💡 Key Point: MQTT is the most widely used IoT application protocol because it is lightweight and works on low-bandwidth networks. Think of it as WhatsApp for IoT devices.

# TOPIC 4: Logical Design of IoT — Functional Blocks

Logical design is the abstract view of the IoT system — it describes WHAT the system does (not HOW it is built in hardware). Think of it as a software architecture diagram.

## 4.1 IoT Functional Blocks

An IoT system has 6 main functional blocks that work together:

| Functional Block | What it Does | Real-World Example |
| --- | --- | --- |
| Device | Performs sensing, actuation, monitoring, and control | Temperature sensor, relay switch, camera |
| Communication | Handles all communication between components | WiFi module, MQTT broker, REST API |
| Services | Provides IoT services: monitoring, control, data publishing, device discovery | Cloud service that logs temperature every minute |
| Management | Governs the IoT system — configuration, updates, monitoring | Device management platform (e.g., AWS IoT Core) |
| Security | Authentication, authorization, data integrity, encryption | TLS encryption, username/password login |
| Application | User interface to monitor, control, and view data | Mobile app, web dashboard |

💡 Key Point: Remember all 6 blocks: Device → Communication → Services → Management → Security → Application. Think: "Does Computer Systems Make Security Applications".

## 4.2 Detailed Explanation of Each Block

### 1. Device Block

Provides sensing (collecting data from environment)

Provides actuation (controlling physical devices)

Can be wearable sensors, industrial machines, smart appliances, etc.

### 2. Communication Block

Uses protocols like MQTT, HTTP, CoAP, WebSocket

Manages how data flows between devices, cloud, and applications

### 3. Services Block

Device Monitoring Services: Track device status

Device Control Services: Send commands to devices

Data Publishing Services: Publish sensor data to subscribers

Device Discovery Services: Find and register new devices

### 4. Management Block

Device configuration and provisioning

Firmware/software updates

Performance monitoring

Fault management

### 5. Security Block

Authentication: Verify identity of devices and users

Authorization: Control who can access what

Data Integrity: Ensure data is not tampered with

Encryption: Protect data in transit and at rest

### 6. Application Block

Smart home apps, industrial dashboards, health monitoring portals

Provides visualization of data (charts, graphs, maps)

Allows remote control of devices

⚠ Remember: Security is a cross-cutting concern — it applies to ALL other functional blocks, not just one layer.

# TOPIC 5: IoT Communication Models

Communication models describe how data flows between entities in an IoT system. There are 4 main models:

## Model 1: Request-Response

The client asks, the server answers. Like asking a question and getting an answer.

Client sends a request to the server

Server processes the request and sends back a response

Each request-response is independent (stateless)

Used by: HTTP, CoAP, REST APIs

📌 Example: Your phone asks the weather server: "What is the temperature?" Server replies: "32°C". Each question is independent.

💡 Key Point: Request-Response is STATELESS — the server does not remember past requests.

## Model 2: Publish-Subscribe

Like a magazine subscription — publishers send content, subscribers receive it, and a broker (like a post office) handles distribution.

Publishers: Devices that generate data (e.g., temperature sensor)

Broker: Manages topics and routes messages (e.g., MQTT broker)

Subscribers: Devices/apps that consume data (e.g., monitoring dashboard)

Publishers do NOT know who the subscribers are

Used by: MQTT, AMQP, DDS

📌 Example: A temperature sensor publishes to topic "/home/livingroom/temp". A mobile app and a thermostat both subscribe to this topic and receive updates automatically.

💡 Key Point: Publish-Subscribe DECOUPLES producers from consumers — they do not need to know each other.

## Model 3: Push-Pull (Pipeline)

Like a factory conveyor belt — producers push items, consumers pick them up when ready.

Producers push data into queues

Consumers pull data from queues when they are ready

Queues act as buffers (handle speed differences between producers and consumers)

Producers do NOT need to know who the consumers are

📌 Example: Sensor data is pushed to a message queue. A data analytics engine pulls and processes data at its own pace, even if the sensor is generating data much faster.

💡 Key Point: Push-Pull is great when producers and consumers work at DIFFERENT SPEEDS — queues prevent data loss.

## Model 4: Exclusive Pair

Like a phone call — a persistent, dedicated connection between two parties.

Client sends a connection setup request to server

Connection stays open until client sends a close request

Both client and server can send messages at any time (full-duplex)

Server is aware of all open connections (stateful)

Used by: WebSocket

📌 Example: A live dashboard showing real-time factory machine data — the browser maintains a persistent WebSocket connection to the server and both sides can send data anytime.

💡 Key Point: Exclusive Pair = STATEFUL + FULL-DUPLEX. Best for real-time applications like live dashboards.

## Comparison of Communication Models

| Feature | Request-Response | Publish-Subscribe | Push-Pull | Exclusive Pair |
| --- | --- | --- | --- | --- |
| State | Stateless | Stateless | Stateless | Stateful |
| Direction | Client → Server | Many-to-Many | Producer → Queue → Consumer | Bi-directional |
| Connection | New per request | Via broker | Via queue | Persistent |
| Real-time? | No | Near real-time | Near real-time | Yes |
| Protocol | HTTP, CoAP | MQTT, AMQP | ZeroMQ | WebSocket |
| Example | REST API call | MQTT sensor data | Message queue | Live dashboard |

## REST vs WebSocket — Detailed Comparison

| Aspect | REST | WebSocket |
| --- | --- | --- |
| State | Stateless | Stateful |
| Direction | Uni-directional (client requests only) | Bi-directional (both sides can send) |
| Communication | Request-Response | Full Duplex |
| TCP Connection | New connection per request | Single persistent connection |
| Header Overhead | High (HTTP headers every request) | Low (only initial handshake) |
| Real-time | Not suitable | Highly suitable |
| Scalability | Easy (stateless = easy horizontal scaling) | Harder (stateful server) |
| Best For | CRUD operations, data retrieval | Live feeds, chat, real-time monitoring |

⚠ Remember: REST for standard data APIs. WebSocket for real-time, continuous data like live sensor dashboards.

# TOPIC 6: IoT Enabling Technologies

These are the key technologies that make IoT possible. Without these, IoT as we know it could not exist.

## Technology 1: Wireless Sensor Networks (WSN)

WSN: A network of distributed sensor devices (nodes) that wirelessly collect and transmit data about the physical environment.

### WSN Architecture:

| Component | Role |
| --- | --- |
| End Nodes | Have sensors attached; collect data; can also act as routers |
| Routers | Route data packets from end nodes to coordinator |
| Coordinator | Collects all data; acts as gateway to connect WSN to Internet |

### WSN Applications in IoT:

Weather monitoring — temperature, humidity sensors deployed across a region

Indoor air quality monitoring — detect harmful gas concentrations

Smart grids — monitor grid at multiple points

Structural health monitoring — detect vibrations in bridges and buildings

Surveillance — motion detection across a large area

WSNs use ZigBee (based on IEEE 802.15.4) — operates at 2.4 GHz, 250 Kb/s, range 10–100 m.

💡 Key Point: WSN superpower: Self-organizing. When nodes fail or new nodes join, the network reconfigures itself automatically.

## Technology 2: Cloud Computing

Cloud Computing: Delivering computing, storage, and networking resources over the Internet on demand, on a pay-as-you-go basis.

| Service Model | What User Gets | Who Manages Infrastructure | Example |
| --- | --- | --- | --- |
| IaaS (Infrastructure as a Service) | Virtual machines and storage | Cloud provider manages hardware; user manages OS & apps | AWS EC2, Google Compute Engine |
| PaaS (Platform as a Service) | Development tools, APIs, runtime environment | Cloud provider manages hardware + OS; user manages app | AWS Elastic Beanstalk, Heroku |
| SaaS (Software as a Service) | Ready-to-use software applications | Cloud provider manages everything | Gmail, Google Docs, Salesforce |

💡 Key Point: IaaS = Rent a computer. PaaS = Rent a workshop. SaaS = Rent a finished product.

📌 Example: IoT using Cloud: Smart city sensors send data to AWS IoT Core (PaaS), stored in DynamoDB (IaaS), viewed via a web dashboard (SaaS).

## Technology 3: Big Data Analytics

Big Data: Data sets so large, fast, or varied that traditional database tools cannot handle them.

### The 3 Vs of Big Data:

| V | Meaning | IoT Example |
| --- | --- | --- |
| Volume | Massive amount of data | Millions of temperature readings per day from city sensors |
| Velocity | Data generated at very high speed | Stock exchange data, real-time vehicle tracking every second |
| Variety | Different types/formats of data | Text logs, sensor numbers, video feeds, GPS coordinates |

📌 Example: A wind farm with 500 turbines each with 100 sensors generates terabytes of data daily — that is Big Data.

💡 Key Point: IoT devices generate massive amounts of data. Big Data analytics tools (like Apache Hadoop, Spark) are needed to extract useful insights.

## Technology 4: Communication Protocols

These form the backbone of IoT — without protocols, devices cannot talk to each other. You have already studied these in the Physical Design section (Link, Network, Transport, Application layers).

💡 Key Point: Key protocols to remember: WiFi, ZigBee (link); IPv6, 6LoWPAN (network); TCP, UDP (transport); MQTT, HTTP, CoAP, WebSocket (application).

## Technology 5: Embedded Systems

Embedded System: A computer system with dedicated hardware and software built to perform a specific task, unlike a general-purpose computer.

### Key Components of an Embedded System:

Microprocessor/Microcontroller: The brain (e.g., ARM Cortex, AVR, PIC)

Memory: RAM (working), ROM (program), Cache (fast access)

Networking: Ethernet/WiFi chips for connectivity

I/O Units: Sensors, display, keyboard, buttons

Storage: Flash memory, SD cards

Specialized Processors: DSP, GPU, ASIC (for specific tasks)

### Examples of Embedded Systems in IoT:

| Device | Embedded System Inside |
| --- | --- |
| Smart thermostat | Microcontroller + temperature sensor + WiFi chip |
| Smart watch | ARM processor + heart rate sensor + Bluetooth + display |
| Industrial robot | DSP + multiple sensors + motor controllers |
| Smart traffic light | Microcontroller + camera + 4G module |

⚠ Remember: Almost every IoT device IS an embedded system. Raspberry Pi, Arduino, NodeMCU are popular embedded platforms for IoT.

# TOPIC 7: IoT Levels & Deployment Templates

IoT systems vary in complexity. Bahga & Madisetti define 6 levels of IoT systems based on the number of nodes, where analysis happens, and where data is stored.

## Components of an IoT System (Quick Reference)

| Component | What It Is |
| --- | --- |
| Device | Physical IoT device — senses, actuates, monitors |
| Resource | Software on device to access sensors/actuators and enable networking |
| Controller Service | Runs on device; sends data to web services; receives commands from app |
| Database | Stores IoT data — local or cloud |
| Web Service | Links device, app, database, and analysis — REST or WebSocket |
| Analysis Component | Analyses IoT data; generates results — local or cloud |
| Application | User interface — monitor, control, visualize |

## The 6 IoT Levels

### Level 1 — Single Node, Everything Local

| Aspect | Details |
| --- | --- |
| Nodes | 1 single device/node |
| Analysis | Done locally on the device |
| Storage | Local database |
| Application | Hosted locally (can be accessed remotely) |
| Suitable For | Low complexity, low data volume, simple analysis |
| Example | Home automation — single device controls lights and appliances using relay switches; REST service updates appliance status in local database |

### Level 2 — Single Node, Data in Cloud

| Aspect | Details |
| --- | --- |
| Nodes | 1 single device/node |
| Analysis | Done locally on the device |
| Storage | Cloud database |
| Application | Cloud-based |
| Suitable For | Big data volume, but simple analysis that can be done locally |
| Example | Smart irrigation — sensor checks soil moisture locally, controls water valve; moisture data sent to cloud for history and visualization |

### Level 3 — Single Node, Cloud Analysis

| Aspect | Details |
| --- | --- |
| Nodes | 1 single device/node |
| Analysis | Done in the cloud |
| Storage | Cloud database |
| Application | Cloud-based |
| Suitable For | Big data + computationally intensive analysis |
| Example | Package tracking — device with accelerometer/gyroscope monitors vibrations; data sent to cloud via WebSocket; cloud triggers alerts if vibration exceeds threshold |

### Level 4 — Multiple Nodes + Observer Nodes

| Aspect | Details |
| --- | --- |
| Nodes | Multiple nodes + observer nodes |
| Analysis | Local + cloud |
| Storage | Cloud database |
| Application | Cloud-based |
| Suitable For | Multiple independent nodes, big data, intensive analysis; observer nodes receive but do not control |
| Example | Noise monitoring — multiple sound sensor nodes, each sends data to cloud independently; cloud aggregates data; observer nodes (apps/displays) subscribe to receive data |

### Level 5 — Multiple Nodes + Coordinator (WSN)

| Aspect | Details |
| --- | --- |
| Nodes | Multiple end nodes + 1 coordinator node |
| Analysis | Cloud |
| Storage | Cloud database |
| Application | Cloud-based |
| Suitable For | Wireless sensor network (WSN) scenarios; large-scale sensing; big data |
| Example | Forest fire detection — multiple nodes (temp, humidity, CO₂ sensors) send data to coordinator; coordinator forwards to cloud; cloud analyses for fire prediction |

### Level 6 — Multiple Independent Nodes + Centralized Controller

| Aspect | Details |
| --- | --- |
| Nodes | Multiple independent end nodes |
| Analysis | Cloud analytics component |
| Storage | Cloud database |
| Application | Cloud-based |
| Controller | Centralized controller sends commands to all nodes |
| Suitable For | Large-scale independent deployments with centralized control |
| Example | Weather monitoring — multiple independent nodes send real-time data via WebSocket to cloud; cloud analyses and visualizes; centralized controller sends calibration/control commands to nodes |

## Quick Summary Table — All 6 Levels

| Level | Nodes | Analysis | Storage | App | Key Example |
| --- | --- | --- | --- | --- | --- |
| 1 | Single | Local | Local | Local | Home Automation |
| 2 | Single | Local | Cloud | Cloud | Smart Irrigation |
| 3 | Single | Cloud | Cloud | Cloud | Package Tracking |
| 4 | Multiple + Observer | Local + Cloud | Cloud | Cloud | Noise Monitoring |
| 5 | Multiple + Coordinator | Cloud | Cloud | Cloud | Forest Fire Detection |
| 6 | Multiple Independent | Cloud | Cloud | Cloud | Weather Monitoring |

💡 Key Point: Pattern: As the level increases, complexity increases — more nodes, data moves to cloud, analysis moves to cloud.

⚠ Remember: Level 4 has OBSERVER NODES (they watch but don't control). Level 5 has a COORDINATOR NODE (collects from all end nodes). Level 6 has a CENTRALIZED CONTROLLER (commands all nodes).

# QUICK REVISION CARD — Module 1 at a Glance

## Must-Know Definitions

IoT: Network of uniquely identified, Internet-connected physical/virtual things that communicate data.

WSN: Distributed wireless sensor nodes + routers + coordinator that monitor physical conditions.

Cloud Computing: On-demand Internet delivery of computing, storage, and services on pay-per-use basis.

Big Data: Data sets too large/fast/varied for traditional tools — characterized by Volume, Velocity, Variety.

Embedded System: Computer hardware+software designed for a specific task (not general purpose).

MQTT: Lightweight publish-subscribe messaging protocol for IoT; runs over TCP; low bandwidth.

REST: Architectural style for web APIs; stateless; uses HTTP; GET/POST/PUT/DELETE.

WebSocket: Full-duplex, persistent TCP connection protocol for real-time bidirectional communication.

6LoWPAN: IPv6 for low-power wireless networks; enables IP on constrained IoT devices.

CoAP: Like HTTP but for constrained devices; runs over UDP instead of TCP.

## Exam-Likely Short Answer Questions

What is IoT? List its 5 characteristics.

Distinguish between IPv4 and IPv6 in the context of IoT.

Compare TCP and UDP.

What are the 4 IoT communication models? Explain each briefly.

Compare REST and WebSocket communication APIs.

List and explain the 6 functional blocks of an IoT system.

What is a Wireless Sensor Network? List its components.

Explain the 3 Vs of Big Data with examples.

What is the difference between IaaS, PaaS, and SaaS?

Differentiate IoT Level-4 and Level-5 with examples.

What are the architectural constraints of REST?

What is the role of a controller service in an IoT system?

Explain IoT Level-6 with a real-world example.

What is the role of a coordinator node in a WSN?

Compare Publish-Subscribe and Push-Pull communication models.

— End of Module 1 Study Notes —

Prepared from: Bahga & Madisetti, "Internet of Things: A Hands-On Approach" | PEMRT525 | JECC