PEMRT525 — IoT & Applications

Chapter 1 — Complete Study Notes

Introduction to Internet of Things

Textbook:

Internet of Things — A Hands-on Approach

Arshdeep Bahga & Vijay Madisetti

JECC  |  Batch MR 2K24  |  Semester S5

KTU Programme Elective — Level 5

Sections: 1.1 Introduction  ·  1.2 Physical Design  ·  1.3 Logical Design  ·  1.4 Enabling Technologies  ·  1.5 IoT Levels

# 1.1  Introduction to Internet of Things

Internet of Things (IoT) comprises things that have unique identities and are connected to the internet. While existing devices such as networked computers and 4G-enabled mobile phones already have unique identities and internet connectivity, the focus of IoT is on the configuration, control, and networking via the internet of devices or "things" that are traditionally not associated with the internet.

These include devices such as thermostats, utility meters, Bluetooth-connected headsets, irrigation pumps, and sensor or control circuits for electric car engines. Experts forecast that by the year 2020 there will be a total of 50 billion devices/things connected to the internet.

The scope of IoT is not limited to just connected devices. Applications on IoT networks extract and create information from lower-level data by filtering, processing, categorizing, condensing, and contextualizing the data. The information obtained is then organized and structured to infer knowledge about the system and its user, its environment, and its operations — allowing a smarter performance.

## 1.1.1  Definition of IoT

"A dynamic global network infrastructure with self-configuring capability based on standard and interoperable communication protocols where physical and virtual things have identities, physical attributes, and virtual personalities and use intelligent interfaces, and are seamlessly integrated into the information network."

## 1.1.2  Characteristics of IoT

- Dynamic and Self-Adapting: IoT devices and systems may have the capability to dynamically adapt with changing contexts and take actions based on their operating conditions. Example: Surveillance cameras can adapt their modes based on whether it is day or night.

- Self-Configuring: IoT devices may have self-configuring capability, allowing a large number of devices to work together to provide certain functionality.

- Interoperable Communication Protocols: IoT devices may support a number of interoperable communication protocols and can communicate with other devices and also with the infrastructure.

- Unique Identity: Each IoT device has a unique identity and a unique identifier (IP address, URI). IoT systems may have intelligent interfaces which adapt based on the context, allow communication with users and the environment.

- Integrated into Information Network: IoT devices are usually integrated into the information network that allows them to communicate and exchange data with other devices and systems.

# 1.2  Physical Design of IoT

Physical design of an IoT system refers to the individual node devices and the protocols used for communicating between them. Physical design includes the hardware (Things) and the communication protocols across all network layers.

## 1.2.1  Things of IoT

The "Things" in IoT usually refers to IoT devices which have unique identities and can perform remote sensing, actuating, and monitoring capabilities. IoT devices can exchange data with other connected devices and applications (directly or indirectly), or collect data from other devices and process the data locally, or send the data to centralized servers or cloud-based application back-ends for processing.

An IoT device may consist of several interfaces for connections to other devices, both wired and wireless. These include:

- Interfaces for sensors (onboard or attached)

- Interfaces for internet connectivity

- Memory and storage interfaces

- Audio/video interfaces

IoT devices can collect various types of data from onboard or attached sensors, such as temperature, humidity, and light intensity. IoT devices can also be of varied types — for instance, wearable sensors, smart watches, LED lights, automobiles, and industrial machines. Almost all IoT devices generate data in some form which, when processed by data analytics systems, leads to useful information to guide further actions locally or remotely.

## 1.2.2  IoT Protocols

IoT protocols span four network layers. The protocol stack for IoT systems is organized as follows:

### A. Link Layer Protocols

Link layer protocols determine how data is physically sent over the network's physical layer or medium (copper wire, electrical cable, or radio wave). The scope of the link layer is the local network connection to which the host is attached. Hosts on the same link exchange data packets over the link layer using the link layer protocol.

| Protocol | Standard | Data Rate | Description |
| --- | --- | --- | --- |
| Ethernet | IEEE 802.3 | 10 Mbps – 40 Gbps | Wired Ethernet standards; coaxial cable, twisted pair, or optical fiber as shared medium. Standards provide data rates from 10 Mbps to 40 Gbps and higher. |
| Wi-Fi | IEEE 802.11 (a/b/g/n/ac) | Up to 1+ Gbps | Wireless LAN. 802.11a operates at 5 GHz; 802.11b and 802.11g at 2.4 GHz; 802.11ac at 5 GHz with highest throughput. |
| WiMAX | IEEE 802.16 | 1.5 Mbps – 1 Gbps | Wireless broadband standard. Recent update provides 100 Mbps for mobile stations and up to 1 Gbps for fixed stations. |
| LR-WPAN (ZigBee) | IEEE 802.15.4 | 40 – 250 kbps | Low-rate wireless personal area network. Basis for ZigBee specification. Low cost, low speed for power-constrained devices. |
| 2G/3G/4G Mobile | GSM / UMTS / LTE | Varies | 2G: GSM, CDMA. 3G: UMTS, CDMA2000. 4G: LTE. Cellular connectivity for IoT devices in the field. |

### B. Network / Internet Layer Protocols

The network layer is responsible for sending IP datagrams from the source network to the destination network. This layer performs host addressing and packet routing. The datagrams contain a source and destination address used to route them from source to destination across multiple networks.

- IPv4 (Internet Protocol version 4): Most deployed internet protocol used to identify devices on a network. Uses a 32-bit address scheme that allows 2³² (approximately 4.3 billion) addresses. As more devices got connected to the internet, IPv4 was succeeded by IPv6.

- IPv6 (Internet Protocol version 6): Newest version of the internet protocol and successor to IPv4. Uses a 128-bit address scheme that allows 2¹²⁸ = 3.4 × 10³⁸ addresses — sufficient to assign unique addresses to every IoT device on earth.

- 6LoWPAN (IPv6 over Low-Power Wireless Personal Area Networks): Brings the IP protocol to low-power devices with limited processing capability. Operates in the 2.4 GHz frequency range and provides data transfer rates up to 250 kbps. Allows IoT devices with very limited resources to be connected to the internet using IPv6.

### C. Transport Layer Protocols

The transport layer provides end-to-end message transfer capability independent of the underlying network. The message transfer can be connection-oriented (with handshake) or connectionless (without handshake). Provides functions such as error control, segmentation, flow control, and congestion control.

- TCP (Transmission Control Protocol): Most widely used transport layer protocol. Used by web browsers with HTTP/HTTPS, email programs (SMTP), and file transfer protocol (FTP). TCP is connection-oriented and stateful. While IP deals with sending packets, TCP ensures reliable transmission of packets in order. Provides error detection so that duplicate packets can be discarded and lost packets are retransmitted. Flow control ensures the sender does not send faster than the receiver can process.

- UDP (User Datagram Protocol): Unlike TCP, UDP is a connectionless protocol and does not require an initial connection setup. Useful for time-sensitive applications that have small data units and do not want the overhead of connection setup. UDP is transaction-oriented and stateless. Does not provide guaranteed delivery, ordering of messages, or duplicate elimination.

### D. Application Layer Protocols

Application layer protocols define how applications interface with the lower layer protocols to send data over the network. Data is encoded by the application layer protocol and encapsulated in the transport layer protocol.

| Protocol | Full Name | Transport | Model | Description |
| --- | --- | --- | --- | --- |
| HTTP | HyperText Transfer Protocol | TCP | Request-Response | Foundation of the World Wide Web. Commands: GET, PUT, POST, DELETE, HEAD, TRACE, OPTIONS. Stateless — each HTTP request is independent. Client can be a browser, IoT device, or mobile application. |
| CoAP | Constrained Application Protocol | UDP | Request-Response | Application layer protocol for M2M communication in constrained environments with limited devices and networks. Like HTTP, CoAP uses a request-response model but runs over UDP. Supports GET, PUT, DELETE. Designed to interface easily with HTTP. |
| WebSocket | WebSocket Protocol | TCP | Exclusive Pair | Allows full-duplex communication over a single socket connection. Based on TCP. Streams of messages sent back and forth between client and server while keeping the TCP connection open. Client can be a browser, mobile app, or IoT device. Reduces network traffic and latency. |
| MQTT | Message Queue Telemetry Transport | TCP | Publish-Subscribe | Lightweight protocol based on publish-subscribe model. IoT devices (clients) connect to MQTT broker and publish messages to topics. Broker forwards messages to clients subscribed to those topics. Well-suited for constrained environments with limited bandwidth. |
| XMPP | Extensible Messaging and Presence Protocol | TCP | Publish-Subscribe | Protocol for real-time communication and streaming XML data between network entities. Powers a wide range of applications: messaging, presence, data syndication, gaming, multiparty chat, voice calls. Supports client-to-server and server-to-client communication. |
| DDS | Data Distribution Service | UDP/TCP | Publish-Subscribe | Data-centric middleware standard for device-to-device (M2M) communication. Publisher creates topics; subscribers subscribe to topics. Provides Quality of Service (QoS) control and configurable reliability. Designed for high-performance real-time applications. |
| AMQP | Advanced Message Queuing Protocol | TCP | Publish-Subscribe | Open application layer protocol for business messaging. Supports point-to-point and publish-subscribe routing and queuing. AMQP broker receives messages from publishers (devices/applications) and routes them to consumers via exchanges and queues. |

# 1.3  Logical Design of IoT

Logical design of an IoT system refers to an abstract representation of the entities and processes without going into the low-level specifics of the implementation. Logical design covers the functional blocks, communication models, and communication APIs of an IoT system.

## 1.3.1  IoT Functional Blocks

An IoT system comprises a number of functional blocks that provide the system with capabilities for identification, sensing, actuation, communication, and management. These functional blocks are:

| Functional Block | Function | Detailed Description |
| --- | --- | --- |
| Device | Sensing, Actuation, Monitoring, Control | An IoT system comprises devices that provide sensing, actuation, monitoring, and control functions. Devices collect data from onboard or attached sensors and can control actuators connected to them. Devices can perform processing locally or offload to cloud. |
| Communication | Handles all communication | The communication block handles the communication for the IoT system. This includes wired and wireless connectivity between devices and between devices and cloud/servers. Manages all protocol-level communication. |
| Services | Device monitoring, control, data publishing, discovery | An IoT system uses various types of IoT services such as services for device monitoring, device control services, data publishing services, and services for device discovery. Services provide the middleware layer connecting devices to applications. |
| Management | Governs the IoT system | Management functional blocks provide various functions to govern the IoT system. This includes configuration management, device provisioning, fault management, performance monitoring, and firmware/software updates. |
| Security | Authentication, Authorization, Integrity, Data security | The security functional block secures the IoT system by providing functions such as authentication, authorization, message and content integrity checking, and data security. Ensures only authorized entities can access and control IoT devices. |
| Application | User interface for control and monitoring | IoT applications provide an interface that the user can use to control and monitor various aspects of the IoT system. Applications also allow users to view the system status and view or analyze the processed data. Can be web or mobile applications. |

## 1.3.2  IoT Communication Models

IoT systems use various communication models. Bahga & Madisetti define four primary communication models:

### Model 1: Request-Response

Request-Response is a communication model in which the client sends a request to the server and the server responds to the request. When the server receives a request, it decides how to respond, fetches the resource, and sends the response to the client.

- Communication type: Stateless — each request-response pair is independent of others

- Client-Server relationship: Tightly coupled — client must wait for server response

- Direction: Half-duplex — client initiates, server responds

- Protocol examples: HTTP, CoAP

- IoT use cases: REST-based cloud dashboards, CoAP for constrained devices, web-based IoT control panels

### Model 2: Publish-Subscribe

Publish-Subscribe is a communication model that involves Publishers, Brokers, and Consumers. Publishers are the source of data. Publishers send data to topics managed by the broker. Publishers are not aware of the consumers. Consumers subscribe to topics managed by the broker. When the broker receives data for a topic from a publisher, it sends the data to all subscribed consumers.

- Communication type: Asynchronous — consumers receive data without polling

- Coupling: Loosely coupled — publishers and consumers are decoupled via the broker

- Direction: One-to-many (one publisher, many subscribers)

- Protocol examples: MQTT, AMQP, DDS

- IoT use cases: Sensor data streaming, real-time monitoring platforms, MQTT broker-based systems (AWS IoT Core, ThingSpeak)

### Model 3: Push-Pull

Push-Pull is a communication model in which the data producers push the data to queues and the consumers pull the data from the queues. Producers do not need to be aware of consumers. Queues help in decoupling the messaging between Producers and Consumers. Queues also act as a buffer which helps in situations when there is a mismatch between the rate at which producers push data and the rate at which consumers pull data.

- Communication type: Asynchronous with buffering

- Coupling: Loosely coupled via queue

- Buffer: Queue absorbs rate mismatch between producer and consumer

- Protocol examples: ZeroMQ, Apache Kafka, Amazon SQS

- IoT use cases: Data pipeline architectures, log aggregation, task queues, batch processing of sensor data

### Model 4: Exclusive Pair

Exclusive Pair is a bidirectional, fully duplex communication model that uses a persistent connection between the client and the server. Once the connection is set up, it remains open until the client sends a request to close the connection. Client and server can send messages to each other after connection setup.

- Communication type: Stateful — server is aware of all open connections

- Direction: Full-duplex — both client and server can initiate messages independently

- Connection: Persistent TCP connection maintained for the duration of the session

- Protocol examples: WebSocket

- IoT use cases: Real-time sensor data feeds to dashboards, live device control systems, Level 3 and Level 6 IoT deployments

| Model | Stateful / Stateless | Duplex | Coupling | Protocol |
| --- | --- | --- | --- | --- |
| Request-Response | Stateless | Half-duplex | Tight | HTTP, CoAP |
| Publish-Subscribe | Stateless | Async (one-to-many) | Loose (via broker) | MQTT, AMQP, DDS |
| Push-Pull | Stateless | Async (buffered) | Loose (via queue) | ZeroMQ, Kafka |
| Exclusive Pair | Stateful | Full-duplex | Tight (persistent) | WebSocket |

## 1.3.3  IoT Communication APIs

IoT systems use two primary types of communication APIs:

### A. REST-Based Communication API

Representational State Transfer (REST) is a set of architectural principles by which web services and web APIs are designed. REST focuses on system resources and how resource states are addressed and transferred. REST APIs follow the request-response communication model.

REST Architectural Constraints:

- Client-Server: Separation of concerns — the client should not be concerned with data storage (concern of the server) and the server should not be concerned with the user interface (concern of the client). This separation allows client and server to be independently developed and deployed.

- Stateless: Each request from client to server must contain all the information necessary to understand the request and cannot take advantage of any stored context on the server. Session state is kept entirely on the client.

- Cacheable: Data within the response to a request must be implicitly or explicitly labeled as cacheable or non-cacheable. If a response is cacheable, a client cache is given the right to reuse that response data for later equivalent requests. This improves efficiency and scalability.

- Layered System: Each component cannot see beyond the immediate layer with which it is interacting. For example, a client cannot tell whether it is connected directly to the end server or to an intermediary. System scalability can be improved by allowing intermediaries to respond to requests instead of the end server.

- Uniform Interface: The method of communication between client and server must be uniform. Resources are identified in requests and are separate from the representation of the resource returned to the client. When a client holds a representation of a resource, it has all the information required to update or delete the resource.

- Code on Demand (optional): Services can provide executable code or scripts for clients to execute in their own context.

### B. WebSocket-Based Communication API

WebSocket API allows bidirectional, full-duplex communication between client and server. Unlike request-response APIs, WebSocket allows full-duplex communication and does not require a new connection to be set up for each message. WebSocket communication begins with a connection setup request sent by the client to the server via HTTP (upgrade request). If the server supports the WebSocket protocol, it responds with the WebSocket handshake response. After connection setup, the client and server can send data or messages to each other in full-duplex mode.

Advantages of WebSocket over REST:

- Reduces network traffic and latency — no overhead for connection setup and termination for each message

- Enables real-time two-way communication — server can push data to client without client polling

- Single TCP connection maintained for the session — more efficient use of network resources

| Property | REST | WebSocket |
| --- | --- | --- |
| State | Stateless — each request is independent | Stateful — server maintains state of all connections |
| Direction | Unidirectional — client initiates requests; server responds | Bidirectional — client and server can both initiate messages |
| Duplex | Half-duplex — request then wait for response | Full-duplex — simultaneous two-way communication |
| TCP Connection | New TCP connection per HTTP request (high overhead) | Single persistent TCP connection for entire session |
| Header Overhead | High — each HTTP request carries full headers | Minimal — only initial handshake has overhead; subsequent messages have very small frame info |
| Real-Time Suitability | Not suitable — client must poll for updates | Ideal — server pushes data immediately when available |
| Scalability | Easy horizontal scaling — stateless, load balancer friendly | Vertical scaling easier — stateful, maintaining connections complicates horizontal scaling |
| IoT Level | Suitable for Level 1 and Level 2 deployments | Required for Level 3 and Level 6 real-time deployments |

# 1.4  IoT Enabling Technologies

IoT is enabled by several technologies including: Wireless Sensor Networks (WSN), Cloud Computing, Big Data Analytics, Communication Protocols, Embedded Systems, Security Protocols and Architectures, Web Services, Mobile Internet, and Semantic Search Engines.

## 1.4.1  Wireless Sensor Networks (WSN)

A Wireless Sensor Network (WSN) comprises distributed devices with sensors which are used to monitor the environmental and physical conditions. A WSN consists of a number of end nodes and routers and a coordinator.

### WSN Architecture Components

- End Nodes: Have several sensors attached to them. End nodes can also act as routers. They perform the sensing function — collecting data on temperature, humidity, pressure, light, motion, or other parameters.

- Routers: Responsible for routing the data packets from end nodes to the coordinator. They relay data from nodes that are out of direct range of the coordinator.

- Coordinator Node: Collects the data from all the nodes. The coordinator also acts as a Gateway that connects the WSN to the internet, allowing the sensor data to reach cloud or server applications.

### IoT Applications using WSN

- Weather monitoring: Nodes collect temperature, humidity, and other data which is aggregated and analyzed

- Indoor air quality monitoring: WSN collects data on indoor air quality and concentrations of various gases

- Soil moisture monitoring: WSN monitors soil moisture at various locations for smart irrigation decisions

- Surveillance: WSN used for collecting surveillance data (motion detection data)

- Smart Grid: Wireless sensor networks used for monitoring the electrical grid at various points

- Structural health monitoring: WSN monitors the health of structures (bridges, buildings) by recording vibration data from sensor nodes deployed at various points in the structure

## 1.4.2  Cloud Computing

Cloud Computing is a transformative computing paradigm that involves delivering applications and services over the internet. Cloud Computing involves provisioning of computing, networking, and storage resources on demand and providing these resources as metered services to users, in a "pay as you go" model.

### Key Characteristics of Cloud Computing

- On-demand self-service: Cloud computing resources can be provisioned on demand by the user without requiring interactions with the Cloud Service Provider. The process of provisioning resources is automated.

- Broad network access: Cloud computing resources can be accessed over the network using standard access mechanisms that provide platform-independent access through the use of heterogeneous client platforms such as workstations, laptops, tablets, and smartphones.

- Resource pooling / Multi-tenancy: The computing and storage resources provided by Cloud Service Providers are pooled to serve multiple users using multi-tenancy. Multi-tenancy allows multiple users to be served by the same physical hardware.

### Cloud Service Models

| Service Model | Full Name | User Can | Provider Manages | Examples |
| --- | --- | --- | --- | --- |
| IaaS | Infrastructure as a Service | Provision computing and storage resources as virtual machines and virtual storage. Deploy OS and applications of choice. Start, stop, configure, and manage virtual machine instances. | The underlying physical infrastructure — servers, storage, networking hardware. | AWS EC2, Microsoft Azure VMs, Google Compute Engine |
| PaaS | Platform as a Service | Develop and deploy applications in the cloud using deployment tools, APIs, software libraries, and services provided by the provider. | The underlying cloud infrastructure including servers, network, operating systems, and storage. | Google App Engine, AWS Elastic Beanstalk, Microsoft Azure App Service |
| SaaS | Software as a Service | Use complete software applications via a thin client interface (browser). Access applications from various smartphones running different operating systems. | Underlying cloud infrastructure including servers, network, storage, application software. User is unaware of underlying architecture. | ThingSpeak, Gmail, Salesforce, Google Docs |

## 1.4.3  Big Data Analytics

Big data is defined as collections of data sets whose volume, velocity (in terms of temporal variations), or variety is so large that it is difficult to store, manage, process, and analyze the data using traditional database and data processing tools.

### The 3 Vs of Big Data

- Volume: Though there is no fixed threshold, big data refers to massive-scale data that is difficult to store, manage, and process using traditional databases and processing architectures. The volume of data generated by modern IT, industrial, and healthcare systems is growing exponentially, driven by the lowering cost of data storage and processing architectures and the need to extract valuable insights.

- Velocity: Velocity refers to how fast data is generated and how frequently it varies. Modern IT, industrial, and other systems are generating data at increasing speeds. Real-time processing of high-velocity IoT sensor data requires distributed computing frameworks.

- Variety: Variety refers to the forms of the data. Big data comes in different forms: structured or unstructured data, including text data, audio, video, and sensor data. IoT systems generate data of many types simultaneously.

### IoT Big Data Examples

- Sensor data generated by IoT systems such as weather monitoring stations

- Machine sensor data collected from sensors embedded in industrial and energy systems for monitoring their health and predicting failures

- Health and fitness data generated by IoT devices such as wearable fitness bands

- Data generated by IoT systems for location tracking of vehicles (GPS fleet data)

- Data generated by retail inventory monitoring systems

### Big Data Analytics Process

Big Data Analytics involves several sequential steps:

- Data Cleaning: Removing or correcting incorrect, corrupted, duplicate, or incomplete data from the dataset

- Data Munging (Wrangling): Transforming and mapping raw data from one format into another format to make it suitable for analysis

- Data Processing: Applying analytics algorithms (statistical analysis, machine learning, etc.) to the cleaned data

- Visualization: Presenting the results in graphical or visual formats that are easy for users to understand and act upon

## 1.4.4  Communication Protocols

Communication protocols form the backbone of IoT systems and enable network connectivity and coupling to applications. Communication protocols allow devices to exchange data over the network. These protocols define the data exchange formats and data encoding schemes for devices and the routing of packets from source to destination.

Other functions provided by communication protocols include:

- Sequence control: Ensuring data packets are processed in the correct order

- Flow control: Preventing a fast sender from overwhelming a slow receiver

- Transmission of lost packet retransmission: Detecting and recovering from lost data packets

Note: Communication protocols are covered in detail in Section 1.2.2 — covering Link Layer (Ethernet, Wi-Fi, WiMAX, LR-WPAN, 2G/3G/4G), Network Layer (IPv4, IPv6, 6LoWPAN), Transport Layer (TCP, UDP), and Application Layer (HTTP, CoAP, WebSocket, MQTT, XMPP, DDS, AMQP).

## 1.4.5  Embedded Systems

An Embedded System is a computer system that has computer hardware and software embedded to perform specific tasks. In contrast to general-purpose computers or personal computers which can perform various types of tasks, embedded systems are designed to perform a specific set of tasks.

### Components of Embedded Systems

- Microprocessors and Microcontrollers: The processing core of the embedded system. Microcontrollers integrate CPU, memory, and I/O on a single chip.

- Memory: RAM (volatile, for runtime data), ROM (non-volatile, for firmware), Cache (fast access buffer), Flash Memory (non-volatile, for data logging)

- Networking Units: Ethernet adapter, Wi-Fi adapter, Bluetooth module, LoRa module — for connectivity

- Input/Output Units: Display, keyboard, touchscreen, LEDs — for human interaction

- Storage: Flash Memory cards (SD, eMMC) for data storage

- Specialist Processors: Digital Signal Processor (DSP) for signal processing, Graphics Processor (GPU) for image processing, Application-specific integrated circuits (ASIC)

### Role in IoT Systems

Embedded systems are the "Things" in IoT. They:

- Run the firmware/software that reads sensor data and controls actuators

- Implement communication protocols (MQTT, HTTP, CoAP) to send/receive data

- Perform local data processing and filtering before sending to the cloud

- Manage power consumption for battery-operated IoT devices

### Common IoT Embedded Platforms

| Platform | Type | Connectivity | PEMRT525 Use |
| --- | --- | --- | --- |
| Arduino Uno / Mega | Microcontroller (AVR ATmega) | Via shields (Ethernet, Wi-Fi, LoRa) | E1 (LoRaWAN), E2 (ZigBee) |
| Raspberry Pi 3/4 | Single-board computer (ARM Cortex-A) | Built-in Wi-Fi, Ethernet, Bluetooth | E3 (GPIO), E4 (Sensor), E5 (Serial), E6 (SPI), E7 (I2C), Projects P1, P7 |
| ESP32 | Microcontroller (Xtensa dual-core) | Built-in Wi-Fi + Bluetooth | Projects P2, P3, P4, P5, P6 |
| ESP8266 | Microcontroller (Xtensa single-core) | Built-in Wi-Fi | IoT prototyping, lightweight nodes |

# 1.5  IoT Levels & Deployment Templates

Bahga & Madisetti define various levels of IoT systems with increasing complexity. Each level is characterized by the number of nodes, where data is stored, where analysis is performed, and where the application runs.

## IoT System Components (Common to All Levels)

An IoT system at any level comprises the following fundamental components:

| Component | Description |
| --- | --- |
| Device | An IoT device allows identification, remote sensing, actuating, and remote monitoring capabilities. It is the physical "thing" in IoT. |
| Resources | Resources are software components on the device for accessing and storing information, for controlling actuators connected to the device, and for enabling network access for the device. |
| Controller Service | A Controller Service is a native service that runs on the device and interacts with the web services. The controller service sends data from the device to the web service and receives commands from the application for controlling the device. |
| Database | Database stores the data generated by IoT devices. The database can be local (stored on the device or a local server) or in the cloud. |
| Web Service | Web Service acts as a link between the device, application, database, and analysis components. Web Services can be implemented using HTTP REST principles or using WebSocket protocol. |
| Analysis Component | The analysis component is responsible for analyzing IoT data and generating results in a form that is easy for the user to understand. Analysis can be performed locally or in the cloud. Results are stored in the local or cloud database. |
| Application | IoT applications provide an interface that the user can use to control and monitor various aspects of the IoT system. Applications also allow users to view the system status and view the processed data. |

## REST vs WebSocket Web Services

IoT systems use either REST or WebSocket web services:

| Property | REST Service | WebSocket Service |
| --- | --- | --- |
| State | Stateless — each request is independent | Stateful — server maintains state of all connections |
| Direction | Unidirectional — request from client, response from server | Bidirectional — both client and server can initiate messages |
| Duplex | Request-Response (half-duplex) | Full-duplex — simultaneous two-way communication |
| TCP Connections | Each HTTP request involves a new TCP connection | Single persistent TCP connection for the entire session |
| Header Overhead | High — each request carries full HTTP headers | Minimal — only initial handshake has overhead |
| Scalability | Easier horizontal scaling (stateless) | Vertical scaling is easier (stateful — connections tracked) |

## 1.5.1  IoT Level 1

A Level 1 IoT system has a single node/device that performs sensing and/or actuation, stores data locally, performs analysis locally, and hosts the application locally. Level 1 IoT systems are suitable for low-cost, low-complexity solutions where the data volume is not big and the analysis requirements are not computationally intensive.

### Architecture

- Node: Single device with sensors/actuators

- Storage: Local database on the device or local server

- Analysis: Done locally on the device (not computationally intensive)

- Application: Deployed locally on the device (accessible remotely via internet)

- Web Service: REST (local)

### Example: Home Automation System

This system consists of a single node that controls lights and appliances in a home remotely. The device interfaces with lights and appliances using electronic relay switches. The status information of each light or appliance is maintained in a local database. REST services deployed locally allow retrieving and updating the state of each light or appliance. The controller service continuously monitors the state and triggers relay switches accordingly. The application, deployed locally, has a user interface for controlling lights or appliances. Since the device is connected to the internet, the application can be accessed remotely as well.

Note: JECC Projects at Level 1: P1 (Smart Digital Notice Board), P2 (Smart AC Controller), P7 (Smart Print Queue Kiosk)

## 1.5.2  IoT Level 2

A Level 2 IoT system has a single node that performs sensing and/or actuation and local analysis. Data is stored in the cloud and the application is usually cloud-based. Suitable for solutions where the data volume is big (hence cloud storage), but the primary analysis requirement is not computationally intensive and can be done locally.

### Architecture

- Node: Single device with sensors/actuators

- Storage: Cloud database

- Analysis: Done locally on the device

- Application: Cloud-based

- Web Service: REST (to cloud)

### Example: Smart Irrigation System

The system consists of a single node that monitors the soil moisture level and controls an irrigation system. The device collects soil moisture data from sensors. The controller service continuously monitors the moisture level. If the moisture level drops below a threshold, the irrigation system is turned ON. REST Web Services are used for storing and retrieving data, which is stored in the cloud database. A cloud-based application visualizes the moisture level over a period of time, which can help in making decisions about irrigation schedules.

Note: JECC Projects at Level 2: P3 (RFID EV Charging Station — data to LAN server), P5 (Staff Location Tracker — heartbeat to LAN server)

## 1.5.3  IoT Level 3

A Level 3 IoT system has a single node. Data is stored and analyzed in the cloud. The application is cloud-based. Level 3 systems are suitable for solutions where the data volume is big and analysis requirements are computationally intensive. Uses WebSocket for real-time data transmission.

### Architecture

- Node: Single device with sensors/actuators

- Storage: Cloud database

- Analysis: Cloud (computationally intensive)

- Application: Cloud-based

- Web Service: WebSocket (real-time data streaming to cloud)

### Example: Package Vibration Tracking System

The system consists of a single node that monitors vibration levels for a package being shipped. The device uses an accelerometer and gyroscope for monitoring vibration levels. The controller service sends sensor data to the cloud in real-time using a WebSocket service. The data is stored in the cloud and visualized using a cloud-based application. The analysis component in the cloud can trigger an alert when vibration levels exceed a threshold.

Advantage of WebSocket in this example: sensor data can be sent in real-time to the cloud. Cloud-based applications can subscribe to the sensor data feeds to view real-time data.

## 1.5.4  IoT Level 4

A Level 4 IoT system has multiple nodes that perform local analysis. Data is stored in the cloud and the application is cloud-based. Level 4 contains local and cloud-based observer nodes which can subscribe to and receive information collected in the cloud from IoT devices.

### Observer Nodes

Observer nodes can process information and use it for various applications. However, observer nodes do NOT perform any control function — they only observe and process data. Level 4 IoT systems are suitable for solutions where multiple nodes are required, the data is big, and analysis requirements are computationally intensive.

### Architecture

- Nodes: Multiple independent sensing/actuating nodes

- Analysis: Local on each node + Cloud (observer analysis)

- Storage: Cloud database

- Application: Cloud-based

- Observer Nodes: Cloud-based nodes that subscribe to IoT data and process it

### Example: Noise Monitoring System

The system consists of multiple nodes placed in different locations for monitoring noise levels in an area. Each node is equipped with a sound sensor. Nodes are independent of each other — each runs its own controller service that sends data to the cloud. The data is stored in a cloud database. The analysis of the data collected from multiple nodes is done in the cloud. Observer nodes can subscribe to the aggregated noise data for further analysis (e.g., noise map generation).

## 1.5.5  IoT Level 5

A Level 5 IoT system has multiple end nodes and one coordinator node. End nodes perform sensing and/or actuation. The coordinator node collects data from the end nodes and sends it to the cloud. Data is stored and analyzed in the cloud. The application is cloud-based.

### Architecture

- End Nodes: Multiple sensor/actuator nodes with NO direct internet connectivity

- Coordinator Node: Collects data from end nodes; acts as Gateway providing internet connectivity

- Storage: Cloud database

- Analysis: Cloud

- Application: Cloud-based

### Example: Forest Fire Detection System

The system consists of multiple nodes placed in different locations for monitoring temperature, humidity, and CO2 levels in a forest. The end nodes are equipped with various sensors such as temperature, humidity, and CO2. The coordinator node collects the data from the end nodes and acts as a Gateway that provides internet connectivity. The controller service on the coordinator device sends the collected data to the cloud. The data is stored in the cloud database. The analysis in the cloud aggregates the data and makes predictions about potential forest fires.

Note: JECC Project P4 (LoRa Campus Weather Station) operates at Level 5: Multiple ESP32 sensor nodes → LoRa Gateway → Flask server. This uses LoRaWAN as the link layer for end nodes.

## 1.5.6  IoT Level 6

A Level 6 IoT system has multiple independent end nodes that perform sensing and/or actuation and send data directly to the cloud. Data is stored in the cloud and analyzed in the cloud. The application is cloud-based. A centralized controller is aware of the status of all the end nodes and sends control commands to the nodes.

### Architecture

- End Nodes: Multiple independent nodes, each with direct internet connectivity

- Centralized Controller: Aware of all nodes; sends control commands to individual nodes

- Storage: Cloud database

- Analysis: Cloud

- Application: Cloud-based

- Web Service: WebSocket (real-time data from all nodes)

### Example: Campus-Wide Weather Monitoring Network

The system consists of multiple nodes placed in different locations, each measuring temperature, humidity, and pressure. The end nodes are equipped with various sensors. Each node independently sends data to the cloud in real-time using WebSocket services. The data is stored in a cloud database. Analysis is done in the cloud to aggregate the data from all nodes and make predictions. A cloud-based application visualizes the data from all nodes simultaneously. The centralized controller monitors all nodes and can send configuration commands to individual nodes.

## IoT Levels — Complete Comparison

| Level | Nodes | Storage | Analysis | Application | Web Service | Example |
| --- | --- | --- | --- | --- | --- | --- |
| Level 1 | Single | Local | Local | Local | REST (local) | Home Automation, AC Controller |
| Level 2 | Single | Cloud | Local (on device) | Cloud | REST (to cloud) | Smart Irrigation, EV Charger |
| Level 3 | Single | Cloud | Cloud | Cloud | WebSocket | Package Vibration Tracking |
| Level 4 | Multiple | Cloud | Local + Cloud (observers) | Cloud | REST / WebSocket | Noise Monitoring System |
| Level 5 | Multi + Coordinator | Cloud | Cloud | Cloud | Coordinator Gateway | Forest Fire Detection, LoRa Weather |
| Level 6 | Multi (Independent) | Cloud | Cloud | Cloud | WebSocket + Controller | Weather Monitoring Network |

# Quick Revision — Chapter 1 Summary

## 1. Physical Design of IoT

- Things: Unique identity + sensing + actuation + monitoring + communication interfaces

- Protocols — Link Layer: 802.3 Ethernet | 802.11 Wi-Fi | 802.16 WiMAX | 802.15.4 LR-WPAN (ZigBee) | 2G/3G/4G

- Protocols — Network Layer: IPv4 (32-bit) | IPv6 (128-bit) | 6LoWPAN (IPv6 for low-power)

- Protocols — Transport Layer: TCP (reliable, connection-oriented) | UDP (lightweight, connectionless)

- Protocols — Application Layer: HTTP | CoAP | WebSocket | MQTT | XMPP | DDS | AMQP

## 2. Logical Design — Functional Blocks

- 6 blocks: Device | Communication | Services | Management | Security | Application

- Services block: device monitoring + device control + data publishing + device discovery

- Security block: authentication + authorization + message integrity + data security

## 3. Communication Models

- Request-Response (HTTP, CoAP): Stateless, half-duplex, tight coupling

- Publish-Subscribe (MQTT, AMQP, DDS): Stateless, async, loose coupling via broker

- Push-Pull (ZeroMQ, Kafka): Stateless, async, loose coupling via queue/buffer

- Exclusive Pair (WebSocket): Stateful, full-duplex, persistent TCP connection

## 4. Communication APIs

- REST: Stateless | Unidirectional | Half-duplex | High overhead | Easy horizontal scaling

- WebSocket: Stateful | Bidirectional | Full-duplex | Minimal overhead | Better for real-time

- REST constraints: Client-Server | Stateless | Cacheable | Layered | Uniform Interface | Code on Demand

## 5. Enabling Technologies

- WSN: End nodes → Routers → Coordinator (Gateway) → Internet

- Cloud Computing: IaaS (VMs) | PaaS (Dev platform) | SaaS (Applications)

- Big Data: Volume + Velocity + Variety | Cleaning → Munging → Processing → Visualization

- Communication Protocols: Backbone of IoT — data format, encoding, routing, flow control

- Embedded Systems: Microcontroller + Memory + Networking + I/O + Storage + Specialist processors

## 6. IoT Levels

- Level 1: 1 node, ALL LOCAL (storage + analysis + app). Example: Home Automation

- Level 2: 1 node, LOCAL analysis, CLOUD storage + app. Example: Smart Irrigation

- Level 3: 1 node, ALL CLOUD, WebSocket real-time. Example: Package Tracking

- Level 4: MULTIPLE nodes, LOCAL analysis, cloud storage + app, observer nodes. Example: Noise Monitor

- Level 5: MULTIPLE end nodes + COORDINATOR gateway, all cloud. Example: Forest Fire

- Level 6: MULTIPLE INDEPENDENT nodes + CENTRAL CONTROLLER, all cloud, WebSocket. Example: Weather Network

Reference: Bahga & Madisetti, "Internet of Things: A Hands-on Approach", Chapter 1  |  PEMRT525 JECC  |  Batch MR 2K24