# Internet of Things: A Hands-On Approach

Arshdeep Bahga & Vijay Madisetti — Chapter 1

— Page 1 —

EE - a
20 Introduction to Internet of Things
1.1. Introduction
Internet of Things (loT) comprises things that have unique identities and are connected to the
Internet. While many existing devices, such as networked computers or 4G-enabled mobile
phones, already have some form of unique identities and are also connected to the Internet,
the focus on IoT is in the configuration, control and networking via the Internet of devices or
"things" that are traditionally not associated with the Internet. These include devices such _
as thermostats, utility meters, a bluetooth-connected headset, irrigation pumps and sensors,
or control circuits for an electric car’s engine. Internet of Things is a new revolution in the
capabilities of the endpoints that are connected to the Internet, and is being driven by the
advancements in capabilities (in combination with lower costs) in sensor networks, mobile
devices, wireless communications, networking and cloud technologies. Experts forecast that
by the year 2020 there will be a total of 50 billion devices/things connected to the Internet.
‘Therefore, the major industry players are excited by the prospects of new markets for their
products. The products include hardware and software components for IoT endpoints, hubs,
or control centers of the loT universe.
Figure 1.1: Inferring information and knowledge from data é:

‘The scope of IoT is not limited to just connecting things (devices, appliances, machines)
to the Internet. IoT allows these things to communicate and exchange data (control &
information, that could include data associated with users) while executing meaningful
applications towards a common user or machine goal. Data itself does not have a meaning

Bahga & Madisetti, © 2015

— Page 2 —

| 1.1. Introduction 21
| S|
Figure 1.2: Applications of loT

until it is contextualized processed into useful information. Applications on IoT networks
extract and create information from lower level data by filtering, processing, categorizing,
condensing and contextualizing the data, ‘This information obtained is then organized and
structured to infer knowledge about the system and/or its users, its environment, and its
operations and progress towards its objectives, allowing a smarter performance, as shown in
Figure 1.1. For example, consider a series of raw sensor measurements ((72,45) ; (84, 56))
generated by a weather monitoring station, which by themselves do not have any meaning ot
Internet of Things - A Hands-On Approach

— Page 3 —

22 Introduction to Internet of Things
context. To give meaning to the data, a context is added, which in this example can be that
cach tuple in data represents the temperature and humidity measured every minute. With this
context added we know the meaning (or information) of the measured data tuples. Further
information is obtained by categorizing, condensing or processing this data. For example,
the average temperature and humidity readings for last five minutes is obtained by averaging
the last five data tuples. The next step is to organize the information and understand the
relationships between pieces of information to infer knowledge which can be put into action.
For example, an alert is raised if the average temperature in last five minutes exceeds 120F,
nd this alert may be conditioned on the user's geographical position as well.

‘The applications of Internet of Things span a wide range of domains including (but not
limited to) homes, cities, environment, energy systems, retail, logistics, industry, agriculture
and health as listed in Figure 1.2. For homes, loT has several applications such as smart
lighting that adapt the lighting to suit the ambient conditions, smart appliances that can
be remotely monitored and controlled, intrusion detection systems, smart smoke detectors,
etc. For cities, oT has applications such as smart parking systems that provide status
updates on available slots, smart lighting that helps in saving energy, smart roads that provide
information on driving conditions‘and structural health monitoring systems. For environment,
IoT has applications such as weather monitoring, air and noise pollution, forest fire detection
and river flood detection systems. For energy systems, IoT has applications such as including
smart grids, grid integration of renewable energy sources and prognostic health management
systems. For retail domain, IoT has applications such as inventory management, smart
payments and smart vending machines. For agriculture domain, IoT has applications such
as smart irrigation systems that help in saving water while enhancing productivity and
‘green house control systems. Industrial applications of IoT include machine diagnosis and
prognosis systems that help in predicting faults and determining the cause of faults and
indoor air quality systems, For health and lifestyle, oT has applications such as health and |
fitness monitoring systems and Wearable electronics. |

|

|

1.1.1. Definition & Characteristics of loT i
‘The Internet of Things (loT) has been defined as (1):
Definition: ‘A dynamic global network infrastructure with self-configuring capabilities j
based on standard and interoperable communication protocols where physical and vial
 imerfaces, and are seamlessly integrated into the information network, often communicate
| data associated with users and their environments, anuccueneatel se ic lpm}
ea ee ie eee
Bahga & Madisett, © 2015

— Page 4 —

i 1.1 Introduction 23
| Let us examine this definition of IoT further to put some of the termis into’ perspective.
‘« Dynamic & Self-Adapting: IoT devices and systems may have the capability to
dynamically adapt with the changing contexts and take actions based on their operating
conditions, user’s context, or sensed environment. For example, consider a surveillance
i é system comprising of a number of surveillance cameras. The surveillance cameras
‘can adapt their modes (to normal or infra-red night modes) based on whether it is
day or night. Cameras could switch from lower resolution to higher resolution modes
when any motion is detected and alert nearby cameras to do the same. In this example,
the surveillance system is adapting itself based on the context and changing (e.¢.,
dynamic) conditions.
‘e Self-Configuring: IoT devices may have self-configuring capability, allowing a large
F ‘number of devices to work together to provide certain functionality (such as weather
| monitoring). These devices have the ability configure themselves (in association with
the IoT infrastructure), setup the networking, and fetch latest software upgrades with
minimal manual or user intervention.
Interoperable Communication Protocols: IoT devices may support a number of
interoperable communication protocols and cant communicate with other devices and
‘also with the infrastructure. We describe some of the commonly used communication
protocols and models in later sections.
| ‘© Unique Identity: Each IoT device has a unique identity and a unique identifier (such
‘as an IP address or a URI). JoT systems may have intelligent interfaces which adapt
based on the context, allow communicating with users and the environmental contexts.
IoT device interfaces allow users to query the devices, monitor their status, and
control them remotely, in association with the control, configuration and management
© Integrated into Information Network: IoT devices are usually integrated into the
} information network that allows them to communicate and exchange data with other
| devices and systems. loT devices can be dynamically discovered in the network, by
} other devices and/or the network, and have the capability to describe themselyes (and
| their characteristics) to other devices or user applications. For example, a weather
monitoring node can describe its monitoring capabilities to another connected node _
| so that they can communicate and exchange data. Integration into the information
t network helps in making IoT systems “smarter” due to the collective intelligence of
the individual devices in collaboration with the infrastructure. Thus, the data from
| a large number of connected weather monitoring JoT nodes can be aggregated and
b analyzed to predict the weather,
re a
f Internet of Things - A Hands-On Approach

— Page 5 —

| 24 Introduction to Internet of Things
| 1.2 Physical Design of loT
| 1.2.1 Things in loT
‘The "Things" in IoT usually refers to loT devices which have unique identities and can
perform remote sensing, actuating and monitoring capabilities. IoT devices can exchange
data with other connected devices and applications (directly or indirectly), or collect data
from other devices and process the data either locally or send the data to centralized servers
or cloud-based application back-ends for processing the data, or perform some tasks locally
and other tasks within the IoT infrastructure, based on temporal and space constraints (i.¢.,
‘memory, processing capabilities, communication latencies and speeds, and deadlines).
| Figure 1.3 shows a block diagram of a typical IoT device. An JoT device may consist of
several interfaces for connections to other devices, both wired and wireless, These include (i)
V/O jnterfaces for sensors, (ii) interfaces for Internet connectivity, (iii) memory and storage
interfaces and (iv) audio/video interfaces. An IoT device can collect various types of data
from the on-board or attached sensors, such as temperature, humidity, light intensity. The
' sensed data can be communicated either to other devices or cloud-based servers/storage. 1oT
i devices can be connected to actuators that allow them to interact with other physical entities
t (including non-IoT devices and systems) in the vicinity of the device, For example, a relay
switch connected to an IoT device can tum an appliance on/off based on the commands sent
) to the IoT device over the Internet.
IoT devices can also be of varied types, for instance, wearable sensors, smart watches,
LED lights, automobiles and industrial machines, Almost all [oT devices generate data in
some form or the other which when processed by data analytics systems leads to useful
information to guide further actions locally or remotely. For instance, sensor data generated
by a soil moisture monitoring device in a garden, when processed can help in determining
the optimum watering schedules. Figure 1.4 shows different types of IoT devices.
1.2.2 loT Protocols
Link Layer
Link layer protocols determine how the data is physically sent over the network's physical
layer or medium (e.g., copper wire, coaxial cable, ot a radio wave). The scope of the link
layer is the local network connection to which host is attached. Hosts on the satne link
exchange data packets over the link layer using link layer protocols. Link layer determines
how the packets are coded and signaled by the hardware device over the medium to which
the host is attached (such as a coaxial cable), Let us now look at some link layer protocols
which are relevant in the context of IoT. j
‘© 802.3 - Ethernet : IEEE 802.3 is a collection of wired Ethernet standards for the link j
iia ba aca oi cae Reena ts a ales
Bahga & Madisett, © 2015

— Page 6 —

. Design of loT 25
Figure 1.3; Generic block diagram of an loT Device

layer. For example, 802.3 is the standard for 1OBASES Ethernet that uses coaxial

cable as a shared medium, 802.3.i is the standard for 1OBASE-T Ethernet over copper

twisted-pair Connections, 802.3,j is the standard for 1OBASE-F Ethernet over fiber

optic connections, 802,3ae is the standard for 10 Gbit/s Ethernet over fiber, and so on.

These standards provide data rates from 10 Mb/s to 40 Gb/s and higher. The shared

medium in Ethernet can be a coaxial cable, twisted-pair wire or an optical fiber. The

| shared medium (i.e., broadcast medium) carries the communication for all the devices

\ ‘on the network, thus data sent by one device can received by all devices subject to
propagation conditions and transceiver capabilities. The specifications of the 802.3

| standards are available on the IEEE 802.3 working group website [2].

} © 802.11 - WiFi : IEEE 802.11 is a collection of wireless local area network (WLAN)
communication standards, including extensive description of the link layer. For
example, 802.1 1a operates in the 5 GHz band, 802.11b and 802.11g operate in the
2.4 GHz band, 802.1 1n operates in the 2.4/5 GHz bands, 802.11ac operates inthe 5 ~
GHz band and 802.1 lad operates in the 60 GHz band. These standards provide data
rates from 1 Mb/s to upto 6.75 Gb/s. The specifications of the 802.11 standards are
available on the IEEE 802.11 working group website [3]

802.16 - WiMax : IEEE 802.16 is a collection of wireless broadband standards,

t including extensive descriptions for the link layer (also called WiMax). WiMax

Internet of Things - A Hands-On Approach

— Page 7 —

26 Introduction to Internet of Things
Figure 1.4: IoT Devices

standards provide data rates from 1.5 Mb/s to 1 Gb/s. ‘The recent update (802. 16m)

provides data rates of 100 Mbit/s for mobile stations and 1 Gbit/s for fixed stations.

‘The specifications of the 802.11 standards are readily available on the IEEE 802.16
working group website [4]

ach ac SL TN et =e

Bahga & Madisetti, © 2015

— Page 8 —

"12 Physical Design of loT 27.
| rrp. fi coar |
=
y |. marr L005 JL AMOP |
— -
hee :
: Lupe |
taken a
foxx hs ar
re
s-cvone azas- nica
: [aoa wi
i +9 Figure 1.5: loT Protocols
| 9) 802.15.4. LR-WPAN + IEEE 802,15.4 is a collection of standards for low-rate
|. swireless personal area networks (LR-WPANS), These standards form the basis of
| ‘specifications for high level communication protocols such as ZigBee. LR-WPAN,
| standards provide data rates from 40 Kb/s 250 Kb/s. These standards provide low-cost
Fi sod low-speed communion for pows:consunae! cere. The ppclicatat
|: the 802,15.4 standards are available on the IEEE 802-15 working group website [5]
- [{ 2G! 3G/ 4G - Mobile Communication : There are different generations of mobile.
iopapennionion roe lncieang eons wmetion (0 ee
".,. (DMA), third generation (3G - including UMTS and CDMA2000), generation
Be HG -ncoting LTE), 10 vibes bated on tna srl ak melee over
|: ellular networks, Data rates for these standards range from 9.6 Kb/s (for 26) to upto
4, 100 Mbs (for 4G) and are available from the 3GPP, websites,
ene ‘of Things - A-Hands-On Approach 3
:

— Page 9 —

28 Introduction to Internet of Things
Network/internet Layer

‘The network layers are responsible for sending of IP datagrams from the source network
to the destination network. ‘This layer performs the host addressing and packet routing.
‘The datagrams contain the source and destination addresses which are used to route them
from the source to destination across multiple networks. Host identification is done using
hierarchical IP addressing schemes such as IPv4 or IPv6.

IPv4 : Internet Protocol version 4 (IPv4) is the most deployed Internet protocol that is
used to identify the devices on a network using a hierarchical addressing scheme. IPv4
uses a 32-bit address scheme that allows total of 2°? or 4,294,967,296 addresses. As
more and more devices got connected to the Internet, these addresses got exhausted
in the year 2011. IPv4 has been succeeded by IPv6. The IP protocols establish
connections on packet networks, but do not guarantee delivery of packets. Guaranteed
delivery and data integrity are handled by the upper layer protocols (such as TCP).
IPy4 is formally described in RFC 791 [6]. :

* IPv6 : Internet Protocol version 6 (IPv6) is the newest version of Internet protocol
and successor to IPv4. IPv6 uses 128-bit address scheme that allows total of 2!8 or
3.4 x 10°8 addresses. IPv4 is formally described in RFC 2460 [7].

* GLOWPAN : 6LOWPAN (IPv6 over Low power Wireless Personal Area Networks)
brings IP protocol to the low-power devices which have limited processing capability.
6LoWPAN operates in the 2.4 GHz frequency range and provides data transfer rates
of 250 Kb/s. 6LOWPAN works with the 802.15.4 link layer protocol and defines
compression mechanisms for IPv6 datagrams over IEEE 802.15.4-based networks
(8).

‘Transport Layer

‘The transport layer protocols provide end-to-end message transfer capability independent
of the underlying network. The message transfer capability can be set up on connections,
either using handshakes (as in TCP) or without handshakes/acknowledgements (as in UDP).
The transport layer provides functions such as error control, segmentation, flow control and
congestion control.

© TCP: Transmission Control Protocol (TCP) is the most widely used transport layer
protocol, that is used by web browsers (along with HTTP, HTTPS application layer
protocols), email programs (SMTP application layer protocol) and file transfer (FTP).
TCP is a connection oriented and stateful protocol. While IP protocol deals with
sending packets, TCP ensures reliable transmission of packets in-order. TCP also
provides error detection capability so that duplicate packets can be discarded and
lost packets are retransmitted. The flow control capability of TCP enstires that

Bahga & Madisetti, © 2015

— Page 10 —

r
1.2. Physical Design of loT 29
rate at which the sender sends the data is not too high for the receiver to process.
‘The congestion control capability of TCP helps in avoiding network congestion and
congestion collapse which can lead to degradation of network performance. TCP is
described in RFC 793 [9].
© UDP : Unlike TCP, which requires carrying out an initial setup procedure, UDP is a
connectionless protocol. UDP is useful for time-sensitive applications that have very
small data units to exchange and do not want the overhead of connection setup. UDP
is a transaction oriented and stateless protocol. UDP does not provide guaranteed
delivery, ordering of messages and duplicate elimination. Higher levels of protocols
can ensure reliable delivery or ensuring connections created are reliable. UDP is
described in RFC 768 [10].
Application Layer
Application layer protocols define how the applications interface with the lower layer
} protocols to send the data over the network. The application data, typically in files, is
| encoded by the application layer protocol and encapsulated in the transport layer protocol
which provides connection or transaction oriented communication over the network. Port
numbers are used for application addressing (for example port 80 for HTTP, port 22 for SSH,
etc.). Application layer protocols enable process-to-process connections using ports.
© HTTP : Hypertext Transfer Protocol (HTTP) is the application layer protocol that
forms the foundation of the World Wide Web (WWW). HTTP includes commands
such as GET, PUT, POST, DELETE, HEAD, TRACE, OPTIONS, etc. The protocol
follows a request-response model where a client sends requests to a server using the
HTTP commands. HTTP is a stateless protocol and each HTTP request is independent
of the other requests. An HTTP client can be a browser or an application running
on the client (e.g., an application running on an IoT device, a mobile application or
other software). HITTP protocol uses Universal Resource Identifiers (URIs) to identify
HTTP resources. HTTP is described in RFC 2616 [11].
© COAP : Constrained Application Protocol (CoAP) is an application layer protocol for
‘machine-to-machine (M2M) applications, meant for constrained environments with
constrained devices and constrained networks. Like HTTP, CoAP is a web transfer
pprotocol and uses a request-response model, however it runs on top of UDP instead of”
TCP. CoAP uses a client-server architecture where clients communicate with servers
using connectionless datagrams. CoAP is designed to easily interface with HTTP.
Like HTTP, CoAP supports methods such as GET, PUT; POST, and DELETE. CoAP
draft specifications are available on IEFT Constrained environments (CORE) Working
Group website [12].
internet of Things ~ A Hands-On Approach

— Page 11 —

cu Introduction to Internet of Things
© WebSocket : WebSocket protocol allows full-duplex communication over a single
socket connection for sending messages between client and server. WebSocket is
based on TCP and allows streams of messages to be sent back and forth between the
client and server while keeping the TCP connection open. The client can be a browser,
a mobile application or an IoT device. WebSocket is described in RFC 6455 (13).
© MQTT:: Message Queue Telemetry Transport (MQTT) is a light-weight messaging
protocol based on the publish-subscribe model. MQTT uses a client-server architecture
where the client (such as an IoT device) connects to the server (also called MQTT
Broker) and publishes messages to topics on the server. The broker forwards the
messages to the clients subscribed to topics. MQTT is well suited for constrained
and the network bandwidth is low. MQTT specifications are available on IBM
developerWorks [14].
 XMPP : Extensible Messaging and Presence Protocol (XMPP) is « protocol for
powers wide range of applications including messaging, presence, data syndication,
gaming, multi-party chat and voice/video calls, XMPP allows sending small chunks
of XML data from one network entity to another in near real-time. XMPP is a
decentralized protocol and uses a client-server architecture, XMPP supports both
client-to-server and server-to-server communication paths. In the context of oT,
XMPP allows real-time communication between loT devices. XMPP is described in
RFC 6120 [15].
¢ DDS : Data Distribution Service (DDS) is a data-centric middleware standard for
device-to-device or machine-to-machine communication. DDS uses a publish-subscribe
model where publishers (¢.g. devices that generate data) create topics to which
subscribers (¢.g., devices that want to consume data) can subscribe. Publisher is an
object responsible for data distribution and the subscriber is responsible for receiving
published data, DDS provides quality-of-service (QoS) control and configurable
reliability. DDS is described in Object Management Group (OMG) DDS specification [16].
* AMQP : Advanced Message Queuing Protocol (AMQP) is an open application
layer protocol for business messaging. AMQP supports both point-to-point and
publisher/subscriber models, routing and queuing. AMQP brokers receive messages
from publishers (¢.g.. devices or applications that generate data) and route them over
‘connections to consumers (applications that process data), Publishers publish the
messages to exchanges which then distribute message copies to queues. Messages are
cither delivered by the broker to the consumers which have subscribed to the queues
or the consumers can pull the messages from the queues. AMQP specification is
Banga & Madisetti, ©) 2015

— Page 12 —

1.3 Logical Design of loT 31
available on the AMQP working group website (17].

1.3 Logical Design of loT

Logical design of an IoT system refers to an abstract representation of the entities and

processes without going into the low-level specifics of the implementation. In this section

we describe the functional blocks of an ToT system and the communication APIs that are

used for the examples in this book. The steps in logical design are described in additional

detail in Chapter-5.

1.3.1 loT Functional Blocks

An IoT system comprises of a number of functional blocks that provide the system the

capabilities for identification, sensing, actuation, communication, and management as shown

in Figure 1.6. These functional blocks are described as follows:

© Device : An IoT system comprises of devices that provide sensing, actuation, monitoring
and control functions. You learned about IoT devices in section 1.2.

¢ Communication : The communication block handles the communication for the IoT
system. You learned about various protocols used for communication by IoT systems
in section 1.2,

‘© Services : An IoT system uses various types of IoT services such as services for
device monitoring, device control services, data publishing services and services for
device discovery.

© Management : Management functional block provides various functions to govern
the IoT system.

© Security Security functional block secures the loT system and by providing functions
such as authentication, authorization, message and content integrity, and data security.

Application : IoT applications provide an interface that the users can use to control
and monitor various aspects of the IoT system. Applications also allow users to view
the system status and view or analyze the processed data.

1.3.2 loT Communication Models:

¢ Request-Response : Request-Response is a communication model in which the
client sends requests to the server and the server responds to the requests. When
the server receives a request, it decides how to respond, fetches the data, retrieves
resource representations, prepares the response, and then sends the response to
the client. Request-Response model is a stateless communication model and each
request-response pair is independent of others. Figure 1.7 shows the client-server

Be
Internet of Things - A Hands-On Approach

— Page 13 —

32 Introduction to internet of Things
|
Figure 1.6: Functional Blocks of loT ]
i
i
|
|
:
‘aren
Pare gs ak
oe
ee
meee 4
eeu
ooh copes
car
Figure 1.7: Request-Response communication model j
interactions in the request-response model. |
+ Publish-Subseribe : Publish-Subscribe is a communication model that involves
publishers, brokers and consumers. Publishers are the source of data, Publishers send
the data tothe topics which are managed by the broker. Publishers are not aware of
the consumers. Consumers subscribe to the topics which are managed by the broker.
‘When the broker receives data fora topic from the publisher, it sends the data to all the”
subscribed consumers. Figure 1.8 shows the publisher-broker-consumer interactions
Baha & Madisoti, © 2015

— Page 14 —

4.3 Logical Design of loT 33
. —-
Figure 1.8: Publish-Subscribe communication model
Queues -
= ease a
to queues from queues
Figure 1.9: Push-Pull communication model
in the publish-subscribe model.
 Push-Pull : Push-Pull is a communication model in which the data producers push the
data to queues and the consumers pull the data from the queues. Producers do not need
to be aware of the consumers. Queues help in decoupling the messaging between the —~
producers and consumers. Queues also act as a buffer which helps in situations when
there is a mismatch between the rate at which the producers push data and the rate rate
at which the consumers pull data. Figure 1.9 shows the publisher-queue-consumer
interactions in the push-pull model.
¢ Exclusive Pair : Exclusive Pair is a bi-directional, fully duplex communication model
Internet of Things - A Hands-On Approach

— Page 15 —

34 Introduction to Internet of Things
Request to setup Connection
| Connection close response
|
i Figure 1.10: Exehisive Pair communication model
hy that uses a persistent connection between the client and server. Once the connection is
setup it remains open until the client sends a request to close the connection. Client
and server can send messages to each other after connection setup. Exclusive pair is
‘a stateful communication model and the server is aware of all the open connections.
Figure 1.10 shows the client-server interactions in the exclusive pair model.
1.3.3 loT Communication APIs.
In the previous section you learned about various communication models, In this section
you will learn about two specific communication APIs which are used in the examples in
this book.
REST-based Communication APIs
Representational State Transfer (REST) [88] is a set of architectural principles by which
you can design web services and web APIs that focus on a system’s resources and how
resource states are addressed and transferred. REST APIs follow the request-response
‘communication model described in previous section. The REST architectural constraints
apply to the components, connectors, and data elements, within a distributed hypermedia
system. The REST architectural constraints are as follows:
aa ee
| Bahiga & Madisett, © 2015
i
Va

— Page 16 —

1.3 Logical Design of loT 35
pce =
ie Aa
RTE sence
(ois pe
eee am Mca)
Figure 1.11; Communication with REST APIs

© Client-Server: The principle behind the client-server constraint is the separation of
concerns. For example, clients should not be concerned with the storage of data which
is a concern of the server. Similarly, the server should not be concerned about the user
interface, which is a concern of the client. Separation allows client and server to be
independently developed and updated.

# Stateless: Each request from client to server must contain all the information necessary
to understand the request, and cannot take advantage of any stored context on the
server. The session state is kept entirely on the client.

© Cache-able: Cache constraint requires that the data within a response to a request
be implicitly or explicitly labeled as cache-able or non-cache-able. If a response is
cache-able, then a client cache is given the right to reuse that response data for later,
equivalent requests. Caching can partially or completely eliminate some interactions ~
and improve efficiency and scalability.

© Layered System: Layered system constraint, constrains the behavior of components
such that each component cannot see beyond the immediate layer with which they are
interacting. For example, a client cannot tell whether it is connected directly to the
end server, or to an intermediary along the way. System scalability can be improved

Internet of Things - A Hands-On Approach

— Page 17 —

36 Introduction to Internet of Things
| REST
Request (GET, PUT, UPDATE or DELETE)
with payload (JSON or XML)
Request (GET, PUT, UPDATE or DELETE) /
with payload SON or XML) |
j
|
j
Figure 1.12; Request-response model used by REST
by allowing intermediaries to respond to requests instead of the end server, without j
the client having to do anything different, |
© Uniform Interface: Uniform Interface constraint requires that the method of
communication between a client and a server must be uniform, Resources are identified
in the requests (by URIs in web based systems) and are themselves separate from the
representations of the resources that are returned to the client. When a client holds
a representation of a resource it has all the information required to update or delete
the resource (provided the client has required permissions). Each message includes
enough information to describe how to process the message.
¢ Code on demand: Servers can provide executable code or scripts for clients to execute
in their context, This constraint is the only one that is optional.

A RESTful web service is a "web API" implemented using HTTP and REST principles. 1
Figure 1.11 shows the communication between client and server using REST APIs. Figure 1.12 j
shows the interactions in the request-response model used by REST. RESTful web service
is a collection of resources which are represented by URIs. RESTful web API has a base
URI (e.g. http://example.com/api/tasks/). The clients send requests to these URIs using the |
Ee

‘Bahga & Madisettl, © 2015

— Page 18 —

. N
1.3 Logical Design of loT 37
Blemem URI | G2 information | hitp:/example.con/api/asks/1/
cise about a resource | (get information on task-1)
oe
ipdass http://example.com/api/tasks/1/
PUT Element URI pa 41 (update task-1 with data
eae: provided in the request)
Delete a | httpy/example.com/api/tasks/1/
DELETE | Element URI | escurce (delete task-1)
‘Table 1.1: HTTP request methods and actions
methods defined by the HTTP protocol (¢.g., GET, PUT, POST, or DELETE). as shown in
Table 1.1, A RESTful web service can support various Internet media types (JSON being the
most popular media type for RESTful web services). IP for Smart Objects Alliance (IPSO
Alliance) has published an Application Framework that defines a RESTful design for use in
IP smart object systems [18]. a
WebSocket-based Communication APIs
WebSocket APIs allow bi-directional, full duplex communication between clients and servers.
WebSocket APIs follow the exclusive pair communication model described in previous
section and as shown in Figure 1.13. Unlike request-response APIs such as REST, the
Internet of Things - A Hands-On Approach

— Page 19 —

38 Introduction to Internet of Things
WebSocket Protocol
Request to setup WebSochet Connection
oe
ree pose S| fu alee
: Bidrectional Communication
(over persistent
aaa peieae acu || a
—
Figure 1.13: Exclusive pair model used by WebSocket APIs
‘WebSocket APIs allow full duplex communication and do not require a new connection to
be setup for each message to be sent. WebSocket communication begins with a connection
setup request sent by the client to the server. This request (called a WebSocket handshake)
is sent over HTTP and the server interprets it as an upgrade request. If the server supports
WebSocket protocol, the server responds to the WebSocket handshake response. After the
connection is setup, the client and server can send data/messages to each other in full-duplex
mode, WebSocket APIs reduce the network traffic and latency as there is no overhead for
connection setup and termination requests for each message. WebSocket is suitable for oT
applications that have low latency or high throughput requirements.
1.4 loT Enabling Technologies
IoT is enabled by several technologies including wireless sensor networks, cloud computing,
big data analytics, embedded systems, security protocols and architectures, communication
protocols, web services, mobile Internet, and semantic search engines. This section provides
‘an overview of some of these technologies which play a key-role in oT.
ee
Bahga & Madisetti, © 2015

— Page 20 —

aa —
1.4 loT Enabling Technologies 39
1.4.1. Wireless Sensor Networks
A Wireless Sensor Network (WSN) comprises of distributed devices with sensors which are
used to monitor the environmental and physical conditions. A WSN consist of a number
| of end-nodes and routers and a coordinator. End nodes have several sensors attached to
| them. End nodes can also act as routers. Routers are responsible for routing the data packets
from end-nodes to the coordinator. The coordinator collects the data from all the nodes.
Coordinator also acts as a gateway that connects the WSN to the Internet, Some examples of
| WSNS used in IoT systems are described as follows:
: ‘ Weather monitoring systems use WSNs in which the nodes collect temperature,
} humidity and other data, which is aggregated and analyzed.
j ‘© Indoor air quality monitoring systems use WSNs to collect data on the indoor air
| quality and concentration of various gases.
‘* Soil moisture monitoring systems use WSNs to monitor soil moisture at various
locations.
| © Surveillance systems use WSNs for collecting surveillance data (such as motion
detection data)
‘© Smart grids use WSNs for monitoring the grid at various points.
‘© Structural health monitoring systems use WSNs to monitor the health of structures
(buildings, bridges) by collecting vibration data from sensor nodes deployed at various
points in the structure.
WSNs are enabled by wireless communication protocols such as IEEE 802.15.4. ZigBee
is one of the most popular wireless technologies used by WSNS. ZigBee specifications are
based on IEEE 802.15.4. ZigBee operates at 2.4 GHz frequency and offers data rates upto
| 250 KB/s and range from 10 to 100 meters depending on the power output and environmental
| conditions. The power of WSNS lies in their ability to deploy large number of low-cost.
and low-power sensing nodes for continuous monitoring of environmental and physical
conditions, WSNs are self-organizing networks. Since WSNs have large number of nodes,
‘manual configuration for each node is not possible. The self-organizing capability of WSN
makes the network robust. In the event of failure of some nodes or addition of new nodes to
the network, the network can reconfigure itself.
1.4.2 Cloud Computing
: Cloud computing is a transformative computing paradigm that involves delivering applications
and services over the Internet, Cloud computing involves provisioning of computing,
networking and storage resources on demand and providing these resources as metered
services to the users, in a "pay as you go” model, Cloud computing resources can be
provisioned on-demand by the users, without requiring interactions with the cloud service
Internet of Things - A Hands-On Approach

— Page 21 —

40 Introduction to Internet of Things
provider. The process of provisioning resources is automated. Cloud computing resources
can be accessed over the network using standard access mechanisms that provide
platform-independent access through the use of heterogeneous client platforms such as
workstations, laptops, tablets and smart-phones. ‘The computing and storage resources
provided by cloud service providers are pooled to serve multiple users using multi-tenancy.
Multi-tenant aspects of the cloud allow multiple users to be served by the same physical |
hardware, Users are assigned virtual resources that run on top of the physical resources. j
Cloud computing services are offered to users in different forms (see the authors’
‘companion book on Cloud Computing, for instance);

Infrastructure-as-a-Service (IaaS) : IaaS provides the users the ability to provision
‘computing and storage resources, These resources are provided to the users as virtual
machine instances and virtual storage. Users can start, stop, configure and manage
the virtual machine instances and virtual storage. Users can deploy operating systems
and applications of their choice on the virtual resources provisioned in the cloud.
‘The cloud service provider manages the underlying infrastructure. Virtual resources
provisioned by the users are billed based on a pay-per-use paradigm.

« Platform-as-a-Service (PaaS) : PaaS provides the users the ability to develop and
deploy application in the cloud using the development tools, application programming
interfaces (APIs), software libraries and services provided by the cloud service
provider. The cloud service provider manages the underlying cloud infrastructure
including servers, network, operating systems and storage. The users, themselves, are
responsible for developing, deploying, configuring and managing applications on the
cloud infrastructure,

¢ Software-as-a-Service (SaaS) : SaaS provides the users a complete software application
or the user interface to the application itself. The cloud service provider manages
the underlying cloud infrastructure including servers, network, operating systems,
storage and application software, and the user is unaware of the underlying architecture
of the cloud. Applications are provided to the user through a thin client interface
(e.g., a browser). SaaS applications are platform independent and can be accessed
from various client devices such as workstations, laptop, tablets and smart-phones,
running different operating systems. Since the cloud service provider manages both
the application and data, the users are able to access the applications from anywhere.

1.4.3 Big Data Analytics

Big data is defined as collections of data sets whose volume, velocity (in terms of its temporal.

variation), or variety, is so large that it is difficult to store, manage, process and analyze

the data using traditional databases and data processing tools. Big data analytics involves
Bahga & Madisetti, © 2015

— Page 22 —

EK
1.4 loT Enabling Technologies a
several steps starting from data cleansing, data munging (or wrangling), data processing and
visualization. Some examples of big data generated by IoT systems are described as follows:
# Sensor data generated by IoT systems such as weather monitoring stations.
Machine sensor data collected from sensors embedded in industrial and energy systems
for monitoring their health and detecting failures.
¢ Health and fitness data generated by 1oT devices such as wearable fitness bands.
© Data generated by IoT systems for location and tracking of vehicles.
F ¢ Data generated by retail inventory monitoring systems.
The underlying characteristics of big data include:
Volume: Though there is no fixed threshold for the volume of data to be considered
as big data, however, typically, the term big data is used for massive scale data that is
; difficult to store, manage and process using traditional databases and data processing
architectures. The volumes of data generated by modem IT, industrial, and health-care
systems, for example, is growing exponentially driven by the lowering costs of data
storage and processing architectures and the need to extract valuable insights from the
data to improve business processes, efficiency and service to consumers.
# Velocity: Velocity is another important characteristic of big data and the primary
reason for exponential growth of data. Velocity of data refers to how fast the data is
‘generated and how frequently it varies. Modern IT, industrial and other systems are
generating data at increasingly higher speeds.
‘© Variety: Variety refers to the forms of the data. Big data comes in different forms
; such as structured or unstructured data, including text data, image, audio, video and
P sensor data,
1.4.4 Communication Protocols
Communication protocols form the backbone of IoT systems and enable network connectivity
and coupling to applications. Communication protocols allow devices to exchange data
over the network. In section 1.2.2 you learned about various link, network, transport
"and application layer protocols, These protocols define the data exchange formats, data
encoding, addressing schemes for devices and routing of packets from source to destination.
Other functions of the protocols include sequence control (that helps in ordering packets
determining lost packets), flow control (that helps in controlling the rate at which the sender
___issending the data so that the receiver or the network is not overwhelmed) and retransmission
of lost packets.
Internet of Things - A Hands-On Approach

— Page 23 —

42 Introduction to Internet of Things
1.4.5 Embedded Systems
An Embedded System is a computer system that has computer hardware and software
embedded to perform specific tasks. In contrast to general purpose computers or personal
computers (PCs) which can perform various types of tasks, embedded systems are designed
to perform a specific set of tasks. Key components of an embedded system include,
microprocessor or microcontroller, memory (RAM, ROM, cache), networking units (Ethernet,
WiFi adapters), input/output units (display, keyboard, etc.) and storage (such as flash’
memory). Some embedded systems have specialized processors such as digital signal
processors (DSPs), graphics processors and application specific processors. Embedded
systems run embedded operating systems such as real-time operating systems (RTOS).
Embedded systems range from low-cost miniaturized devices such as digital watches to
devices such as digital cameras, point of sale terminals, vending machines, appliances (such
as washing machines), etc, In the next chapter we describe how such devices form an integral
part of oT systems.

1
1.5 loT Levels & Deployment Templates }
In this section we define various levels of loT systems with increasing completely. An oT |
system comprises of the following components: i

* Device: An IoT device allows identification, remote sensing, actuating and remote
monitoring capabilities, You learned about various examples of loT devices in section
12.1, j

Resource: Resources are software components on the IoT device for accessing,
processing, and storing sensor information, or controlling actuators connected to the
device, Resources also include the software components that enable network access
for the device.

© Controller Service: Controller service is a native service that runs on the device
and interacts with the web services. Controller service sends data from the device to
the web service and receives commands from the application (via web services) for
controlling the device. j

‘© Database: Database can be either local or in the cloud and stores the data generated
by the lo device.

‘© Web Service: Web services serve as a link between the IoT device, application,
database and analysis components. Web service can be either implemented using
HTTP and REST principles (REST service) or using WebSocket protocol (WebSocket
service). A comparison of REST and WebSocket is provided below:

ae
Bahga & Madisett, © 2015
1

— Page 24 —

—e as
1.5 loT Levels & Deployment Templates 43
~ Stateless/Stateful: REST services are stateless in nature. Each request contains
all the information needed to process it. Requests are independent of each other.
WebSocket on the other hand is stateful in nature where the server maintains the
state and is aware of all the open connections.
~ Uni-directional/Bi-directional: REST services operate over HTTP and are
uni-directional, Request is always sent by a client and the server responds to the
requests, On the other hand, WebSocket is a bi-directional protocol and allows
both client and server to send messages to each other.
— Request-Response/Full Duplex: REST services follow a request-response
j ‘communication model where the client sends requests and the seryer responds
to the requests, WebSocket on the other hand allow full-duplex communication
t between the client and server, i.e., both client and server can send messages to
i each other independently.
; ~ TCP Connections: For REST services, each HTTP request involves setting up
} anew TCP connection. WebSocket on the other hand involves a single TCP
P connection over which the client and server communicate in a full-duplex mode.
~ Header Overhead: REST services operate over HTTP, and each request is
independent of others. Thus each request carries HTTP headers which is an
5 overhead, Due the overhead of HTTP headers, REST is not suitable for real-time
applications. WebSocket on the other hand does not involve overhead of headers.
5 After the initial handshake (that happens over HTTP), the client and server
ie exchange messages with minimal frame information. Thus WebSocket is suitable
zi for real-time applications.
Bs = Scalability: Scalability is easier in the case of REST services as requests are
Bhs independent and no state information needs to be maintained by the server.
ao ‘Thus both horizontal (scaling-out) and vertical scaling (scaling-up) solutions
a are possible for REST services. For WebSockets, horizontal scaling can be
cumbersome due to the stateful nature of the communication, Since the server
oa maintains the state of a connection, vertical scaling is easier for WebSockets
than horizontal scaling.

_ @ Analysis Component: The Analysis Component is responsible for analyzing the 1oT
data and generate results in a form which are easy for the user to understand. Analysis
of loT data can be performed either locally or in the cloud. Analyzed results are stored
inthe local or cloud databases.

__ @ Application: IoT applications provide an interface that the users can use to control
and monitor various aspects of the oT system. Applications also allow users to view
___ the system status and view the processed data.

internet of Things - A Hands-On Approach
B.

y

— Page 25 —

!
44 Introduction to Internet of Things
1.5.1 loT Levelt
A level-I IoT system has a single node/device that performs sensing and/or actuation, stores
data, performs analysis and hosts the application as shown in Figure 1.14, Level-1 IoT
systems are suitable for modeling low-cost and low-complexity solutions where the data
involved is not big and the analysis requirements are not computationally intensive,

Let us now consider an example of a level-1 IoT system for home automation. The
system consists of a single node that allows controlling the lights and appliances in a home
remotely. The device used in this system interfaces with the lights and appliances using
electronic relay switches. The status information of each light or appliance is maintained in
a local database, REST services deployed locally allow retrieving and updating the state of
each light or appliance in the status database. The controller service continuously monitors
the state of each light or appliance (by retrieving state from the database) and triggers the
relay switches accordingly. The application which is deployed locally has a user interface
for controlling the lights or appliances. Since the device is connected to the Internet, the

f application can be accessed remotely as well. !
1.5.2. loT Level-2
A level-2 JoT system has a single node that performs sensing and/or actuation and local
analysis as shown in Figure 1,15. Data is stored in the cloud and application is usually
cloud-based. Level-2 IoT systems are suitable for solutions where the data involved is big,
however, the primary analysis requirement is not computationally intensive and can be done
locally itself.

Let us consider an example of a level-2 IoT system for smart irrigation, The system
consists of a single node that monitors the soil moisture level and controls the irrigation
system. The device used in this system collects soil moisture data from sensors. The
controller service continuously monitors the moisture levels. If the moisture level drops
below a threshold, the irrigation system is turned on. For controlling the irrigation system
actuators such as solenoid valves can be used. The controller also sends the moisture data to
the computing cloud. A cloud-based REST web service is used for storing and retrieving
moisture data which is stored in the cloud database. A cloud-based application is used for
visualizing the moisture levels over a period of time, which can help in making decisions |
about irrigation schedules. ]
1.5.3 loT Level-3 |
A level-3 IoT system has a single node, Data is stored and analyzed in the cloud and
application is cloud-based as shown in Figure 1.16. Level-3 IoT systems are suitable for

Bahga & Madisetti, © 2015
|
|
l

— Page 26 —

F
4.5 loT Levels & Deployment Templates 45
a loT Level-1
I
: Local | Cloud
i ‘
a |
Bi: { :
Bi REST/WebSocket ‘i
i Communication '
'
; i
: '
j I
A '
a I
BS |
a 1
Bi 1
() 1
ce 1
a |
i I
1
A \
Be I
Fae 1
ee 1
o Monitoring Node
of performs analysis, stores data.
as
ig ts Figure 1.14: loT Level-1
solutions where the data involved is big and the analysis requirements are computationally
"Letts consider an example of a level-2 1oT system for tracking package handling. The ~
em consists of a single node (for a package) that monitors the vibration levels for a
[package being shipped. The device in this system uses accelerometer and gyroscope sensors
lor monitoring vibration levels. The controller service sends the sensor data to the cloud
‘inreal-time using a WebSocket service. The data is stored in the cloud and also visualized
‘using a cloud-based application. The analysis components in the cloud can trigger alerts if
ak

— Page 27 —

46 Introduction to Internet of Things
loT Level-2 }
Local | Cloud
1
\
\
1
1
\
{ REST/WebSocket
REST/WebSocket Communication
Communication
ia aa i i
'
\
1 RES)
\ ;
!
\
Monitoring Node “
performs analysis Cloud Storage
Figure 1.15: loT Level-2
the vibration levels become greater than a threshold. The benefit of using WebSocket service
instead of REST service in this example is that, the sensor data can be sent in real time to the
cloud, Moreover, cloud based applications can subscribe to the sensor data feeds for viewing
the real-time data.
1.5.4 loT Level-4
‘A level-4 IoT system has multiple nodes that perform local analysis. Data'is stored in the
cloud and application is cloud-based as shown in Figure 1.17. Level-4 contains local and
ea eee at
Bahga & Madisetti, © 2015

— Page 28 —

! 1,5 loT Levels & Deployment Templates 47
loT Level-3
Local ’ Cloud
1
1
1
1
1
'
; 1 REST/WebSocket
REST/WebSocket Communication
Sominuncaion
1
1
1
1
1
1
1
1
Monitoring Node : i
Cloud Storage & Analysis
Figure 1.16: loT Level-3
cloud-based observer nodes which can subscribe to and receive information collected in
the cloud from IoT devices. Observer nodes can process information and use it for various -
| applications, however, observer nodes do not perform any control functions. Level-4 loT
systems are suitable for solutions where multiple nodes are required, the data involved is big
and the analysis requirements are computationally intensive.
Let us consider an example of a level-4 IoT system for noise monitoring. The system
consists of multiple nodes placed in different locations for monitoring noise levels in an area.
| Internet of Things - A Hands-On Approach
|

— Page 29 —

i _rtninatucttonitoliriomnet TOS
‘The nodes in this example are equipped with sound sensors. Nodes are independent of each
other. Each node runs its own controller service ‘that sends the data to the cloud. The data is
stored in a cloud database. The analysis of data collected from a number of nodes is done in
the cloud, A cloud-based application is used for visualizing the aggregated data.
loT Level-4
| Local | Cloud
'
'
'
| = | |
Communication hisead
'
'
ese :
| 5 mt { \
'
| gaa
'
Viagae
sever tay
coud tone
Figure 1.17: IoT Level-4
15.5 loT Level-5
A level-5 IoT system has multiple end nodes and one coordinator node as shown in
Figure 1.18. The end nodes that perform sensing and/or actuation. Coordinator node
collects data from the end nodes and sends to the cloud. Data is stored and analyzed inthe
cloud and application is cloud-based. Level-5 IoT systems are suitable for solutions based |
‘on wireless sensor networks, in which the data involved is big and the analysis requirements |
‘are computationally intensive.

Let us consider an example of a level-5 IoT system for forest fire detection. The system
consists of multiple nodes placed in different locations for monitoring temperature, humidity
and carbon dioxide (CQ>) levels in a forest. ‘The end nodes in this example are equipped with
(i  ,

Bahga & Madisetti, © 2015
| “

— Page 30 —

oan
1.5 loT Levels & Deployment Templates 49
various sensors (such as temperature, humidity and CO2). The coordinator node collects
the'data from the end nodes and acts as a gateway that provides Internet connectivity to the
IoT system. The controller service on the coordinator device sends the collected data to the
cloud. The data is stored in a cloud database. ‘The analysis of data is done in the computing
cloud to aggregate the data and make predictions. A cloud-based application is used for
, visualizing the data.
i loT Level-5
if Local + Cloud
i
I
ni
i
EST/WebSo0
Communication
|
i
1
1
k {
= !
E I
| foe
F Coordinator
‘ oud storage 8
E ‘nails
s Figure 1.18: IoT Level-5
1.5.6 loT Level-6
A level-6 IoT system has multiple independent end nodes that perform sensing and/or
actuation and send data to the cloud. Data is stored in the cloud and application is cloud-based
‘as shown in Figure 1.19. The analytics component analyzes the data and stores the results
i. inthe cloud database. The results are visualized with the cloud-based application. The
‘centralized controller is aware of the status of all the end nodes and sends control commands
Internet of Things ~ A Hands-On Approach 5

— Page 31 —

50 Introduction to Internet of Things
to the nodes.
' Let us consider-an example of @ level-6 IoT system for weather monitoring: ‘The
| system consists of multiple nodes placed in different locations for monitoring temperature,
| humidity and pressure in an area. The end nodes are equipped with various sensors (such as
| temperature, pressure and humidity), ‘The end nodes send the data to the cloud in real-time
| using a WebSocket service. The data is stored ina cloud database. The analysis of data is
i done in the cloud to aggregate the data and make predictions. A cloud-based application is
used for visualizing the data.
loT Level-6
| Local I ‘Cloud
t '
} t ager
‘REST /WebSocket Fone
= Ea
ew ia
} / iz
'
|
| nie
conrad
OS ee
Analysis:
| Figure 1.19: oT Level-6
|
| Summary
| Internet of Things (IoT) refers to physical and virtual objects that have unique identities
and are connected to the Internet. This allows the development of intelligent applications
that make energy, logistics, industrial control, retail, agriculture and many other domains of
oS a eer
| Bahga & Madisetti, © 2015

— Page 32 —

"15 loT Levels & Deployment Templates 51
human endeavour "smarter". IoT allows different types of devices, appliances, users and
‘machines to communicate and exchange data. The applications of Internet of Things (IoT)
span a wide range of domains including (but not limited to) homes, cities, environment,
"energy systems, retail, logistics, industry, agriculture and health. Things in IoT refers to
| "ToT devices which have unique identities and allow remote sensing, actuating and remote
"monitoring capabilities. Almost all [oT devices generate data in some form or the other
"which when processed by data analytics systems leads to useful information to guide further
_— actions. ‘You learned about IoT protocols for link, network, transport and application
"layers. Link layer protocols determine how the data is physically sent over the network. The
"network/internet layers is responsible for sending of IP datagrams from the source network to
the destination network. The transport layer protocols provides end-to-end message transfer
capability independent of the underlying network, Application layer protocols define how the
applications interface with the lower layer protocols to send the data over the network. You
"earned about functional blocks of an IoT system including device communication, services,
‘management, security and application blocks. You learned about [oT communication models
{ such as request-response, publish-subscribe, push-pull and exclusive pair. You learned about
F REST-based and WebSocket-based communication APIs. REST is a set of architectural
principles by which you can design web services and web APIs that focus on a system's
resources and how resource states are addressed and transferred. A RESTful web service
is a web API implemented using HTTP and REST principles. WebSocket APIs allow
bi-directional, full duplex communication between clients and servers. You learned about
‘enabling technologies of IoT such as wireless sensor networks, cloud computing, big data
analytics, communication protocols and embedded systems, Finally, you learned about loT
levels. A level-1 IoT system has a single node/device that performs sensing and/or actuation,
stores data, performs analysis and hosts the application. A level-2 IoT system has a single
node that performs sensing and/or actuation and local analysis. A level-3 IoT system has
a single node. Data is stored and analyzed in the cloud and application is cloud-based. A
level-4 IoT system has multiple nodes that perform local analysis. Data is stored in the
cloud and application is cloud-based. A level-5 loT system has ettgftrend nodes and one
coordinator node. A level-6 IoT system has multiple indepefgeAt end ite hat perform
sensing and/or actuation and send data to the cloud. ‘opaaw\\
Aes No OLOA SAO :
Review Questions C21 Nn B
s
1. Describe an example of an IoT system in which Inférmtation and kfipivledge are
| inferred from data. Bary
} 2. Why do IoT systems have to be self-adapting and self-configuring?
| ain as aaiibe eee Se
| Internet of Things - A Hands-On Approach 6

— Page 33 —

52 Introduction to Internet of Things
3. What is the role of things and Internet in loT?
4, What is the function of communication functional block in an JoT system? q
5. Describe an example of IoT service that uses publish-subscribe communication model,
6. Describe an example of IoT service that uses WebSocket-based communication.
7. What are the architectural constraints of REST?
8. What is the role of a coordinator in wireless sensor network?
9. What is the role of a controller service in an IoT system? ‘v
a
a ee
zi Bahga & Madisetti, © 2015

— Page 34 —

We

Fe soiiaie Scans . 5, ais gente ie o
| IgT Appice tor Dialect
i
/ © Energy Systems
_ @ Retail nah Si ‘
(|S tosistis a
| eae ne
| Sa erie De eer ha!
f
i
|
te
| :
f

— Page 35 —

54 Domain Specific loTs
2.1 Introduction
‘The Internet of Things (IoT) applications span a wide range of domains including (but not
limited to) homes, cities, environment, energy systems, retail, logistics, industry, agriculture
and health. This chapter provides an overview of various types of IoT applications for each
of these domains. In the later chapters the reader is guided through detailed implementations
of several of these applications.
2.2 Home Automation
2.2.1 Smart Lighting
‘Smart lighting for homes helps in saving energy by adapting the lighting to the ambient
conditions and switching on/off or dimming the lights when needed. Key enabling technologies
for smart lighting include solid state lighting (such as LED lights) and IP-enabled lights. For
Solid state lighting solutions both spectral and temporal characteristics can be configured
to adapt illumination to various needs, Smart lighting solutions for home achieve energy
savings by sensing the human movements and their environments and controlling the lights
accordingly. Wireless-enabled and Internet connected lights can be controlled remotely
from IoT applications such as a mobile or web application. Smart lights with sensors for
‘occupancy, temperature, lux level, etc., can be configured to adapt the lighting (by changing
the light intensity, color, etc.) based on the ambient conditions sensed, in order to provide a
good ambiance, In [19] controllable LED lighting system is presented that is embedded with
ambient intelligence gathered from a distributed smart wireless sensor network to optimize
and control the lighting system to be more efficient and user-oriented, A solid state lighting
model is described in [20] and implemented on a wireless sensor network that provides
services for sensing illumination changes and dynamically adjusting luminary brightness
according to user preferences. In chapter-9 we provide a case study on a smart lighting
system,
2.2.2 Smart Appliances
Modern homes have a number of appliances such as TVs, refrigerators, music systems,
‘washer/dryers, etc. Managing and controlling these appliances can be cumbersome, with
each appliance having its own controls or remote control’. Smart appliances make the
management easier and also provide status information to the users remotely. For example,
smart washer/dryers that can be controlled remotely and notify when the washing/drying
cycle is complete. Smart thermostats allow controlling the temperature remotely and can
learn the user preferences [22]. Smart refrigerators can keep track of the items stored
(using RFID tags) and send updates to the users when an item is low on stock. Smart
Bahga & Madisetti, © 2015

— Page 36 —

2:2: Home Automation 55
“Ys allows users to search and stream videos and movies from the Intémet/on a local
"storage drive, search TV channel schedules and fetch news, weather updates and other
‘content from the Internet. OpenRemote [21] is an open source automation platform for
“homes and buildings. OpenRemote is platform agnostic and works with standard hardware,
‘With OpenRemote, users can control various appliances using mobile or web applications.
“OpenRemote comprises of three components - a Controller that manages scheduling and
"runtime integration between devices, « Designer that allows you to create both configurations
"for the controller and create user interface designs and Control Panels that allow you to
" finleract with devices and control them. An JoT-based appliance control system for smart
"homes is described in [23], that uses a smart central controller to set up a wireless sensor
‘and actuator network and control modules for appliances.

Bam

a og

Reeth

Hs 8} = xR es

Fe ch Re ting oF fn

Rati ig) Ties a. ¢

Be wi fv to te RN

fet é Ry

a sehc ae

io a — ris |

E 2 age i o> i
om & oh eae mig) ri}

Fe ses (i ee | tee

.—d | Cees. fice

> ad ee bee ee" eee

BP Su “page o il ae Cal ea

fie ae Si ge Fn Fis

Per eRS SS a4 so

so) pe NS a =

i = S Fei

Bee ne SK W =

i same \ Son. 3% i

Figure 2.1: Applications of IoT for homes

Oe

Internet of Things - A Hands-On Approach

— Page 37 —

56 Domain Specific loTs
2.2.3. Intrusion Detection
Home intrusion detection systems use security cameras and sensors (such as PIR sensors and
door sensors) to detect intrusions and raise alerts. Alerts can be in the form of an SMS or
‘an email sent to the user. Advanced systems can even send detailed alerts such as an image
‘grab or a short video clip sent as an email attachment. A cloud controlled intrusion detection
system is described in [24] that uses location-aware services, where the geo-location of each
node of a home automation system is independently detected and stored in the cloud. In
the event of intrusions, the cloud services alert the accurate neighbors (who are using the
home automation system) or local police. In {25}, an intrusion detection system based on
UPnP technology is described. The system uses image processing to recognize the intrusion
and extract the intrusion subject and generate Universal-Plug-and-Play (UPnP-based) instant
‘messaging for alerts, In chapter-9 we provide a case study on an intrusion detection system.
2.2.4 Smoke/Gas Detectors
‘Smoke detectors are installed in homes and buildings to detect smoke that is typically an early
sign of fire. Smoke detectors use optical detection, ionization or air sampling techniques to
detect smoke. Alerts raised by smoke detectors can be in the form of signals to a fire alarm
system. Gas detectors can detect the presence of harmful gases such as carbon monoxide
(CO), liquid petroleum gas (LPG), etc. A smart smoke/gas detector [22] can raise alerts in
human voice describing where the problem is, send or an SMS or email to the user or the
local fire safety department and provide visual feedback on its status (healthy, battery-low,
etc.), In [26], the design of a system that detects gas leakage and smoke and gives visual
level indication, is described.
2.3 Cities 3
2.3.1 Smart Parking
Finding a parking space during rush hours in crowded cities can be time consuming and
frustrating, Furthermore, drivers blindly searching for parking spaces create additional traffic
congestion. Smart parking make the search for parking space easier and convenient for
drivers, Smart parking are powered by ToT systems that detect the number of empty parking
slots and send the information over the Internet to smart parking application back-ends.
‘These applications can be accessed by the drivers from smart-phones, tablets and in-car
navigation systems. In smart parking, sensors are used for each parking slot, to detect
whether the slot is empty or occupied. This information is aggregated by a local controller
and then sent over the Internet to the database. In [29], Polycarpou et. al. describe latest
trends in parking availability monitoring, parking reservation and dynamic pricing schemes.
Bahga & Madisetti, © 2015

— Page 38 —

233 Cities 87
Design and implementation of a prototype smart parking system based on wireless sensor
| network technology with features like remote parking monitoring, automated guidance, and
| __ parking reservation mechanism is described in [30]. In chapter-9 we provide a case study
K ‘on a smart parking system.
ae
R #) erent Ss ws
fe : 1 iy aa
; au La
c= ale 4 ny ae Shs
Bs pn) einen
| Qe eels
(| gad fe ey eae Ti
(| Be Cae
Sie Be a
Ee * pacar
I Figure 2.2: Applications of IoT for cities
| 23:2. Smart Lighting
| Smiart lighting systems for roads, parks and buildings can help in saving energy. According
| tan IEA report [27], lighting is responsible for 19% of global electricity use and around
| 6% of global greenhouse gas emissions. Smart lighting allows lighting to be dynamically
| controlled and also adaptive to the ambient Conditions. Smart lights connected to the Internet
| can be controlled remotely to configure lighting schedules and lighting intensity, Custom”
"lighting configurations can be set for different situations such as a foggy day, a festival,
etc, Smart lights equipped with sensors can communicate with other lights and exchange
| information on the sensed ambient conditions to adapt the lighting. Castro et. al. (28]
| describe the need for smart lighting system in smart cities, smart lighting features and how
to develop interoperable smart lighting solutions.
Internet of Things - A Hands-On Approach
|
|

— Page 39 —

58 Domain Specific loTs
2.3.3 Smart Roads
‘Smart roads equipped with sensors can provide information on driving conditions, travel time
estimates and alerts in case of poor driving conditions, traffic congestions and accidents. Such
information can help in making the roads safer and help in reducing traffic jams. Information
sensed from the roads can be communicated via Internet to cloud-based applications and
social media and disseminated to the drivers who subscribe to such applications. In [31], a
distributed and autonomous system of sensor network nodes for improving driving safety.
on public roads in proposed. The system can provide the drivers and passengers with a
consistent view of the road situation a few hundred meters ahead of them or a few dozen
miles away, so that they can react to potential dangers early enough.
2.3.4 Structural Health Monitoring
‘Structural Health Monitoring systems use a network of sensors to monitor the vibration
levels in the structures such as bridges and buildings. The data collected from these sensors
is analyzed to assess the health of the structures. By analyzing the data it is possible to detect
cracks and mechanical breakdowns, locate the damages to a structure and also calculate the
remaining life of the structure. Using such systems, advance warnings can be given in the
case of imminent failure of the structure. An environmental effect removal based structural
health monitoring scheme in an IoT environment is proposed in [32]. Since structural
health monitoring systems use large number of wireless sensor nodes which are powered by
traditional batteries, researchers are exploring energy harvesting technologies to harvesting
ambient energy, such as mechanical vibrations, sunlight, and wind [33, 34].
2.3.5 Surveillance
‘Surveillance of infrastructure, public transport and events in cities is required to ensure
safety and security. City wide surveillance infrastructure comprising of large number of
distributed and Internet connected video surveillance cameras can be created, The video
feeds from surveillance cameras can be aggregated in cloud-based scalable storage solutions.
Cloud-based video analytics applications can be developed to search for patterns or specific
events from the yideo feeds. In [35] a smiart city surveillance system is described that
leverages benefits of cloud data stores. ~ '*
2.3.6 Emergency Response
IoT systems can be used for monitoring the critical infrastructure in cities such as buildings,
‘gas and water pipelines, public transport and power substations. loT systems for fire
detection, gas and water teakage detection can help in generating alerts and minimizing
Bahga & Madisetti, © 2015

— Page 40 —

— Page 41 —

60 Domain Specific loTs
‘AK
—— a rilh 1 ae
en P K i . Eo
apap aan NE TT Aca
AR A Ee Ta ee
ee a
EA | AEN RR rE,
beri iio. eas a Rae
: soni : a
ae sy A
ge 4; Sees pk
ESB Yh) ete
ae EY) Bie
Se ae or s
LEE; :
“fiver Flood Detaeion
Figure 2.3: Applications of IoT for environment
2.4.3. Noise Pollution Monitoring
Due to growing urban development, noise levels in tities have increased and even become
alarmingly high in some cities. Noise pollution caf cause health hazards for humans due to
sleep disruption and stress. Noise pollution monitoring can help in generating noise maps
for cities. Urban noise maps can help the policy makers in urban planning and making
policies to control noise levels near residential areas, schools and parks, IoT based noise
pollution monitoring systems use a number of noise monitoring stations that are deployed at
different places in a city. The data on noise levels from the stations is collected on servers
or in the cloud. The collected data is then aggregated to generate noise maps. In [43], @
noise mapping study for a city is presented which revealed that the city suffered from serious
noise pollution. In [44], the design of smart phone application is described that allows the
users to continuously measure noise levels and send to a central server where all generated
Bahga & Madisett, © 2015

— Page 42 —

Energy 61
“information is aggregated and mapped to a meaningful noise visualization map.
nie
| 24.4. Forest Fire Detection
Forest fires can cause damage to natural resources, property and human life. There can be
| different causes of forest fires including lightening, human negligence, volcanic eruptions and
spatks from rock falls. Early detection of forest fires can help in minimizing the damage, IoT
"based forest fire detection systems use a number of monitoring nodes deployed at different
| Jocations in a forest. Each monitoring node collects measurements on ambient conditions
including temperature, humidity, light levels, etc. A system for early detection of forest
| fires is described in [45] that provides early warning of a potential forest fire and estimates
the scale and intensity of the fire if it materializes. In [46], a forest fire detection system
based on wireless sensor networks in presented. The system uses multi-criteria detection
__ which is implemented by the artificial neural network (ANN). The ANN fuses sensing data
corresponding to multiple attributes of a forest fire (such as temperature, humidity, infrared
| and visible light) to detect forest fires.
245 River Floods Detection
River floods can cause extensive damage to the natural and human resources and human
life. River floods occur due to continuous rainfall which cause the river levels to rise and
flow rates to increase rapidly. Early warnings of floods can be given by monitoring the
‘water level and flow rate. IoT based river flood monitoring system use a number of sensor
nodes that monitor the water level (using ultrasonic sensors) and flow rate (using the flow
velocity sensors). Data from a number of such sensor nodes is aggregated in a server or in
the cloud. Monitoring applications raise @lerts when rapid increase in water level and flow
rate is detected. In [47], a river flood monitoring system in described that measures river and
‘weather conditions through wireless sensor nodes equipped with different sensors. In [48], 2
motes-based sensor network for river flood monitoring is described. The system includes a
water level monitoring module, network video recorder module, and data processing module
that provides flood information in the form of raw data, predicted data, and video feed.
2.5 Energy ;
2.5.1. Smart Grids
‘Smart Grid is a data communications network integrated with the electrical grid that collects
and analyzes data captured in near-real-time about power transmission, distribution, and
consumption, Smart Grid technology provides predictive information and recommendations
to utilities, their suppliers, and their customers on how best to manage power. Smart
Internet of Things - A Hands-On Approach

— Page 43 —

62 Domain Specific loTs
Grids collect data regarding electricity generation (centralized or distributed), consumption
(instantaneous or predictive), storage (or conversion of energy into other forms), distribution
and equipment health data, Smart grids use high-speed, fully integrated, two-way
‘communication technologies for real-time information and power exchange. By using loT
based sensing and measurement technologies, the health of equipment and the integrity of
the grid can be evaluated. Smart meters can capture almost real-time consumption, remotely
control the consumption of electricity and remotely switch off supply when required, Power
thefts can be prevented using smart metering. By analyzing the data on power generation,
transmission and consumption smart girds can improve efficiency throughout the electric
system, Storage collection and analysis of smarts grids data in the cloud can help in dynamic
optimization of system operations, maintenance, and planning. Cloud-based monitoring of
smart grids data can improve energy usage levels via energy feedback to users coupled with
real-time pricing information, Real-time demand response and management strategies can
bbe used for lowering peak demand and overall load via appliance control and energy storage
mechanisms. Condition monitoring data collected from power generation and transmission
systems can help in detecting faults and predicting outages. In [49], application of [oT in
smart grid power transmission is described.
2.5.2 Renewable Energy Systems
Due to the variability in the output from renewable energy sources (such as solar and wind),
integrating them into the grid can cause grid stability and reliability problems. Variable
output produces local voltage swings that can impact power quality. Existing grids were
designed to handle power flows from centralized generation sources to the loads through
transmission and distribution lines, When distributed renewable energy sources are integrated
into the grid, they create power bi-directional power flows for which the grids were not
originally designed, IoT based systems integrated with the transformers at the point of
interconnection measure the electrical variables and how much power is fed into the grid. To
‘ensure the grid stability, one solution is to simply cut off the overproduction. For wind energy
systems, closed-loop controls can be used to regulate the voltage at point of interconnection
which coordinate wind turbine outputs and provides reactive power support [52].
2.5.3 Prognostics |
Energy systems (smart grids, power plants, wind turbine farms, for instance) have a large
number of critical components that must function correctly so that the systems can perform
their operations correctly. For example, a wind turbine has a number of critical components,
e.g., bearings, tuming gears, for instance, that must be monitored carefully as wear and tear
in such critical components or sudden change in operating conditions of the machines can
Bahga & Madisettl, © 2015

— Page 44 —

Bee? emt
BE
i Be sur dep h G

ij i pa TP:

Be iho SY. cs = tia ae

; N HE

i = See RET a Pep
| ERECT craic caret tk tay A uf

Der epee ee Opera cane pte: gages es shat Tf.

| iment 2 ‘ ey |
| Se REE a NIN so byt
| See nee ee |
fi: Sle Ve eo
oo Pia ee

ae Sec ee Meri ener |
bean, Ra ssa

ie aie cee

| RE RRCONIROES

i A =

gai: Figure 2.4: Applications of IoT for energy systems

a

HE ut

| fesuilt in failures. In systems such as power grids, real-time information is collected using
specialized electrical sensors called Phasor Measurement Units (PMU) at the substations,
| The information received from PMUs must be monitored in real-time for estimating the
"state of the system and for predicting failures, Energy systems have thousands of sensors
that gather real-time maintenance data continuously for condition monitoring and failure
___ prediction purposes. IoT based prognostic real-time health management systems can predict
| performance of machines or energy systems by analyzing the extent of deviation of a system
_ fromits normal operating profiles. Analyzing massive amounts of maintenance datacollected
_ from sensors in energy systems and equipment can provide predictions for the impending
__ failures (potentially in real-time) so that their reliability and availability can be improved.
__ Prognostic health management systems have been developed for different energy systems.
_ OpenPDC [50] is a set of applications for processing of streaming time-series data collected
_ from Phasor Measurement Units (PMUs) in real-time. A generic framework for storage,
_ processing and analysis of massive machine maintenance data, collected from a large number
| Wemet of Things ~A Hands-On Approach

— Page 45 —

64 Domain Specific loTs
~ of sensors embedded in industrial machines, in a cloud computing environment was proposed
in [51].
2.6 Retail
2.6.1 Inventory Management
Inventory management for retail has become increasingly important in the recent years with
the growing competition. While over-stocking of products can result in additional storage
‘expenses and risk (in case of perishables), under-stocking can lead to loss of revenue. IoT
systems using Radio Frequency Identification (RFID) tags can help in inventory management
and maintaining the right inventory levels. RFID tags attached to the products allow them
to be tracked in real-time so that the inventory levels can be determined accurately and
products which are low on stock can be replenished, Tracking can be done using RFID
readers attached to the retail store shelves or in the warehouse. IoT systems enable remote
monitoring of inventory using the data collected by the RFID readers. In [53], an RFID
data-based inventory management system for time-sensitive materials is described.
2.6.2 Smart Payments
‘Smart payment solutions such as contact-less payments powered by technologies such as
Near field communication (NFC) and Bluetooh. Near field communication (NFC) is a set of
standards for smart-phones and other devices to communicate with each other by bringing
them into proximity or by touching them. Customers can store the credit card information in
their NEC-enabled smart-phones and make payments by bringing the smart-phones near the
point of sale terminals, NFC maybe used in combination with Bluetooh, where NEC (which
offers low speeds) initiates initial pairing of devices to establish a Bluetooh connection while
the actual data transfer takes place over Bluetooh. The applications of NFC for contact-less
payments are described in [54, 55].
2.6.3 Smart Vending Machines
‘Smart vending machines connected to the Internet allow remote monitoring of inventory
levels, elastic pricing of products, promotions, and contact-less payments using NFC.
Smart-phone applications that communicate with smart vending machines allow user
preferences to be remembered and learned with time. ‘When a user moves from one vending
machine to the other and pairs the smart-phone with the vending machine, a user specific
interface is presented. Users can save their preferences and favorite products. Sensors in
a smart vending machine monitor its operations and send the data to the cloud which can
be used for predictive maintenance. Smart vending machines can communicate with other
cE es
Bahga & Madisetti, © 2015

— Page 46 —

~ 27 Logistics 65
vending machines in their vicinity and share their inventory levels so that the customers
can be routed to the nearest machine in case a product goes out of stock in a machine. For
perishable items, the smart vending machines can reduce the price as the expiry date nears.
‘New products can be recommended to the customers based on the purchase history and
preferences.

—— ax | v 1 a “=P

— ee =
a, (et Ill i oO: <a
sat eg lila eC a

sive ta | aS tay Vo oe

id», Papen au | a eee Ci
Socal meget ie ik hi te “6
aa? care ey A Pee)
Peptic a Ete (fees 9) age
“Hpeeelentig SER ewe, Ae Bi We
ipa aa) is a | Peake yt pay Ne
3 Aimee re Naccee Ne
a SSeS?
oat = | See

fase! | Sh Lz

iM ae

i se prod

its |

=
‘Shar Payment

1 Figure 2.5: Applications of IoT for retail
2.7 Logistics

2.7.1. Route Generation & Scheduling

Modern transportation systems are driven by data collected from multiple sources which
is processed to provide new services to the stakeholders, By collecting large amount of
Re ee
Internet of Things - A Hands-On Approach

— Page 47 —

66 Domain Specific loTs

data from various sources and processing the data into useful information, data-driven

transportation systems can provide new services such as advanced route guidance [62,

63], dynamic vehicle routing (64), anticipating customer demands for pickup and delivery

problem, for instance. Route generation and scheduling systems can generate end-to-end

routes using combination of route patterns and transportation modes and feasible schedules

based on the availability of vehicles, As the transportation network grows in size and

complexity, the number of possible route combinations increases exponentially. IoT based

systems backed by the cloud can provide fast response to the route generation queries and

can be scaled up to serve a large transportation network. j

2.7.2 Fleet Tracking

Vehicle fleet tracking systems use GPS technology to track the locations of the vehicles in

real-time, Cloud-based fleet tracking systems can be scaled up on demand to handle large

number of vehicles. Alerts can be generated in case of deviations in planned routes. The

vehicle locations and routes data can be aggregated and analyzed for detecting bottlenecks

in the supply chain such as traffic congestions on routes, assignments and generation of

alternative routes, and supply chain optimization. In [58], a fleet tracking system for

commercial vehicles is described. The system can analyze messages sent from the vehicles

to identify unexpected incidents and discrepancies between actual and planned data, so that

remedial actions can be taken.

2.7.3 Shipment Monitoring

Shipment monitoring solutions for transportation systems allow monitoring the conditions

inside containers. For example, containers carrying fresh food produce can be monitored

to prevent spoilage of food. IoT based shipment monitoring systems use sensors, such as

temperature, pressure, humidity, for instance, to monitor the conditions inside the containers

and send the data to the cloud, where it can be analyzed to detect food spoilage. The analysis

and interpretation of data on the environmental conditions in the container and food truck

positioning can enable more effective routing decisions in real time. Therefore, it is possible

to take remedial measures such as - the food that has a limited time budget before it gets

rotten can be re-routed to a closer destinations, alerts can be raised to the driver and the

distributor about the transit conditions, such as container temperature exceeding the allowed

limit, humidity levels going out of the allowed limit, for instance, and corrective actions

ccan be taken before the food gets damaged. A cloud-based framework for real-time fresh

food supply tracking and monitoring was proposed in [61]. For fragile products, vibration

levels during shipments can be tracked using accelerometer and gyroscope sensors attached

to loT devices. In [59], a system for monitoring container integrity and operating conditions
Bahga & Madisetti, © 2015

— Page 48 —

27 Logistics 67
ee eae
CANT Wh
fj ‘Re i] CD
ew ~ |
re fo) o =,
Sane |
ow :
eS NS re
F Sah ee I cue
; pee. agate areas é
Figure 2.6: Applications of IoT for logistics
is described. The system monitors the vibration patterns of a container and its contents
to reyeal information related to its operating environment and integrity during transport,
handling and storage.
_ 2.7.4 Remote Vehicle Diagnostics
Remote vehicle diagnostic systems can detect faults in the vehicles or warn of impending
faults. These diagnostic systems use on-board loT devices for collecting data on vehicle
operation (such as speed, engine RPM, coolent temperature, fault code number) and status of
Various vehicle sub-systems. Such data can be captured by integrating on-board diagnostic
systems with IoT devices using protocols such as CAN bus. Modern commercial vehicles
__Stipport on-board diagnostic (OBD) standards such as OBD-II. OBD systems provide
_ real-time data on the status of vehicle sub-systems and diagnostic trouble codes which
allow rapidly identifying the faults in the vehicle. [oT based vehicle diagnostic systems can
send the vehicle data to centralized servers or the cloud where it can be analyzed to generate
alerts and suggest remedial actions, In [60], a real-time online vehicle diagnostics and early
fault estimation system is described. The system makes use of on-board vehicle diagnostics
ep
| __ Internet of Things - A Hands-On Approach
|
|

— Page 49 —

68 Domain Specific loTs
device and expert system to achieve real-time vehicle diagnostics and fault warning.
2.8 Agriculture
2.8.1 Smart Irrigation
‘Smart irrigation systems can improve crop yields while saving water. Smart irrigation
systems use IoT devices with soil moisture sensors to determine the amount of moisture in
the soil and release the flow of water through the irrigation pipes only when the moisture
levels go below a predefined threshold, Smart irrigation systems also collect moisture level _|
measurements on a server or in the cloud where the collected data can be analyzed to plan
watering schedules. Cultivar’s RainCloud [56] is a device for smart irrigation that uses water
valves, soil sensors and a WiFi enabled programmable computer. f
j ‘i SS
| Spe
, Vain nt \ :
arr g
Wi wee ee |
Af J XT EN bas Ae. th
My Vi By eee Aree | Yee cabal Nene,
# ee
Seg ae Seen ea eras
PRG, BPR go Oi eR aL UE fet meee Lea hth
Bates oP: OD dey ape eke aaa
RT TE ESSE oe arc el ee Nee i
SLO SOR ae IP SRM eS
Figure 2.7; Applications of IoT for agriculture
Bahga & Madisetti, © 2015.

— Page 50 —

2.9 Industry 69
28.2 Green House Control
Green houses are structures with glass or plastic roofs that provide conducive environment for
growth of plants. The climatological conditions inside a green house can be monitored and
controlled to provide the best conditions for growth of plants. The temperature, humidity, soil
moisture, light and carbon dioxide levels are monitored using sensors and the climatological
conditions are controlled automatically using actuation devices (such as valves for releasing
water and switches for controlling fans), IoT systems play an important role in green
house contro! and help in improving productivity. The data collected from various sensors
; is stored on centralized servers or in the cloud where analysis is performed to optimize
; the control strategies and also correlate the productivity with different control strategies.
In [57], the design of a wireless sensing and control system for precision green house
, management is described. The system uses wireless sensor network to monitor and control
the agricultural parameters like temperature and humidity in real time for better management
and maintenance of agricultural production.
2.9 Industry
2.9.1 Machine Diagnosis & Prognosis
Machine prognosis refers to predicting the performance of a machine by analyzing the
data on the current operating conditions and how much deviations exist from the normal
operating conditions. Machine diagnosis refers to determining the cause of a machine
fault. oT plays a major role in both prognosis and diagnosis of industrial machines.
Industrial machines have a large number of components that must function correctly for the
‘machine to perform its operations. Sensors in machines can monitor the operating conditions
‘such as (temperature and vibration levels). The sensor data measurements are done on
timescales of few milliseconds to few seconds, which leads to generation of massive amount
of data. IoT based systems integrated with cloud-based storage and analytics back-ends
can help in storage, collection and analysis of such massive scale machine sensor data.
‘A number of methods have been proposed for reliability analysis and fault prediction in
machines. Case-based reasoning (CBR) is a commonly used method that finds solutions to
new problems based on past experience. This past experience is organized and represented
as cases in a case-base. CBR is an effective technique for problem solving in the fields in
which it is hard to establish a quantitative mathematical model, such as machine diagnosis
and prognosis, Since for each machine, data from a very large number of sensors is collected,
using such high dimensional data for creation of case library reduces the case retrieval
efficiency. Therefore, data reduction and feature extraction methods are used to find the
representative set of features which have the same classification ability as the complete of
Internet of Things - A Hands-On Approach

— Page 51 —

70 Domain Specific loTs
ns me \ OS
gg f eS
ro. a
= == oie PRS SE
* Beri tlt
& SS ele |
nor A IL
oe Thali eT : Eee
eal ! aS
are aes 4, \
ate fad sg \
EPRI foc |
GRE | (1) \\ LAS |
oh Deo SUS Ae
oa Ee
[s: SL a a
Figure 2.8: Applications of loT for industry
features. A CBR based machine fault diagnosis and prognosis approach is described in [51].
A survey on recent trends in machine diagnosis and prognosis algorithms is presented in [65].
2.9.2 Indoor Air Quality Monitoring
Monitoring indoor air quality in factories is important for health and safety of the workers.
Harmful and toxic gases such as carbon monoxide (CO), nitrogen monoxide (NO), Nitrogen
Dioxide (NO), etc., can cause serious health problems. IoT based gas monitoring systems
can help in monitoring the indoor air quality using various gas sensors. The indoor air
quality can vary for different locations. Wireless sensor networks based IoT devices can
identify the hazardous zones, so that corrective measures can be taken to ensure proper
ventilation. In [66] a hybrid sensor system for indoor air quality monitoring is presented,
which contains both stationary sensors (for accurate readings and calibration) and mobile
Bahga & Madisetti, © 2015

— Page 52 —

240 Health & Lifestyle mA
cS (for coverage). In [67] a wireless solution for indoor air quality monitoring is
described that measures the environmental parameters like temperature, humidity, gaseous
pollutants ‘aerosol and particulate matter to determine the indoor air quality.
ce “&
= \e ae
His. tae) iY
a i ) [;
Bi: Fl
9 of y -
| epmcneh M Aers.
= A AS pa
 |=--o> <A) acoanon
yi: Sips ct a a Soe
Debris isha Pa GEOL oy me
b
: Figure 2.9: Applications of IoT for health
phe
2.10 Health & Lifestyle
2A 1 Health & Fitness Monitoring
‘Wearable IoT devices that allow non-invasive and continuous monitoring of physiological
pa eters can help in continuous health and fitness monitoring. These wearable devices
may can be in various forms such as belts and wrist-bands. The wearable devices form a
int 'of Things - A Hands-On Approach