JYOTHI ENGINEERING COLLEGE OF COMPUTING

Department of Mechatronics Engineering

Second Internal Examination — CIE

PEMRT525 — IoT and Applications

| Batch: MR 2K24 ¦ Semester: V | SET A |
| --- | --- |

| Modules Covered: 3 & 4 (Part A — Raspberry Pi) | Total Marks: 42 |
| --- | --- |

PART A — Answer ALL Questions  (4 × 3 = 12 Marks from Module 3 | 4 × 3 = 12 Marks from Module 4)

Module 3 — Unified Data Standards & Protocols

Q1.  What is IEEE 802.15.4? List the frequency bands used in its Physical Layer.   [CO3]  [K1]  [3 Marks]

Q2.  What are the three device types in a ZigBee network? State the role of each.   [CO3]  [K1]  [3 Marks]

Q3.  What is Spreading Factor in LoRaWAN? How does it affect data rate and communication range?   [CO3]  [K2]  [3 Marks]

Q4.  Differentiate between LTE-M and NB-IoT with respect to data rate and mobility support.   [CO3]  [K2]  [3 Marks]

Module 4 (Part A) — Raspberry Pi

Q5.  What is GPIO? State the voltage level of Raspberry Pi GPIO pins and why 5V must not be connected directly.   [CO4]  [K1]  [3 Marks]

Q6.  Why is a current-limiting resistor required when connecting an LED to a Raspberry Pi GPIO pin? Calculate a suitable resistor value.   [CO4]  [K2]  [3 Marks]

Q7.  List the four SPI signals (MOSI, MISO, SCLK, CE0) and state the GPIO pin number for each on Raspberry Pi.   [CO4]  [K1]  [3 Marks]

Q8.  How does I2C identify multiple devices connected on the same two-wire bus? State the two I2C pins on Raspberry Pi.   [CO4]  [K2]  [3 Marks]

PART B — Answer ALL Questions  (1 × 9 Marks from Module 3 | 1 × 9 Marks from Module 4)

Module 3

Q9.  Explain the IEEE 802.15.4 MAC (Media Access Control) layer. In your answer include: (i) the role of the MAC layer and what CSMA-CA is, (ii) the difference between beacon-enabled (slotted CSMA-CA) and non-beacon (unslotted CSMA-CA) channel access, (iii) any four real-world uses of IEEE 802.15.4, and (iv) the improvements introduced by 802.15.4e (industrial use) and 802.15.4g (smart utility networks).   [CO3]  [K2]  [9 Marks]

Module 4 (Part A)

Q10.  Explain the Serial (UART), SPI, and I2C interfaces of Raspberry Pi. For each interface describe: (i) the GPIO pin numbers used and their functions, (ii) number of wires and whether it is synchronous or asynchronous, and (iii) a simple Python code snippet using the appropriate library — pyserial for Serial, spidev for SPI, smbus2 for I2C.   [CO4]  [K3]  [9 Marks]

JYOTHI ENGINEERING COLLEGE OF COMPUTING

Department of Mechatronics Engineering

Second Internal Examination — CIE

PEMRT525 — IoT and Applications

| Batch: MR 2K24 ¦ Semester: V | SET B |
| --- | --- |

| Modules Covered: 3 & 4 (Part A — Raspberry Pi) | Total Marks: 42 |
| --- | --- |

PART A — Answer ALL Questions  (4 × 3 = 12 Marks from Module 3 | 4 × 3 = 12 Marks from Module 4)

Module 3 — Unified Data Standards & Protocols

Q1.  What is the role of the MAC layer in IEEE 802.15.4? Briefly explain the CSMA-CA channel access mechanism.   [CO3]  [K2]  [3 Marks]

Q2.  What are the network topologies supported by ZigBee? Name the standard on which ZigBee is based.   [CO3]  [K1]  [3 Marks]

Q3.  What does the LoRaWAN star-of-stars topology consist of? Name the four node types in the LoRaWAN network.   [CO3]  [K1]  [3 Marks]

Q4.  What is NB-IoT? State its three deployment modes and give two IoT applications where it is used.   [CO3]  [K2]  [3 Marks]

Module 4 (Part A) — Raspberry Pi

Q5.  Differentiate between BCM mode and BOARD mode in Raspberry Pi GPIO programming. Which mode is most commonly used and why?   [CO4]  [K2]  [3 Marks]

Q6.  Write a Python code snippet using RPi.GPIO to read a pushbutton switch connected to GPIO25. Print "Pressed" when the button is pushed.   [CO4]  [K3]  [3 Marks]

Q7.  State the Python library used for I2C communication on Raspberry Pi. Write two lines of code to read a byte from an I2C device at address 0x48.   [CO4]  [K3]  [3 Marks]

Q8.  Compare SPI and I2C interfaces of Raspberry Pi in terms of: number of wires, synchronous/asynchronous, number of devices supported, and Python library used.   [CO4]  [K2]  [3 Marks]

PART B — Answer ALL Questions  (1 × 9 Marks from Module 3 | 1 × 9 Marks from Module 4)

Module 3

Q9.  A smart agriculture company wants to place 200 soil moisture sensors across a 15 km² farm. Each sensor transmits small data packets once every 30 minutes and must run on battery power for at least 3 years without replacement. The farm has no cellular coverage. (i) Which IoT communication protocol would you recommend for this system? (ii) Justify your choice by comparing it with at least one alternative protocol. (iii) Explain the architecture of your chosen protocol — Physical Layer, MAC Layer, and network topology.   [CO3]  [K3]  [9 Marks]

Module 4 (Part A)

Q10.  How many ways can a communication device interface with Raspberry Pi? Name all the communication interfaces available and explain each one. For each interface include: (i) the interface name and the GPIO pins used with their pin numbers, (ii) how the interface works and how many devices it can support, and (iii) a Python code snippet (3–4 lines) using the correct library — pyserial, spidev, or smbus2.   [CO4]  [K3]  [9 Marks]

📌  Study Guide for Students

To prepare for both Set A and Set B, learn the following 10 answers only:

3-Mark Questions (learn all 8 unique answers):

1. IEEE 802.15.4 — purpose, frequency bands, MAC layer & CSMA-CA

2. ZigBee — three device types, roles, topologies, standard it is based on

3. LoRaWAN — Spreading Factor (SF), star-of-stars topology & node types

4. LTE-M vs NB-IoT — data rate, mobility, deployment modes

5. Raspberry Pi GPIO — voltage level, BCM vs BOARD mode

6. LED interfacing — resistor calculation, Python snippet to blink

7. SPI interface — four signals (MOSI/MISO/SCLK/CE0) and GPIO pins

8. I2C interface — address-based device identification, SDA/SCL pins, smbus2 read

9-Mark Questions (learn 2 answers — same answer covers both Set A and Set B):

1. Module 3 — Set A: IEEE 802.15.4 MAC Layer (CSMA-CA, uses, 802.15.4e/4g)  |  Set B: LoRaWAN scenario — answer = LoRaWAN: Physical Layer (CSS/SF), MAC Layer (Class A/B/C), star-of-stars topology

2. Module 4 — Set A: RPi Serial/SPI/I2C interfaces  |  Set B: "How many ways can a device interface with RPi?" — same answer: Serial (GPIO14/15, pyserial), SPI (GPIO8-11, spidev), I2C (GPIO2/3, smbus2)