APPENDIX 10 — TIPPING BUCKET RAIN GAUGE WITH ESP32 AND I2C OLED DISPLAY

AIM

To interface a tipping bucket rain gauge (reed switch output) with the ESP32 using an external interrupt to count bucket tips, calculate total rainfall and rainfall rate, and display the measurements on a 128×64 I2C OLED display (SSD1306).

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | Interrupt counting, display |
| 2 | Tipping Bucket Rain Gauge | Reed switch output, 0.2 mm/tip | 1 | Rainfall measurement |
| 3 | OLED Display | SSD1306, 128×64, I2C (0x3C) | 1 | Display rainfall data |
| 4 | Resistor (optional) | 10 kΩ pull-up (if no module pull-up) | 1 | Reed switch pull-up (ESP32 internal used) |
| 5 | Jumper Wires | Male-to-Female/Male | 1 set | Connections |
| 6 | USB Cable | Micro USB / Type-C | 1 | Power & programming |

WORKING PRINCIPLE OF TIPPING BUCKET RAIN GAUGE

A tipping bucket rain gauge collects rain water in a funnel which drains into a small see-saw bucket. When the bucket fills with a precise volume of water (equivalent to 0.2 mm of rain over the funnel area), it tips — emptying itself and bringing the other side into position. Each tip activates a reed switch (a magnetically operated switch) momentarily. Counting these switch closures gives the total rainfall.

| Parameter | Description |
| --- | --- |
| Reed Switch Output | Momentary short circuit (normally open) — each tip closes the switch briefly |
| Typical Sensitivity | 0.2 mm or 0.5 mm per tip — check the gauge datasheet or calibration label |
| Pulse Duration | ~10–100 ms per tip (varies with tip speed and magnet strength) |
| Contact Bounce | Reed switches bounce — must debounce (50 ms delay in ISR) |
| ESP32 Input | GPIO configured as INPUT_PULLUP — switch pulls pin to GND on each tip |
| Interrupt Type | FALLING edge — triggered when reed switch closes (HIGH → LOW) |

CIRCUIT DIAGRAM

Tipping Bucket ESP32

Reed Switch

Terminal 1 ─────────────────── GPIO4 (INPUT_PULLUP)

Terminal 2 ─────────────────── GND

I2C OLED SSD1306 (128×64)

VCC ────────────────────────── 3.3V

GND ────────────────────────── GND

SDA ────────────────────────── GPIO21

SCL ────────────────────────── GPIO22

Signal behaviour:

Resting (no rain) : GPIO4 = HIGH (internal pull-up to 3.3V)

Each bucket tip : GPIO4 = LOW (reed switch pulls to GND) → FALLING edge → ISR

Figure A10.1: Tipping bucket reed switch → GPIO4; OLED SSD1306 → GPIO21/GPIO22

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Reed switch terminal 1 | GPIO4 | INPUT_PULLUP — FALLING interrupt attached |
| Reed switch terminal 2 | GND | Completes circuit when switch closes |
| OLED VCC | 3.3V | SSD1306 runs on 3.3V (some modules have onboard regulator for 5V too) |
| OLED GND | GND | Common ground |
| OLED SDA | GPIO21 | ESP32 default I2C data |
| OLED SCL | GPIO22 | ESP32 default I2C clock |

THEORY

Rainfall Calculation

| Parameter | Formula |
| --- | --- |
| Total Rainfall | Total (mm) = tipCount × MM_PER_TIP |
| Rainfall Rate (per 60s window) | Rate (mm/hr) = (tipsInPeriod × MM_PER_TIP) × 60 |
| Live Rate (extrapolated) | Rate (mm/hr) = (tipsInPeriod × MM_PER_TIP) × (3600000 / elapsed_ms) |
| MM_PER_TIP | 0.2 mm (standard) — check gauge calibration label and adjust if different |

ESP32 External Interrupt

attachInterrupt(digitalPinToInterrupt(REED_PIN), onTip, FALLING) registers the onTip() function to execute automatically each time GPIO4 transitions from HIGH to LOW. The ISR is marked IRAM_ATTR to store it in IRAM (Internal RAM) so it executes even during flash operations. Since ISRs must be fast, only the counter is incremented and a debounce timestamp is checked — no Serial prints or delays inside the ISR.

SSD1306 OLED Display

| Property | Value |
| --- | --- |
| Resolution | 128 × 64 pixels |
| Interface | I2C (default address 0x3C; some modules use 0x3D) |
| Library | Adafruit SSD1306 + Adafruit GFX |
| Text sizes | setTextSize(1) = 6×8px per char; setTextSize(2) = 12×16px |
| Drawing | drawLine(), drawRect(), fillRect(), drawCircle() — from Adafruit GFX |

LIBRARY INSTALLATION (Arduino IDE)

1. Open Sketch → Include Library → Manage Libraries.

2. Search "Adafruit SSD1306" → install. Also install "Adafruit GFX Library" when prompted.

3. Both libraries are required — SSD1306 uses GFX for drawing primitives.

PROGRAM CODE (ESP32 — Arduino IDE)

/* ============================================================

Appendix 10 — Tipping Bucket Rain Gauge + I2C OLED (ESP32)

Reed switch: GPIO4 (INPUT_PULLUP) — other terminal GND

OLED SSD1306 128x64: SDA→GPIO21, SCL→GPIO22, VCC→3.3V

Each tip = 0.2 mm rainfall (adjust MM_PER_TIP for your gauge)

Interrupt on FALLING edge counts every tip

============================================================ */

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_W 128

#define SCREEN_H 64

#define OLED_ADDR 0x3C // common SSD1306 I2C address

#define REED_PIN 4 // reed switch — INPUT_PULLUP

#define MM_PER_TIP 0.2f // mm of rain per bucket tip (calibrate)

Adafruit_SSD1306 display(SCREEN_W, SCREEN_H, &Wire, -1);

volatile uint32_t tipCount = 0; // incremented in ISR

uint32_t lastTipTime = 0; // for debounce

uint32_t periodStart = 0; // start of 1-min rate window

uint32_t tipsInPeriod = 0; // tips in current minute

// ── Interrupt Service Routine — called on each reed switch close ──

void IRAM_ATTR onTip() {

uint32_t now = millis();

if (now - lastTipTime > 50) { // 50 ms debounce

tipCount++;

tipsInPeriod++;

lastTipTime = now;

}

}

void updateDisplay(float totalMm, float rateMmHr) {

display.clearDisplay();

// ── Title bar ──

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(20, 0);

display.println("RAIN GAUGE");

display.drawLine(0, 10, 127, 10, SSD1306_WHITE);

// ── Total rainfall ──

display.setCursor(0, 15);

display.setTextSize(1);

display.print("Total Rain:");

display.setCursor(0, 25);

display.setTextSize(2);

display.print(totalMm, 1);

display.print(" mm");

// ── Rate ──

display.setCursor(0, 46);

display.setTextSize(1);

display.print("Rate: ");

display.print(rateMmHr, 1);

display.print(" mm/hr");

// ── Tip count ──

display.setCursor(0, 56);

display.print("Tips: ");

display.print(tipCount);

display.display();

}

void setup() {

Serial.begin(115200);

// OLED init

if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("SSD1306 not found — check wiring/address");

while (true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(10, 25);

display.println("Rain Gauge Ready");

display.display();

delay(1500);

// Reed switch interrupt

pinMode(REED_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(REED_PIN), onTip, FALLING);

periodStart = millis();

Serial.println("Tipping Bucket Rain Gauge Started");

Serial.println("Tips ¦ Total(mm) ¦ Rate(mm/hr)");

}

void loop() {

float totalMm = tipCount * MM_PER_TIP;

// ── Compute rainfall rate every 60 seconds ──

uint32_t elapsed = millis() - periodStart;

float rateMmHr = 0.0;

if (elapsed >= 60000) {

rateMmHr = (tipsInPeriod * MM_PER_TIP) * 60.0; // mm/hr

tipsInPeriod = 0;

periodStart = millis();

} else {

// Live estimate: extrapolate current period to full hour

if (elapsed > 0)

rateMmHr = (tipsInPeriod * MM_PER_TIP) * (3600000.0 / elapsed);

}

updateDisplay(totalMm, rateMmHr);

Serial.print(tipCount);

Serial.print(" ¦ ");

Serial.print(totalMm, 2);

Serial.print(" mm ¦ ");

Serial.print(rateMmHr, 1);

Serial.println(" mm/hr");

delay(1000); // update display every second

}

EXPECTED OLED DISPLAY

+────────────────────────────────+

¦ RAIN GAUGE ¦

¦────────────────────────────────¦

¦ Total Rain: ¦

¦ 2.0 mm ¦

¦ ¦

¦ Rate: 12.0 mm/hr ¦

¦ Tips: 10 ¦

+────────────────────────────────+

Figure A10.2: SSD1306 OLED display layout — title bar, total rainfall (large), rate and tip count below

EXPECTED SERIAL MONITOR OUTPUT (115200 baud)

Tipping Bucket Rain Gauge Started

Tips ¦ Total(mm) ¦ Rate(mm/hr)

0 ¦ 0.00 mm ¦ 0.0 mm/hr

1 ¦ 0.20 mm ¦ 12.0 mm/hr

2 ¦ 0.40 mm ¦ 12.0 mm/hr

5 ¦ 1.00 mm ¦ 6.0 mm/hr

10 ¦ 2.00 mm ¦ 4.8 mm/hr

OBSERVATION TABLE

| Time (min) | Tip Count | Total Rainfall (mm) | Rate (mm/hr) | Remarks |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Table A10.1: Record tip count, total rainfall, and rate at 1-minute intervals

CALIBRATION NOTE

To verify MM_PER_TIP for your specific gauge:

1. Pour a known volume of water (e.g., 50 mL) slowly into the funnel.

2. Count the number of tips displayed.

3. Calculate: MM_PER_TIP = (Volume_mL / Funnel_Area_cm²) / tipCount

4. Funnel area (cm²) = π × r², where r = radius of the funnel opening in cm.

5. Update MM_PER_TIP in the code with the calibrated value and re-upload.

For a standard WS-3000 or Davis tipping bucket gauge, MM_PER_TIP = 0.2 mm. For larger gauges it may be 0.5 mm or 1.0 mm.

PROCEDURE

1. Connect the tipping bucket reed switch: Terminal 1 → GPIO4, Terminal 2 → GND.

2. Connect the OLED SSD1306: VCC → 3.3V, GND → GND, SDA → GPIO21, SCL → GPIO22.

3. Install Adafruit SSD1306 and Adafruit GFX libraries in Arduino IDE.

4. Verify the OLED I2C address (typically 0x3C). If OLED does not start, run an I2C scanner and update OLED_ADDR.

5. Check the gauge datasheet for MM_PER_TIP. Update the value in the code if needed.

6. Upload the code. Open Serial Monitor at 115200 baud.

7. Observe OLED — should show "Rain Gauge Ready", then switch to the measurement screen.

8. Simulate rain by manually tipping the bucket by hand or pouring a small amount of water.

9. Observe tip count, total rainfall, and rate update on both OLED and Serial Monitor.

10. Record values at 1-minute intervals in the observation table.

11. Verify: Total (mm) = tip count × 0.2. Compare with observation.

RESULT

RESULT The tipping bucket rain gauge was successfully interfaced with the ESP32. Each bucket tip closed the reed switch, generating a FALLING edge on GPIO4 which triggered the IRAM_ATTR ISR via attachInterrupt(). A 50ms software debounce prevented false counts. Total rainfall was calculated as tipCount × 0.2 mm and displayed on the SSD1306 128×64 I2C OLED (GPIO21/SDA, GPIO22/SCL) along with the live rainfall rate in mm/hr, updated every second. Serial Monitor provided a concurrent log of all measurements.