APPENDIX 11 — IR SENSOR / PIR SENSOR AND RELAY MODULE
INTERFACING WITH RASPBERRY PI

AIM

To interface an IR obstacle sensor and a PIR motion sensor with the Raspberry Pi GPIO, and control a relay module based on the sensor output using Python (RPi.GPIO library).

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Single Board Computer | Raspberry Pi (any 40-pin model) | 1 | Main controller |
| 2 | IR Obstacle Sensor | FC-51 / LM393 IR module | 1 | Detect object / obstacle |
| 3 | PIR Motion Sensor | HC-SR501 | 1 | Detect human presence |
| 4 | Relay Module | 5V SPDT with optocoupler | 1 | Switch load |
| 5 | LED + 470 Ω resistor | Any colour | 1 | Load for relay (test) |
| 6 | Jumper Wires | Female-to-Female | 1 set | GPIO connections |

PART A — IR OBSTACLE SENSOR WITH RELAY

Working Principle

An IR obstacle sensor module (FC-51 / LM393) has an IR LED and an IR photodiode. The IR LED continuously emits infrared light. When an object comes close, the reflected IR hits the photodiode — the LM393 comparator outputs LOW (active-LOW). When the path is clear, the output is HIGH.

| Condition | IR Module OUT | GPIO17 reads | Relay action |
| --- | --- | --- | --- |
| Object / obstacle in front | LOW (0V) | GPIO.LOW | ON — load activated |
| Path clear | HIGH (3.3V) | GPIO.HIGH | OFF — load deactivated |

Circuit — IR Sensor and Relay

IR Sensor Module Raspberry Pi (40-pin)

VCC ────────────────── Pin 1 (3.3V)

GND ────────────────── Pin 6 (GND)

OUT ────────────────── Pin 11 (GPIO17, BCM)

Relay Module

VCC ────────────────── Pin 4 (5V)

GND ────────────────── Pin 9 (GND)

IN ────────────────── Pin 13 (GPIO27, BCM)

Relay contacts (test load):

5V ─── COM ─── NO ─── LED (470Ω) ─── GND

Figure A11.1: IR sensor → GPIO17; Relay module → GPIO27; LED as test load on relay NO contact

| Pin | From | To | Notes |
| --- | --- | --- | --- |
| IR VCC | IR Sensor | RPi Pin 1 (3.3V) | FC-51 works on 3.3V and 5V |
| IR GND | IR Sensor | RPi Pin 6 (GND) | Common ground |
| IR OUT | IR Sensor | RPi Pin 11 (GPIO17) | Active-LOW — LOW when obstacle |
| Relay VCC | Relay module | RPi Pin 4 (5V) | Relay coil needs 5V |
| Relay GND | Relay module | RPi Pin 9 (GND) | Common ground |
| Relay IN | Relay module | RPi Pin 13 (GPIO27) | 3.3V from RPi — most modules accept this |

Program Code — IR Sensor + Relay (Python 3)

#!/usr/bin/env python3

# IR Obstacle Sensor + Relay — Raspberry Pi

# IR sensor OUT → GPIO17 (BCM) ¦ Relay IN → GPIO27 (BCM)

# IR module output: LOW = obstacle detected, HIGH = clear (active-LOW)

import RPi.GPIO as GPIO

import time

IR_PIN = 17 # IR sensor OUT pin

RELAY_PIN = 27 # Relay module IN pin

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(IR_PIN, GPIO.IN)

GPIO.setup(RELAY_PIN, GPIO.OUT)

GPIO.output(RELAY_PIN, GPIO.HIGH) # relay OFF (active-LOW module)

print("IR Sensor + Relay Test — Press Ctrl+C to stop")

try:

while True:

if GPIO.input(IR_PIN) == GPIO.LOW: # obstacle detected

GPIO.output(RELAY_PIN, GPIO.LOW) # relay ON

print("Obstacle detected — Relay ON")

else: # path clear

GPIO.output(RELAY_PIN, GPIO.HIGH) # relay OFF

print("Clear — Relay OFF")

time.sleep(0.5)

except KeyboardInterrupt:

print("Stopped.")

finally:

GPIO.output(RELAY_PIN, GPIO.HIGH) # relay OFF on exit

GPIO.cleanup()

Expected Output — IR Sensor

IR Sensor + Relay Test — Press Ctrl+C to stop

Clear — Relay OFF

Clear — Relay OFF

Obstacle detected — Relay ON

Obstacle detected — Relay ON

Clear — Relay OFF

PART B — PIR MOTION SENSOR WITH RELAY

Working Principle

The HC-SR501 PIR sensor detects infrared radiation emitted by warm bodies. When motion is detected, the output pin goes HIGH (3.3V). When no motion, it stays LOW. The sensitivity (range) and time delay can be adjusted using two potentiometers on the module.

| Condition | PIR OUT | GPIO17 reads | Relay action |
| --- | --- | --- | --- |
| Motion / person detected | HIGH (3.3V) | GPIO.HIGH | ON — load activated |
| No motion | LOW (0V) | GPIO.LOW | OFF — load deactivated |

HC-SR501 Adjustment

| Potentiometer | Function | Direction to Increase |
| --- | --- | --- |
| Sx (left) | Detection range: 3 m – 7 m | Clockwise |
| Tx (right) | Output pulse width: 5 s – 200 s | Clockwise |
| Jumper H/L | H = repeat trigger (stays HIGH while motion present) L = single pulse per detection | Set to H for continuous control |

Circuit — PIR Sensor and Relay

HC-SR501 PIR Raspberry Pi (40-pin)

VCC ────────────────── Pin 2 (5V) ← PIR needs 5V

GND ────────────────── Pin 6 (GND)

OUT ────────────────── Pin 11 (GPIO17, BCM)

Relay Module

VCC ────────────────── Pin 4 (5V)

GND ────────────────── Pin 9 (GND)

IN ────────────────── Pin 13 (GPIO27, BCM)

Relay contacts (test load):

5V ─── COM ─── NO ─── LED (470Ω) ─── GND

Note: PIR VCC must be 5V (Pin 2 or Pin 4). PIR OUT is 3.3V and is safe to connect directly to GPIO17.

Program Code — PIR Sensor + Relay (Python 3)

#!/usr/bin/env python3

# PIR Motion Sensor + Relay — Raspberry Pi

# PIR OUT → GPIO17 (BCM) ¦ Relay IN → GPIO27 (BCM)

# PIR output: HIGH = motion detected, LOW = no motion (active-HIGH)

import RPi.GPIO as GPIO

import time

PIR_PIN = 17 # HC-SR501 OUT pin

RELAY_PIN = 27 # Relay module IN pin

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(PIR_PIN, GPIO.IN)

GPIO.setup(RELAY_PIN, GPIO.OUT)

GPIO.output(RELAY_PIN, GPIO.HIGH) # relay OFF at start

print("Waiting 30s for PIR warm-up...")

time.sleep(30)

print("PIR Sensor + Relay Ready — Press Ctrl+C to stop")

try:

while True:

if GPIO.input(PIR_PIN) == GPIO.HIGH: # motion detected

GPIO.output(RELAY_PIN, GPIO.LOW) # relay ON

print("Motion detected — Relay ON")

else: # no motion

GPIO.output(RELAY_PIN, GPIO.HIGH) # relay OFF

print("No motion — Relay OFF")

time.sleep(0.5)

except KeyboardInterrupt:

print("Stopped.")

finally:

GPIO.output(RELAY_PIN, GPIO.HIGH)

GPIO.cleanup()

Expected Output — PIR Sensor

Waiting 30s for PIR warm-up...

PIR Sensor + Relay Ready — Press Ctrl+C to stop

No motion — Relay OFF

No motion — Relay OFF

Motion detected — Relay ON

Motion detected — Relay ON

Motion detected — Relay ON

No motion — Relay OFF

RELAY MODULE — ACTIVE-LOW vs ACTIVE-HIGH

Most 5V relay modules with optocouplers are active-LOW — the relay energises when IN pin is LOW and de-energises when IN is HIGH. This is the opposite of the sensor logic.

| Relay Type | Relay ON when IN = | Relay OFF when IN = | GPIO.output to turn ON |
| --- | --- | --- | --- |
| Active-LOW (most modules) | LOW (0V) | HIGH (3.3V) | GPIO.LOW |
| Active-HIGH | HIGH (3.3V) | LOW (0V) | GPIO.HIGH |

Both codes above assume active-LOW relay module. If your relay turns ON at startup and turns OFF when triggered, swap GPIO.LOW and GPIO.HIGH for RELAY_PIN in the code.

RASPBERRY PI GPIO PIN MAP (REFERENCE)

| Physical Pin | BCM GPIO | Function | Used For |
| --- | --- | --- | --- |
| Pin 1 | — | 3.3V Power | IR sensor VCC (3.3V) |
| Pin 2 | — | 5V Power | PIR sensor VCC (5V) |
| Pin 4 | — | 5V Power | Relay module VCC |
| Pin 6 | — | GND | Common ground |
| Pin 9 | — | GND | Relay GND |
| Pin 11 | GPIO17 | Digital Input | Sensor OUT (IR / PIR) |
| Pin 13 | GPIO27 | Digital Output | Relay module IN |

PROCEDURE

1. Connect the IR sensor: VCC → Pin 1 (3.3V), GND → Pin 6, OUT → Pin 11 (GPIO17).

2. Connect the relay module: VCC → Pin 4 (5V), GND → Pin 9, IN → Pin 13 (GPIO27).

3. Connect the test LED with 470Ω resistor between relay NO contact and 5V / GND.

4. Open Terminal on Raspberry Pi. Create the script: nano ir_relay.py

5. Paste the IR sensor code. Save (Ctrl+X → Y → Enter).

6. Run: python3 ir_relay.py

7. Bring an object close to the IR sensor — LED and relay should activate.

8. Move the object away — relay should deactivate.

9. Record observations in the table.

10. Stop the script (Ctrl+C). Create a new file: nano pir_relay.py

11. Paste the PIR sensor code. Save and run: python3 pir_relay.py

12. Wait 30 seconds for the PIR to warm up.

13. Walk in front of the PIR sensor — relay and LED activate.

14. Stand still or move away — relay deactivates after the Tx delay.

15. Compare the behaviour of the IR sensor (detects object) vs PIR sensor (detects motion/heat).

OBSERVATION TABLE

| Sl. | Sensor Used | Condition | Sensor OUT | Relay State | LED |
| --- | --- | --- | --- | --- | --- |
| 1 | IR | Obstacle in front | LOW |  |  |
| 2 | IR | Path clear | HIGH |  |  |
| 3 | PIR | Motion detected | HIGH |  |  |
| 4 | PIR | No motion | LOW |  |  |

RESULT

RESULT The IR obstacle sensor (FC-51) and PIR motion sensor (HC-SR501) were successfully interfaced with the Raspberry Pi GPIO using Python 3 and the RPi.GPIO library. The IR sensor output (active-LOW) on GPIO17 and the relay module IN (active-LOW) on GPIO27 were correctly read and controlled respectively. When an obstacle was detected by the IR sensor (OUT = LOW), the relay was energised (IN = LOW) and the connected LED load turned ON. Similarly, when the PIR sensor detected motion (OUT = HIGH), the relay was energised accordingly. GPIO.cleanup() ensured safe shutdown of all pins after the script exited.