APPENDIX 12 — SENDING DATA TO FIREBASE REALTIME DATABASE (ESP32)

AIM

To send sensor data (counter, uptime, temperature, humidity) from the ESP32 to the Google Firebase Realtime Database over WiFi and verify the data in the Firebase console in real time.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | WiFi + Firebase client |
| 2 | DHT11 Sensor (Part B) | DHT11 module | 1 | Real sensor data to upload |
| 3 | 10 kΩ Resistor (Part B) | Pull-up for DHT DATA | 1 | DHT signal stability |
| 4 | USB Cable | Micro USB / Type-C | 1 | Power & programming |
| 5 | Smartphone / PC | With internet browser | 1 | Firebase console verification |

INTRODUCTION TO FIREBASE REALTIME DATABASE

Firebase Realtime Database is a cloud-hosted NoSQL database by Google. Data is stored as JSON and synchronised to all connected clients in real time. For IoT projects, the ESP32 sends data as key-value pairs to a database URL. The data can be viewed, queried, and used by any web or mobile application connected to the same Firebase project.

| Feature | Details |
| --- | --- |
| Database type | NoSQL — data stored as a JSON tree |
| Protocol | HTTPS REST API (library handles this internally) |
| Free tier | 1 GB storage, 10 GB/month download, 100 simultaneous connections |
| Latency | Real-time sync — data appears in console within 1–2 seconds |
| Alternatives | ThingSpeak (time-series), Adafruit IO, AWS IoT, Google Sheets via Apps Script |

FIREBASE PROJECT SETUP

Step 1 — Create Firebase Project

1. Open https://console.firebase.google.com and sign in with your Google account.

2. Click "Add project" → enter a project name (e.g., ESP32-IoT-Lab) → Continue.

3. Disable Google Analytics (not needed) → Create project → Continue.

Step 2 — Enable Realtime Database

1. In the left sidebar click "Build" → "Realtime Database".

2. Click "Create Database" → choose a location (e.g., us-central1) → Next.

3. Select "Start in test mode" (allows read/write without authentication for 30 days) → Enable.

4. Note the database URL shown at the top — it looks like: your-project-default-rtdb.firebaseio.com

Step 3 — Get Database Secret Key

1. Click the gear icon (Project Settings) → Service Accounts tab.

2. Scroll down to "Database secrets" → click "Show" → copy the secret key.

3. This is your FIREBASE_AUTH value in the code.

4. (Newer Firebase projects use authentication tokens — if secret is not available, see the note below.)

Note: For newer Firebase projects, use the Legacy Token method. Go to Project Settings → General → copy the Web API Key. Then use Firebase.begin(host, apiKey) and set database rules to allow public read/write temporarily for testing.

Step 4 — Database Security Rules (Test Mode)

{

"rules": {

".read": true,

".write": true

}

}

These rules allow anyone to read and write. For production/real deployment, add authentication. Change rules at: Realtime Database → Rules tab.

LIBRARY INSTALLATION (Arduino IDE)

1. Open Arduino IDE → Sketch → Include Library → Manage Libraries.

2. Search "Firebase ESP32 Client" → install by Mobizt.

3. Also ensure "ArduinoJson" is installed (search "ArduinoJson" by Benoit Blanchon).

4. For Part B: also install "DHT sensor library" by Adafruit.

DATA STRUCTURE IN FIREBASE

Data is organised as a JSON tree. Each Firebase.setXxx() call writes to a path inside the database:

{

"esp32": {

"count" : 12,

"uptime" : 60.5,

"status" : "online"

},

"dht": {

"temperature" : 28.5,

"humidity" : 62.3,

"status" : "online"

}

}

Figure A12.1: Firebase Realtime Database JSON structure after the ESP32 sends data

| Function | Data Type | Example |
| --- | --- | --- |
| Firebase.setInt(fbdo, path, value) | Integer | Firebase.setInt(fbdo, "/esp32/count", 5) |
| Firebase.setFloat(fbdo, path, value) | Float | Firebase.setFloat(fbdo, "/dht/temp", 28.5) |
| Firebase.setString(fbdo, path, value) | String | Firebase.setString(fbdo, "/status", "ok") |
| Firebase.setBool(fbdo, path, value) | Boolean | Firebase.setBool(fbdo, "/relay", true) |
| Firebase.getFloat(fbdo, path) | Read float | float t = fbdo.floatData() |
| Firebase.getString(fbdo, path) | Read string | String s = fbdo.stringData() |

PART A — PROGRAM CODE: BASIC (NO SENSOR)

This code requires no sensor. It sends a counter, uptime in seconds, and a status string to Firebase every 5 seconds. Use this to verify Firebase connectivity first.

/* ============================================================

Appendix 12 — Send Data to Firebase Realtime Database

Part A: Basic (no sensor) — sends counter + uptime

Library: FirebaseESP32 by Mobizt

(Arduino Library Manager → search "Firebase ESP32 Client")

============================================================ */

#include <WiFi.h>

#include <FirebaseESP32.h>

// ── Firebase credentials ─────────────────────────────────────

#define FIREBASE_HOST "your-project-default-rtdb.firebaseio.com"

#define FIREBASE_AUTH "YourDatabaseSecretKeyHere"

// ── WiFi credentials ─────────────────────────────────────────

#define WIFI_SSID "Your_WiFi_SSID"

#define WIFI_PASS "Your_WiFi_Password"

FirebaseData fbdo; // Firebase data object

int count = 0;

unsigned long sendTime = 0;

void setup() {

Serial.begin(115200);

// Connect to WiFi

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500); Serial.print(".");

}

Serial.println("\nWiFi Connected: " + WiFi.localIP().toString());

// Connect to Firebase

Firebase.begin(FIREBASE_HOST, FIREBASE_AUTH);

Firebase.reconnectWiFi(true);

Serial.println("Firebase Connected.");

}

void loop() {

if (millis() - sendTime >= 5000) { // send every 5 seconds

sendTime = millis();

count++;

float uptime = millis() / 1000.0;

// ── Write to Firebase paths ────────────────────────────

if (Firebase.setInt(fbdo, "/esp32/count", count))

Serial.print("count OK ");

else Serial.print("count FAIL: " + fbdo.errorReason() + " ");

if (Firebase.setFloat(fbdo, "/esp32/uptime", uptime))

Serial.print("uptime OK ");

else Serial.print("uptime FAIL ");

if (Firebase.setString(fbdo, "/esp32/status", "online"))

Serial.println("status OK");

else Serial.println("status FAIL");

Serial.print("Sent → count: "); Serial.print(count);

Serial.print(" uptime: "); Serial.print(uptime, 1);

Serial.println(" s");

}

}

PART B — PROGRAM CODE: WITH DHT11 SENSOR

Connect DHT11 DATA to GPIO4 (10kΩ pull-up to 3.3V). This code sends real temperature and humidity to Firebase.

/* Part B — Send DHT11 Temperature + Humidity to Firebase */

#include <WiFi.h>

#include <FirebaseESP32.h>

#include <DHT.h>

#define FIREBASE_HOST "your-project-default-rtdb.firebaseio.com"

#define FIREBASE_AUTH "YourDatabaseSecretKeyHere"

#define WIFI_SSID "Your_WiFi_SSID"

#define WIFI_PASS "Your_WiFi_Password"

#define DHTPIN 4

#define DHTTYPE DHT11

FirebaseData fbdo;

DHT dht(DHTPIN, DHTTYPE);

unsigned long sendTime = 0;

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(WIFI_SSID, WIFI_PASS);

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println("\nWiFi OK");

Firebase.begin(FIREBASE_HOST, FIREBASE_AUTH);

Firebase.reconnectWiFi(true);

Serial.println("Firebase OK");

}

void loop() {

if (millis() - sendTime >= 5000) {

sendTime = millis();

float temp = dht.readTemperature();

float hum = dht.readHumidity();

if (isnan(temp) ¦¦ isnan(hum)) {

Serial.println("DHT read failed"); return;

}

Firebase.setFloat(fbdo, "/dht/temperature", temp);

Firebase.setFloat(fbdo, "/dht/humidity", hum);

Firebase.setString(fbdo,"/dht/status", "online");

Serial.print("Temp: "); Serial.print(temp);

Serial.print(" C Hum: "); Serial.print(hum);

Serial.println(" % → Firebase sent");

}

}

EXPECTED SERIAL MONITOR OUTPUT (115200 baud)

Connecting to WiFi.....

WiFi Connected: 192.168.1.105

Firebase Connected.

count OK uptime OK status OK

Sent → count: 1 uptime: 5.0 s

count OK uptime OK status OK

Sent → count: 2 uptime: 10.0 s

count OK uptime OK status OK

Sent → count: 3 uptime: 15.1 s

VERIFYING DATA IN FIREBASE CONSOLE

1. Open https://console.firebase.google.com → your project → Realtime Database.

2. The JSON tree updates live as the ESP32 sends data — you will see the values change every 5 seconds.

3. Expand the tree: click the arrow next to the database root → esp32 → count / uptime / status.

4. Each value refreshes automatically — no need to reload the page.

Firebase Console → Realtime Database view:

your-project-default-rtdb

└─ esp32

├─ count : 7

├─ status : "online"

└─ uptime : 35.1

└─ dht

├─ humidity : 62.3

├─ status : "online"

└─ temperature : 28.5

OBSERVATION TABLE

| Sl. | Time (s) | count (Firebase) | uptime (Firebase) | Temp (°C) | Hum (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 1 |  |  |  |
| 2 | 10 | 2 |  |  |  |
| 3 | 15 | 3 |  |  |  |
| 4 | 20 | 4 |  |  |  |
| 5 | 25 | 5 |  |  |  |

Table A12.1: Record values observed in Firebase console and Serial Monitor at 5-second intervals

COMPARISON OF IoT DATABASE OPTIONS

| Platform | Protocol | Free Tier | Best For | ESP32 Library |
| --- | --- | --- | --- | --- |
| Firebase RTDB | HTTPS / WebSocket | 1 GB, 100 conn. | Real-time dashboards | FirebaseESP32 (Mobizt) |
| ThingSpeak | HTTPS REST | 3M msgs/year | Time-series charts | ThingSpeak library |
| Adafruit IO | MQTT / HTTPS | 5 feeds, 30 msg/min | Simple dashboards | Adafruit MQTT library |
| AWS IoT Core | MQTT / HTTPS | Free tier 12 months | Production / enterprise | MQTT + PubSubClient |
| Google Sheets | HTTPS (Apps Script) | Unlimited (free) | Logging / reports | WiFiClientSecure |

PROCEDURE

1. Create a Firebase project at console.firebase.google.com and enable Realtime Database in test mode.

2. Note the database URL (e.g., your-project-default-rtdb.firebaseio.com) and database secret key.

3. Install FirebaseESP32 by Mobizt and ArduinoJson in Arduino IDE Library Manager.

4. Open the Part A code. Replace FIREBASE_HOST, FIREBASE_AUTH, WIFI_SSID, and WIFI_PASS with your credentials.

5. Upload the code. Open Serial Monitor at 115200 baud.

6. Confirm "WiFi Connected" and "Firebase Connected" messages appear.

7. Observe "count OK  uptime OK  status OK" repeating every 5 seconds.

8. Open Firebase console → Realtime Database — verify values updating in real time.

9. Record values in the observation table at each 5-second interval.

10. For Part B: connect DHT11 to GPIO4, replace code with Part B version, re-upload, and verify temperature/humidity in Firebase.

11. Try modifying a value directly in the Firebase console (click on a value and edit) — the ESP32 code can read it back using Firebase.getFloat() or Firebase.getString().

RESULT

RESULT The ESP32 was successfully connected to the Google Firebase Realtime Database over WiFi using the FirebaseESP32 library by Mobizt. Data values (counter, uptime, and status string) were written to specific JSON paths in the database using Firebase.setInt(), Firebase.setFloat(), and Firebase.setString() functions every 5 seconds. The data was verified in real time in the Firebase console, where values updated automatically without page refresh. In Part B, live DHT11 temperature and humidity readings were also uploaded and verified in the database, demonstrating a complete IoT cloud data logging system.