APPENDIX 2 — RELAY CONTROL VIA BLYNK IoT (ESP32 + SL100)

AIM

To control a relay using the Blynk IoT platform through an ESP32 microcontroller, with an SL100 NPN transistor as the relay driver, allowing ON/OFF control of a load from a mobile application over the internet.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | WiFi + Blynk control |
| 2 | Relay | 5V SPDT Relay | 1 | Switch load circuit |
| 3 | Transistor | SL100 NPN | 1 | Drive relay coil |
| 4 | Resistor | 1 kΩ, 1/4 W | 1 | Base current limiting |
| 5 | Diode | 1N4007 | 1 | Flyback protection across relay coil |
| 6 | LED + 470 Ω | Any colour | 1 | Load indicator (optional) |
| 7 | Breadboard + Jumpers | — | 1 set | Connections |
| 8 | Smartphone | Android / iOS with Blynk | 1 | Remote control |

CIRCUIT DIAGRAM

ESP32 GPIO2 ──[1kΩ]──→ Base of SL100

Emitter → GND

Collector → Relay Coil (−)

5V ──────────────────→ Relay Coil (+)

Relay COM → Load Supply

Relay NO → Load

Load other terminal → Supply return

GPIO2 HIGH → SL100 ON → Relay energised → Load ON

GPIO2 LOW → SL100 OFF → Relay de-energised → Load OFF

Figure A2.1: ESP32 GPIO2 → 1kΩ → SL100 base → relay coil → load

| Connection | From | To | Notes |
| --- | --- | --- | --- |
| 1kΩ resistor | ESP32 GPIO2 | SL100 Base | Limits base current |
| SL100 Emitter | — | GND | Common ground |
| SL100 Collector | — | Relay Coil (−) | Drives relay |
| Relay Coil (+) | — | 5V (VIN) | Relay supply |
| 1N4007 diode | Relay Coil (−) | Relay Coil (+) | Cathode to + : flyback protection |
| Relay NO | — | Load | Normally Open contact |

THEORY

The SL100 is an NPN silicon transistor. A small base current (from GPIO2 through 1kΩ) drives a larger collector current through the relay coil, energising the relay. When GPIO2 is LOW, the transistor is cut off and the relay is de-energised.

Blynk IoT Platform:

| Virtual Pin | Widget | Action |
| --- | --- | --- |
| V1 | Button (Switch mode) | Sends 1 (ON) or 0 (OFF) to ESP32 via BLYNK_WRITE(V1) handler |

The 1N4007 diode across the relay coil provides flyback protection — when the coil is de-energised, the collapsing magnetic field generates a reverse voltage spike that would damage the transistor without the diode.

PROGRAM CODE (ESP32 — Arduino IDE)

#define BLYNK_TEMPLATE_ID "TMPLxxxxxxxxx"

#define BLYNK_TEMPLATE_NAME "Relay Control"

#define BLYNK_AUTH_TOKEN "YourAuthTokenHere"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#define RELAY_PIN 2 // GPIO2 drives SL100 base via 1kΩ

char ssid[] = "Your_WiFi_SSID";

char pass[] = "Your_WiFi_Password";

BLYNK_WRITE(V1) { // Blynk app Button on Virtual Pin V1

int val = param.asInt(); // 1 = ON, 0 = OFF

digitalWrite(RELAY_PIN, val);

Serial.print("Relay: ");

Serial.println(val ? "ON" : "OFF");

}

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW); // relay OFF at start

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

void loop() {

Blynk.run();

}

BLYNK SETUP

1. Sign in at https://blynk.cloud and create a new Template (Hardware: ESP32, Connection: WiFi).

2. Add a Datastream: V1 (Integer, Min 0, Max 1).

3. In Web Dashboard, add a Button widget → assign to V1 → set to Switch mode.

4. Copy TEMPLATE_ID, TEMPLATE_NAME, and AUTH_TOKEN into the code.

5. In the Blynk mobile app, create a dashboard with a Button widget on V1.

PROCEDURE

1. Build the circuit: GPIO2 → 1kΩ → SL100 base; SL100 collector → relay coil (−); 5V → relay coil (+); 1N4007 across coil; relay NO → load.

2. Install Blynk library in Arduino IDE (Sketch → Manage Libraries → search "Blynk").

3. Update BLYNK_TEMPLATE_ID, BLYNK_AUTH_TOKEN, ssid, and pass in the code.

4. Upload the code. Open Serial Monitor at 115200 baud.

5. Confirm "Connected to Blynk" appears in Serial Monitor.

6. Open the Blynk app → press the Button widget.

7. Observe the relay clicking and the load switching ON and OFF.

8. Verify relay state matches the button state in the app.

RESULT

RESULT The relay was successfully controlled through the Blynk IoT platform using an ESP32 and SL100 transistor driver. The BLYNK_WRITE(V1) handler received button state changes from the Blynk app over WiFi and toggled GPIO2 accordingly, driving the SL100 transistor which energised or de-energised the relay coil, switching the connected load ON and OFF remotely.