APPENDIX 3 — EM18 RFID READER BASIC INTERFACE (ESP32)

AIM

To interface the EM18 125 kHz RFID reader with an ESP32 microcontroller using UART serial communication and read and display the 12-character card UID on the Serial Monitor whenever a compatible card is presented.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | Read serial data |
| 2 | RFID Reader | EM18, 125 kHz | 1 | Read card UID |
| 3 | RFID Cards / Key Fobs | EM4100, 125 kHz | 2+ | Tags to scan |
| 4 | Resistor | 1 kΩ, 1/4 W | 1 | Voltage divider upper |
| 5 | Resistor | 2 kΩ, 1/4 W | 1 | Voltage divider lower |
| 6 | Breadboard + Jumpers | — | 1 set | Connections |

CIRCUIT DIAGRAM

IMPORTANT: EM18 TX is 5V TTL. Use 1kΩ/2kΩ voltage divider to bring signal to ~3.33 V. SEL must be connected to GND to enable UART output.

EM18 Module ESP32

VCC → 5V (VIN)

GND → GND

TX → [1kΩ] → node → GPIO3 (RX0)

node → [2kΩ] → GND

SEL → GND (enable UART mode)

BEEP → [1kΩ] → LED → GND (optional)

Voltage at node = 5 × 2/(1+2) = 3.33 V (safe for ESP32)

Figure A3.1: EM18 TX → 1kΩ/2kΩ divider → GPIO3 (RX0)

THEORY

The EM18 reads passive EM4100 cards at 125 kHz. When a card is in range (~8–12 cm), it reads the 40-bit unique ID and transmits 12 ASCII bytes over UART at 9600 baud.

| Bytes | Content | Description |
| --- | --- | --- |
| 0–1 | "XX" (2 hex chars) | Version / Customer code byte |
| 2–9 | "XXXXXXXX" (8 hex chars) | Card unique ID (4 bytes) |
| 10–11 | "XX" (2 hex chars) | XOR checksum of all 5 data bytes |

SEL pin: LOW (GND) = UART serial output; HIGH or floating = Wiegand output. Must be tied to GND for this experiment.

PROGRAM CODE (ESP32 — Arduino IDE)

/* EM18 RFID Basic — ESP32

EM18 TX → 1kΩ/2kΩ divider → GPIO3 (RX0)

EM18 SEL → GND (UART mode)

Serial baud: 9600 */

int count = 0;

char card_no[12];

void setup()

{

Serial.begin(9600);

}

void loop()

{

if (Serial.available())

{

count = 0;

while (Serial.available() && count < 12)

{

card_no[count] = Serial.read();

count++;

delay(5);

}

Serial.print(card_no);

}

}

PROCEDURE

1. Build the voltage divider: 1kΩ from EM18 TX to a node; 2kΩ from that node to GND; node to GPIO3 (RX0).

2. Connect EM18 VCC to VIN (5V) and GND to GND. Connect SEL to GND.

3. Upload the code. Open Serial Monitor at 9600 baud.

4. Bring an EM4100 125 kHz card within 8–12 cm of the reader.

5. Observe the 12-character card data printed in the Serial Monitor.

6. Test with a second card and confirm a different ID is displayed.

RESULT

RESULT The EM18 125 kHz RFID reader was successfully interfaced with the ESP32 using UART0 (GPIO3/RX0) through a 1kΩ/2kΩ voltage divider. The SEL pin was tied to GND to select UART mode. When a 125 kHz card was brought near the reader, the 12-character card data was received and displayed on the Serial Monitor at 9600 baud.