APPENDIX 4 — EM18 RFID ACCESS CONTROL SYSTEM (ESP32)

AIM

To implement an RFID-based access control system using the EM18 reader and ESP32, where only pre-registered card UIDs are granted access (LED ON for 1 second), and unregistered cards are denied (LED blinks 3 times), with XOR checksum verification.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | Access control logic |
| 2 | RFID Reader | EM18, 125 kHz | 1 | Read card UID |
| 3 | RFID Cards | EM4100 125 kHz | 2+ | Authorised + unauthorised tags |
| 4 | LED | Any colour, 5mm | 1 | Access indicator |
| 5 | Resistor | 470 Ω, 1/4 W | 1 | LED current limiting |
| 6 | Resistor | 1 kΩ + 2 kΩ | 1 each | Voltage divider for EM18 TX |

CIRCUIT DIAGRAM

| EM18 Pin | Connection | Notes |
| --- | --- | --- |
| VCC | 5V (VIN) | EM18 supply |
| GND | GND | Common ground |
| TX | 1kΩ → node → GPIO16 (RX2) | 2kΩ from node to GND; node to GPIO16 |
| SEL | GND | UART mode select |
| LED Anode | GPIO2 → 470Ω → LED | Access indicator |

Uses Serial2 (GPIO16/RX2) to leave Serial0 free for debug output.

THEORY

XOR Checksum Verification:

The EM18 sends 12 ASCII characters: 10 hex characters (5 data bytes) + 2 hex characters (XOR checksum). The checksum is XOR of all 5 data bytes. The checksumOk() function verifies integrity before accepting the card ID.

Access Control Logic:

| Condition | LED Response | Serial Output |
| --- | --- | --- |
| Checksum fails | No response | "Checksum error" |
| UID in allowedIDs[] | ON for 1 second | "ACCESS GRANTED" |
| UID not in allowedIDs[] | Blinks 3 times (200 ms each) | "ACCESS DENIED" |

PROGRAM CODE (ESP32 — Arduino IDE)

#define RXD2 16

#define TXD2 17

#define LED_PIN 2

const byte NUM_ALLOWED = 2;

const String allowedIDs[NUM_ALLOWED] = {"1A2B3C4D5E", "0011223344"};

bool isAllowed(const String &id) {

for (byte i = 0; i < NUM_ALLOWED; i++)

if (id == allowedIDs[i]) return true;

return false;

}

byte hexVal(char c) {

if (c >= "0" && c <= "9") return c - "0";

if (c >= "A" && c <= "F") return c - "A" + 10;

if (c >= "a" && c <= "f") return c - "a" + 10;

return 0;

}

bool checksumOk(const char *buf) {

byte x = 0;

for (int i = 0; i < 10; i += 2)

x ^= (hexVal(buf[i]) << 4) ¦ hexVal(buf[i+1]);

byte received = (hexVal(buf[10]) << 4) ¦ hexVal(buf[11]);

return x == received;

}

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, RXD2, TXD2);

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

Serial.println("RFID Access Control Ready");

}

void loop() {

if (Serial2.available() >= 12) {

char buf[13]; buf[12] = 0;

for (int i = 0; i < 12; i++) {

buf[i] = Serial2.read();

delay(5);

}

String uid = String(buf).substring(0, 10);

Serial.print("Card: "); Serial.println(uid);

if (!checksumOk(buf)) {

Serial.println("Checksum error");

return;

}

if (isAllowed(uid)) {

Serial.println("ACCESS GRANTED");

digitalWrite(LED_PIN, HIGH);

delay(1000);

digitalWrite(LED_PIN, LOW);

} else {

Serial.println("ACCESS DENIED");

for (int i = 0; i < 3; i++) {

digitalWrite(LED_PIN, HIGH); delay(200);

digitalWrite(LED_PIN, LOW); delay(200);

}

}

}

}

REGISTERING A CARD UID

1. Run Appendix 3 (basic read) code first. Scan your card and note the 10-character UID.

2. In the access control code, add that UID to the allowedIDs[] array.

3. Re-upload the code.

PROCEDURE

1. Connect EM18 as described. Connect LED with 470Ω to GPIO2.

2. Register at least one card UID in the allowedIDs[] array.

3. Upload the code. Open Serial Monitor at 115200 baud.

4. Scan a registered card — LED stays ON for 1 second; Serial shows "ACCESS GRANTED".

5. Scan an unregistered card — LED blinks 3 times; Serial shows "ACCESS DENIED".

6. Record UIDs and responses in the observation table.

RESULT

RESULT An RFID-based access control system was implemented on ESP32 using the EM18 reader on Serial2 (GPIO16). XOR checksum verification confirmed data integrity. Pre-registered card UIDs triggered LED ON for 1 second (ACCESS GRANTED), while unregistered UIDs caused 3 LED blinks (ACCESS DENIED), demonstrating a functional access control system.