APPENDIX 6 — NEO-6M GPS MODULE WITH ESP32

AIM

To interface the u-blox NEO-6M GPS module with the ESP32 using UART2 and the TinyGPS++ library to read and display GPS location data (latitude, longitude, altitude, satellites, HDOP) on the Serial Monitor.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | Parse NMEA data |
| 2 | GPS Module | u-blox NEO-6M (3.3V TTL) | 1 | GPS receiver |
| 3 | GPS Antenna | Ceramic patch antenna (included) | 1 | Signal reception |
| 4 | Jumper Wires | Male-to-Female | 4 | Connections |

CIRCUIT DIAGRAM

NEO-6M Module ESP32

VCC → 3.3V

GND → GND

TX → GPIO16 (RX2) (no level shifting needed — 3.3V TTL)

RX → GPIO17 (TX2) (optional — only needed for config)

No voltage divider required — NEO-6M is 3.3V TTL, same as ESP32.

Place GPS module near a window or outdoors for satellite fix.

THEORY

NMEA Sentences:

| Sentence | Contains | Key Fields |
| --- | --- | --- |
| $GPGGA | Fix data | Latitude, Longitude, Altitude, Satellites, HDOP |
| $GPRMC | Recommended minimum | Lat, Lng, Speed, Date, Time |
| $GPGSV | Satellites in view | Satellite IDs, elevation, azimuth, SNR |

TinyGPS++ library parses these NMEA sentences automatically. HDOP (Horizontal Dilution of Precision) indicates fix quality — HDOP < 2.0 is good; < 1.0 is excellent.

LIBRARY INSTALLATION

Arduino IDE → Sketch → Manage Libraries → search "TinyGPS++" → install by Mikal Hart.

PROGRAM CODE (ESP32 — Arduino IDE)

#include <TinyGPS++.h>

#include <HardwareSerial.h>

#define GPS_RX 16 // ESP32 GPIO16 ← GPS TX

#define GPS_TX 17 // ESP32 GPIO17 → GPS RX (optional)

TinyGPSPlus gps;

HardwareSerial gpsSerial(2); // UART2

void setup() {

Serial.begin(115200);

gpsSerial.begin(9600, SERIAL_8N1, GPS_RX, GPS_TX);

Serial.println("NEO-6M GPS — Waiting for fix...");

}

void loop() {

while (gpsSerial.available())

gps.encode(gpsSerial.read());

if (gps.location.isUpdated()) {

Serial.print("Lat: "); Serial.print(gps.location.lat(), 6);

Serial.print(" Lng: "); Serial.print(gps.location.lng(), 6);

Serial.print(" Alt: "); Serial.print(gps.altitude.meters(), 1);

Serial.print(" m Sat: "); Serial.print(gps.satellites.value());

Serial.print(" HDOP: "); Serial.println(gps.hdop.hdop(), 2);

}

}

OBSERVATION TABLE

| Reading | Latitude | Longitude | Altitude (m) | Satellites | HDOP |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

PROCEDURE

1. Connect NEO-6M: VCC → 3.3V, GND → GND, TX → GPIO16, RX → GPIO17.

2. Install TinyGPS++ library in Arduino IDE.

3. Upload the code. Open Serial Monitor at 115200 baud.

4. Place the GPS module near an open window or take it outdoors for satellite reception.

5. Wait for fix (may take 1–5 minutes on first use — LED on module blinks when locked).

6. Record latitude, longitude, altitude, satellite count, and HDOP in the observation table.

7. Verify location against Google Maps using the displayed coordinates.

RESULT

RESULT The NEO-6M GPS module was successfully interfaced with the ESP32 using UART2 (GPIO16/RX2) with direct 3.3V TTL connection — no voltage divider required. The TinyGPS++ library parsed NMEA sentences and provided latitude, longitude, altitude, satellite count, and HDOP. GPS coordinates were verified against Google Maps and found to be accurate.