APPENDIX 7 — EVENT DURATION TIMER USING PUSH BUTTON & ESP32

AIM

To measure the time duration between two push button press events using the ESP32 millis() function — the first press starts the timer and the second press stops it — and display the elapsed time in seconds (with millisecond precision) on the Serial Monitor.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | Timer & serial output |
| 2 | Push Button | Momentary tactile switch | 1 | Start / stop event |
| 3 | Breadboard + Jumpers | — | 1 set | Connections |
| 4 | USB Cable | Micro USB / Type-C | 1 | Power & Serial Monitor |

CIRCUIT DIAGRAM

Push Button ESP32

Terminal 1 → GPIO4 (INPUT_PULLUP)

Terminal 2 → GND

No external resistor needed — ESP32 internal pull-up used.

Resting state: GPIO4 = HIGH (3.3V via internal pull-up)

Pressed state: GPIO4 = LOW (shorted to GND)

THEORY

millis() returns the number of milliseconds elapsed since the ESP32 was powered on. It runs from a hardware timer continuously in the background.

| Step | Operation |
| --- | --- |
| 1st press | startTime = millis() — record start timestamp |
| 2nd press | elapsed = millis() − startTime — duration in ms |
| Convert | seconds = elapsed / 1000.0 (float for decimal places) |
| Display | Serial.print(seconds, 3) — 3 decimal places |

Software debounce: after LOW detected, wait 50ms and re-read. If still LOW → confirmed press. Wait for button release + 50ms delay prevents repeat triggering.

PROGRAM CODE (ESP32 — Arduino IDE)

#define BTN_PIN 4 // push button — other terminal to GND

bool timing = false;

unsigned long startTime = 0;

void setup() {

Serial.begin(115200);

pinMode(BTN_PIN, INPUT_PULLUP);

Serial.println("Event Duration Timer Ready");

Serial.println("Press button to START...");

}

void loop() {

if (digitalRead(BTN_PIN) == LOW) {

delay(50);

if (digitalRead(BTN_PIN) == LOW) {

if (!timing) {

startTime = millis();

timing = true;

Serial.println("Timer STARTED.");

Serial.println("Press again to STOP...");

} else {

unsigned long elapsed = millis() - startTime;

timing = false;

Serial.println("Timer STOPPED.");

Serial.print("Duration : ");

Serial.print(elapsed / 1000.0, 3);

Serial.println(" seconds");

Serial.print(" ");

Serial.print(elapsed);

Serial.println(" ms");

Serial.println("─────────────────────────────");

Serial.println("Press button to START again...");

}

while (digitalRead(BTN_PIN) == LOW);

delay(50);

}

}

}

EXPECTED SERIAL OUTPUT

Event Duration Timer Ready

Press button to START...

Timer STARTED.

Press again to STOP...

Timer STOPPED.

Duration : 4.382 seconds

4382 ms

─────────────────────────────

Press button to START again...

OBSERVATION TABLE

| Trial | Event | Duration (seconds) | Duration (ms) |
| --- | --- | --- | --- |
| 1 | Button pair 1 |  |  |
| 2 | Button pair 2 |  |  |
| 3 | Button pair 3 |  |  |
| 4 | Button pair 4 |  |  |

PROCEDURE

1. Connect one terminal of the push button to GPIO4 and the other to GND. No external resistor needed.

2. Upload the code. Open Serial Monitor at 115200 baud.

3. Press the button once — Serial Monitor shows "Timer STARTED."

4. Wait 5 seconds and press the button again.

5. Observe the duration in seconds and milliseconds.

6. Record start and stop times and computed duration in the observation table.

7. Repeat for 3 more trials with different intervals.

RESULT

RESULT The event duration timer was successfully implemented using a push button on GPIO4 (INPUT_PULLUP) and the ESP32 millis() function. The first press recorded the start timestamp; the second press computed the elapsed duration. Results were displayed with 3-decimal-place second precision (millisecond resolution) on the Serial Monitor. Software debouncing prevented false triggering.