APPENDIX 8 — ENERGY UNIT CALCULATOR (ESP32 + ACS712-20A)

AIM

To calculate electrical energy consumed by a load during a measured time interval using the ACS712-20A current sensor and ESP32. A push button starts and stops measurement. Supply voltage is assumed as 230 V AC with unity power factor.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | ADC, timer, processing |
| 2 | Current Sensor | ACS712-20A Module | 1 | Measure load current |
| 3 | Push Button | Momentary tactile switch | 1 | Start / stop measurement |
| 4 | Resistor | 10 kΩ + 20 kΩ, 1/4 W | 1 each | Voltage divider for SOUT |
| 5 | AC Load | Resistive (bulb/heater) | 1 | Load under measurement |

CIRCUIT DIAGRAM

ACS712 VCC → 5V (VIN)

ACS712 GND → GND

ACS712 SOUT → [10kΩ] → node → GPIO34

node → [20kΩ] → GND

Push button → GPIO4 (INPUT_PULLUP) ¦ other terminal → GND

AC Load circuit (230V mains — handle with care):

230V Line → ACS712 IP+ ── [Load] ── IP− → 230V Neutral

WARNING: IP+/IP− carry 230V mains. ACS712 provides 2.1kV galvanic isolation between the mains side and signal side.

THEORY

| Parameter | Formula / Value |
| --- | --- |
| Supply Voltage | 230 V AC (assumed) |
| Power Factor | 1.0 (unity — resistive load) |
| Current from sensor | I (A) = (V_out − 2.5) / 0.1 |
| Power | P (W) = 230 × I × 1.0 |
| Energy per interval | dE (Wh) = P × (dt / 3600000) [dt in ms] |
| Total Energy | E_total = Σ dE (sum over all samples) |
| Units consumed | kWh = E_total / 1000 |

PROGRAM CODE (ESP32 — Arduino IDE)

#define BTN_PIN 4

#define CURRENT_PIN 34

#define V_SUPPLY 230.0

#define POWER_FACTOR 1.0

#define SENSITIVITY 0.100 // ACS712-20A: 100 mV/A

#define V_ZERO 2.500

#define DIVIDER_K 0.6667

#define SAMPLES 150

#define SAMPLE_MS 500

bool measuring = false;

unsigned long startTime = 0;

unsigned long lastSample = 0;

float energyWh = 0.0;

float peakCurrent = 0.0;

float readCurrent() {

long sum = 0;

for (int i = 0; i < SAMPLES; i++) {

sum += analogRead(CURRENT_PIN);

delayMicroseconds(100);

}

float V_adc = ((float)sum / SAMPLES / 4095.0) * 3.3;

float V_out = V_adc / DIVIDER_K;

return abs((V_out - V_ZERO) / SENSITIVITY);

}

void setup() {

Serial.begin(115200);

pinMode(BTN_PIN, INPUT_PULLUP);

analogReadResolution(12);

analogSetAttenuation(ADC_11db);

Serial.println("Energy Calculator Ready — Press button to START");

}

void loop() {

if (digitalRead(BTN_PIN) == LOW) {

delay(50);

if (digitalRead(BTN_PIN) == LOW) {

if (!measuring) {

measuring = true; startTime = millis();

lastSample = millis(); energyWh = 0; peakCurrent = 0;

Serial.println("\nMeasurement STARTED. Current ¦ Power ¦ Energy");

} else {

measuring = false;

unsigned long elapsed = millis() - startTime;

Serial.println("\n===== ENERGY REPORT =====");

Serial.print("Duration : "); Serial.print(elapsed/1000.0,2); Serial.println(" s");

Serial.print("Peak I : "); Serial.print(peakCurrent,3); Serial.println(" A");

Serial.print("Energy : "); Serial.print(energyWh,4); Serial.println(" Wh");

Serial.print("Units (kWh): "); Serial.println(energyWh/1000.0,7);

Serial.println("=========================");

}

while (digitalRead(BTN_PIN) == LOW); delay(50);

}

}

if (measuring && (millis() - lastSample >= SAMPLE_MS)) {

unsigned long dt = millis() - lastSample; lastSample = millis();

float I = readCurrent();

float P = V_SUPPLY * I * POWER_FACTOR;

energyWh += P * (dt / 3600000.0);

if (I > peakCurrent) peakCurrent = I;

Serial.print(I,3); Serial.print(" A ¦ ");

Serial.print(P,1); Serial.print(" W ¦ ");

Serial.print(energyWh,5); Serial.println(" Wh");

}

}

OBSERVATION TABLE

| Trial | Load | Duration (s) | Peak I (A) | Energy (Wh) | Units (kWh) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

PROCEDURE

1. Connect ACS712 SOUT through 10kΩ/20kΩ divider to GPIO34. Connect button to GPIO4 → GND.

2. Connect the AC load in series through ACS712 IP+ and IP−.

3. Upload the code. Open Serial Monitor at 115200 baud.

4. Switch ON the AC load.

5. Press button once — current, power, and running energy appear every 500 ms.

6. Press button again to stop — energy report is displayed.

7. Record values in the observation table.

RESULT

RESULT The energy unit calculator was successfully implemented. The ACS712-20A measured current via a 10kΩ/20kΩ divider on GPIO34. A push button on GPIO4 started and stopped measurement. Energy was integrated as dE = P × dt every 500ms and accumulated. The final report showed total duration, peak current, energy in Wh, and units in kWh.