APPENDIX 9 — DHT11/DHT22 SENSOR WITH I2C LCD AND BLYNK IoT (ESP32)

AIM

To interface the DHT11/DHT22 temperature and humidity sensor with the ESP32, display readings on a 16×2 I2C LCD, and simultaneously send data to the Blynk IoT cloud platform for real-time remote monitoring.

COMPONENTS REQUIRED

| Sl. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 Dev Board | 1 | WiFi, processing, ADC |
| 2 | Temp & Humidity Sensor | DHT11 or DHT22 module | 1 | Measure temp & humidity |
| 3 | I2C LCD Display | 16×2 with PCF8574 backpack | 1 | Local display |
| 4 | Resistor | 10 kΩ, 1/4 W | 1 | Pull-up for DHT DATA pin |
| 5 | Smartphone | Blynk app installed | 1 | Remote monitoring |

CIRCUIT DIAGRAM

DHT11/22 ESP32

VCC → 3.3V

GND → GND

DATA → GPIO4 + [10kΩ pull-up from DATA to 3.3V]

I2C LCD (PCF8574 backpack)

VCC → VIN (5V)

GND → GND

SDA → GPIO21

SCL → GPIO22

If LCD stays blank: run I2C scanner to check address (0x27 or 0x3F).

THEORY

| Parameter | DHT11 | DHT22 (AM2302) |
| --- | --- | --- |
| Temp Range | 0–50 °C | -40–80 °C |
| Temp Accuracy | ±2 °C | ±0.5 °C |
| Humidity Range | 20–80% RH | 0–100% RH |
| Humidity Accuracy | ±5% RH | ±2–5% RH |
| Sampling Rate | Max 1/second | Max 1 per 2 seconds |

Blynk Virtual Pins: V0 = Temperature (°C), V1 = Humidity (%). The 2000ms timer interval satisfies both DHT11 (min 1s) and DHT22 (min 2s) requirements.

LIBRARY INSTALLATION

1. DHT sensor library — search "DHT sensor library" → install by Adafruit (also install Adafruit Unified Sensor when prompted).

2. LiquidCrystal_I2C — search "LiquidCrystal I2C" → install by Frank de Brabander.

3. Blynk — search "Blynk" → install by Blynk.

PROGRAM CODE (ESP32 — Arduino IDE)

#define BLYNK_TEMPLATE_ID "TMPLxxxxxxxxx"

#define BLYNK_TEMPLATE_NAME "DHT Monitor"

#define BLYNK_AUTH_TOKEN "YourAuthTokenHere"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include <DHT.h>

#include <LiquidCrystal_I2C.h>

#define DHTPIN 4

#define DHTTYPE DHT11 // change to DHT22 if needed

char ssid[] = "Your_WiFi_SSID";

char pass[] = "Your_WiFi_Password";

DHT dht(DHTPIN, DHTTYPE);

LiquidCrystal_I2C lcd(0x27, 16, 2);

BlynkTimer timer;

void sendSensorData() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

if (isnan(humidity) ¦¦ isnan(temperature)) {

Serial.println("DHT read failed!");

return;

}

lcd.setCursor(0, 0);

lcd.print("Temp: "); lcd.print(temperature, 1); lcd.print(" C ");

lcd.setCursor(0, 1);

lcd.print("Hum: "); lcd.print(humidity, 1); lcd.print(" % ");

Blynk.virtualWrite(V0, temperature);

Blynk.virtualWrite(V1, humidity);

Serial.print("Temp: "); Serial.print(temperature,1);

Serial.print(" C ¦ Hum: "); Serial.print(humidity,1); Serial.println(" %");

}

void setup() {

Serial.begin(115200);

lcd.init(); lcd.backlight();

lcd.setCursor(0,0); lcd.print("DHT IoT Monitor");

lcd.setCursor(0,1); lcd.print("Connecting...");

dht.begin();

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

lcd.clear();

timer.setInterval(2000L, sendSensorData);

}

void loop() {

Blynk.run();

timer.run();

}

EXPECTED LCD DISPLAY

+────────────────+

│ Temp: 28.5 C │

│ Hum: 62.3 % │

+────────────────+

OBSERVATION TABLE

| Sl. | Time | Temp LCD (°C) | Hum LCD (%) | Temp Blynk | Hum Blynk |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |

PROCEDURE

1. Connect DHT DATA to GPIO4 with a 10kΩ pull-up from DATA to 3.3V.

2. Connect I2C LCD: SDA → GPIO21, SCL → GPIO22, VCC → VIN, GND → GND.

3. Install Adafruit DHT, LiquidCrystal_I2C, and Blynk libraries.

4. Set up Blynk template with V0 (Temp, 0–60°C) and V1 (Humidity, 0–100%) datastreams.

5. Update BLYNK_AUTH_TOKEN, ssid, and pass in the code. Upload.

6. Open Serial Monitor at 115200 baud — confirm WiFi connected and readings appear.

7. Check LCD — Line 1: temperature, Line 2: humidity, updating every 2 seconds.

8. Open Blynk app — verify V0 and V1 widgets receive live data.

9. Record readings from LCD and Blynk in the observation table at 4 different times.

RESULT

RESULT The DHT11/DHT22 sensor was successfully interfaced with the ESP32. Temperature and humidity readings were displayed on the 16×2 I2C LCD (GPIO21/SDA, GPIO22/SCL) and simultaneously published to Blynk IoT cloud via WiFi using virtual pins V0 and V1. LCD and Blynk readings were consistent, confirming correct sensor reading, local display, and cloud connectivity.