QUESTION BANK

Internet of Things & Applications  |  PEMRT525

KTU  |  S5  |  Batch MR 2K24  |  JECC

COURSE OUTCOMES (COs)

| CO | Description |
| --- | --- |
| CO1 | Describe the fundamental concepts, characteristics, and architecture of IoT systems. |
| CO2 | Explain the physical and logical design of IoT systems including protocols and functional blocks. |
| CO3 | Apply IoT communication models and enabling technologies to real-world IoT scenarios. |
| CO4 | Analyze M2M, SDN, NFV, sensors, actuators, and WSN concepts for IoT design. |
| CO5 | Evaluate and select appropriate communication protocols and criteria for IoT applications. |

BLOOM'S TAXONOMY LEVELS

| L1 — Remember | L2 — Understand | L3 — Apply | L4 — Analyse | L5 — Evaluate | L6 — Create |
| --- | --- | --- | --- | --- | --- |
| Define, List, State, Recall, Name | Explain, Describe, Classify, Summarise | Apply, Use, Demonstrate, Solve | Differentiate, Analyse, Examine, Compare | Evaluate, Justify, Assess, Recommend | Design, Create, Formulate, Construct |

How to Use: 3-Mark questions require a definition + 2–3 key points (½–1 page). 9-Mark questions require detailed explanation with diagrams/tables (2–3 pages). Each question is tagged with Bloom's Level (L1–L5) and Course Outcome (CO1–CO5). All answers can be reproduced from the Module 1 & 2 Teaching Notes.

MODULE 1 — QUESTION BANK

Introduction to IoT | Architecture | Physical & Logical Design | Communication Models | Enabling Technologies | IoT Levels

| Q1 ¦ L1 | Q2 ¦ L2 | Q3 ¦ L4 | Q4 ¦ L2 | Q5 ¦ L3 | Q6 ¦ L4 |
| --- | --- | --- | --- | --- | --- |
| REMEMBER | UNDERSTAND | ANALYSE | UNDERSTAND | APPLY | ANALYSE |

SECTION A — 3 Mark Questions  (Short Answer)

Q1 [3 Marks] Define IoT. List and recall any FIVE characteristics of IoT with a one-line description of each. 🎯 Bloom's Level: L1 — REMEMBER 📚 Course Outcome: CO1 📖 Topic 1 — Introduction to IoT

✏  Answer Key Points:

IoT = network of uniquely identified, Internet-connected physical/virtual 'things' that sense, communicate, and act.

5 characteristics: (1) Dynamic & Self-Adapting — changes behaviour based on context. (2) Self-Configuring — sets up without manual intervention. (3) Interoperable — supports multiple protocols. (4) Unique Identity — each device has a unique ID/IP. (5) Network-Integrated — discovers and exchanges data with other devices.

Mnemonic: DSIUI — Dynamic, Self-Configuring, Interoperable, Unique, Integrated.

Q2 [3 Marks] Describe the four layers of IoT system architecture and explain the role of each layer in the data flow. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO1 📖 Topic 2 — IoT Architecture

✏  Answer Key Points:

4 layers: (1) Sensing — sensors/actuators collect physical world data. (2) Network — transmits data via WiFi/ZigBee/LTE. (3) Processing/Middleware — cloud stores and processes data. (4) Application — user-facing dashboards and control apps.

Data flows UPWARD (sensor→cloud); commands flow DOWNWARD (app→device).

Example: soil sensor (Sensing) → WiFi (Network) → AWS (Processing) → farmer app (Application).

Q3 [3 Marks] Differentiate between IPv4 and IPv6. Justify why IPv6 is essential for IoT deployments. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO2 📖 Topic 3 — Physical Design / Network Layer

✏  Answer Key Points:

IPv4: 32-bit, ~4.3 billion addresses, uses NAT, dot-decimal notation.

IPv6: 128-bit, 3.4 × 10³⁸ addresses, no NAT needed, colon-hex notation.

Justification: IoT is expected to connect 50+ billion devices — IPv4 addresses are exhausted. IPv6 + 6LoWPAN enables IP addressing on every constrained sensor node.

Q4 [3 Marks] Explain the Publish-Subscribe communication model. Identify the three roles and the protocol that implements it. State ONE key advantage over Request-Response. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO3 📖 Topic 5 — Communication Models

✏  Answer Key Points:

Publish-Subscribe: publishers send data to named topics; a broker routes messages; subscribers receive from topics they are subscribed to.

Three roles: Publisher (data producer), Broker (message router/post office), Subscriber (data consumer).

Protocol: MQTT (also AMQP, DDS).

Key advantage: DECOUPLING — publishers and subscribers do not need to know each other; they can operate independently.

Q5 [3 Marks] Identify which IoT functional block is responsible for authentication, encryption, and data integrity. Justify why this block is described as a 'cross-cutting concern' across all other blocks. 🎯 Bloom's Level: L3 — APPLY 📚 Course Outcome: CO2 📖 Topic 4 — Logical Design / Functional Blocks

✏  Answer Key Points:

The Security functional block handles: Authentication (verify identity), Authorization (access control), Data Integrity (prevent tampering), Encryption (protect data in transit/at rest).

Cross-cutting concern: security does not belong to one layer — it must be applied at Device, Communication, Services, Management, and Application levels simultaneously.

Example: a smart meter must authenticate the cloud server (device level), encrypt sensor data (communication level), and restrict dashboard access (application level).

Q6 [3 Marks] Compare TCP and UDP at the transport layer. Analyse which protocol is preferred for real-time IoT sensor streaming and why. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO3 📖 Topic 3 — Physical Design / Transport Layer

✏  Answer Key Points:

TCP: connection-oriented, reliable delivery, ordered packets, flow control, higher overhead. UDP: connectionless, unreliable, no ordering, low overhead, fast.

For real-time streaming (live sensor data, video): UDP is preferred — low latency is more important than guaranteed delivery; a lost temperature reading is acceptable but delay is not.

Protocol mapping: CoAP uses UDP (constrained IoT); MQTT uses TCP (reliable messaging); WebSocket uses TCP (persistent live dashboards).

| Q1 ¦ L2 | Q2 ¦ L2 | Q3 ¦ L3 | Q4 ¦ L4 | Q5 ¦ L4 | Q6 ¦ L5 |
| --- | --- | --- | --- | --- | --- |
| UNDERSTAND | UNDERSTAND | APPLY | ANALYSE | ANALYSE | EVALUATE |

SECTION B — 9 Mark Questions  (Long Answer)

Q1 [9 Marks] Explain the Physical Design of IoT with the four-layer IoT protocol stack. Describe the protocols used at each layer and justify their selection for IoT environments. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO2 📖 Topic 3 — Physical Design

✏  Answer Key Points:

Physical design = actual hardware (IoT devices) + the protocol stack they use to communicate.

Layer 1 — Link Layer: 802.3 (Ethernet), 802.11 (WiFi), 802.15.4 (ZigBee/low-power), LTE. Responsible for physical signal transmission.

Layer 2 — Network/IP Layer: IPv4, IPv6, 6LoWPAN. Responsible for addressing and routing. IPv6 needed for scale; 6LoWPAN compresses IPv6 headers for constrained devices.

Layer 3 — Transport Layer: TCP (reliable, ordered) for MQTT/HTTP; UDP (fast, lightweight) for CoAP/real-time streams.

Layer 4 — Application Layer: HTTP (web APIs), CoAP (constrained devices, runs over UDP), MQTT (lightweight pub-sub, runs over TCP), WebSocket (real-time full-duplex), XMPP.

Include table: Layer | Protocols | Key Characteristic | IoT Example.

Conclusion: layered stack enables end-to-end communication from a resource-constrained sensor to a cloud application.

Q2 [9 Marks] Describe all FOUR IoT communication models. Explain their operation with examples and summarise the differences in a comparison table. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO3 📖 Topic 5 — Communication Models

✏  Answer Key Points:

Model 1 — Request-Response: stateless; client sends request, server replies. Used by HTTP, CoAP. Example: mobile app queries weather API for temperature.

Model 2 — Publish-Subscribe: Publisher → Broker → Subscriber; decouples data producers and consumers. Used by MQTT. Example: temperature sensor publishes to '/home/temp'; dashboard subscribes.

Model 3 — Push-Pull (Pipeline): Producers push to a queue; consumers pull at their own pace. Handles speed mismatch. Example: sensor data queued for batch analytics.

Model 4 — Exclusive Pair: persistent full-duplex connection (WebSocket); stateful; both sides send anytime. Example: live factory equipment dashboard.

Comparison table columns: Model | Stateful? | Persistent connection? | Protocol | Best use case.

Conclusion: choose the model based on real-time needs — REST for standard APIs, WebSocket for live data, MQTT for sensor telemetry.

Q3 [9 Marks] Apply the IoT functional block model to a smart irrigation system. Map each system function to its corresponding functional block and justify your mapping. 🎯 Bloom's Level: L3 — APPLY 📚 Course Outcome: CO2 📖 Topic 4 — Logical Design

✏  Answer Key Points:

IoT functional blocks: Device, Communication, Services, Management, Security, Application.

Smart irrigation mapping: Device — soil moisture sensors + water pump actuators. Communication — MQTT over WiFi transmits sensor readings. Services — device discovery when new sensor nodes are added; data publishing to cloud. Management — firmware OTA updates to field sensors, fault alerts. Security — TLS encryption of MQTT, authentication of farmer's mobile app. Application — mobile dashboard showing soil moisture charts + manual pump override.

Diagram: show 6 blocks with smart irrigation examples in each.

Key insight: Security wraps ALL blocks — the irrigation gateway authenticates every device and encrypts all commands.

Conclusion: the functional block model is technology-independent — it can map to any IoT application domain.

Q4 [9 Marks] Analyse THREE IoT enabling technologies — Wireless Sensor Networks (WSN), Cloud Computing, and Big Data Analytics. Examine how each technology addresses a specific IoT challenge and explain how they form an integrated IoT pipeline. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO3 📖 Topic 6 — Enabling Technologies

✏  Answer Key Points:

WSN: network of sensor nodes + routers + coordinator + gateway. Uses ZigBee/802.15.4. Challenge addressed: large-scale distributed sensing. Self-organising topology. Applications: weather, smart grid, structural health monitoring.

Cloud Computing: on-demand computing/storage/services over Internet. Models: IaaS (rent server — EC2), PaaS (rent platform — AWS IoT Core), SaaS (rent app — Salesforce). Challenge addressed: IoT devices have limited local storage/compute — cloud provides unlimited scale.

Big Data Analytics: 3 Vs — Volume (petabytes from millions of devices), Velocity (real-time streaming data), Variety (structured sensor readings + unstructured video/audio). Tools: Hadoop (batch), Spark (stream). Challenge addressed: traditional databases cannot process IoT-scale data.

Integrated pipeline: WSN collects → network transmits → Cloud stores → Big Data analyses → Application presents insights. Example: wind farm (500 turbines × 100 sensors = Big Data) — WSN transmits → AWS stores → Spark analyses vibrations → dashboard alerts engineer.

Diagram: linear pipeline with WSN → Gateway → Cloud → Analytics → App.

Q5 [9 Marks] Compare REST API and WebSocket API across key criteria. Analyse the six architectural constraints of REST and evaluate which API is more suitable for a live IoT sensor dashboard versus a historical data query service. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO3 📖 Topic 5 — Communication Models / APIs

✏  Answer Key Points:

REST definition: architectural style for stateless web APIs using HTTP verbs (GET/POST/PUT/DELETE). WebSocket: persistent, full-duplex TCP connection for bidirectional real-time communication.

Six REST constraints: (1) Client-Server — UI and data storage separated. (2) Stateless — each request contains all context, no session stored. (3) Cacheable — responses can be cached to reduce load. (4) Uniform Interface — standard HTTP verbs, resource URIs, self-descriptive messages. (5) Layered System — client cannot tell if it is talking to a load balancer or origin server. (6) Code on Demand (optional) — server can send executable code (JavaScript).

WebSocket: stateful, persistent, server pushes without client request, low latency, no HTTP header overhead after handshake.

Comparison table: Criterion | REST | WebSocket — Connection type, State, Direction, Latency, Overhead, Best for.

Evaluation: Live IoT sensor dashboard → WebSocket (low latency, server push, continuous data). Historical data query → REST (stateless, cacheable, standard HTTP tooling).

Real example: live power grid monitoring = WebSocket; monthly energy report API = REST.

Q6 [9 Marks] Evaluate the six IoT deployment levels defined by Bahga & Madisetti. Justify which level is most appropriate for a multi-building campus energy monitoring system and explain your reasoning. 🎯 Bloom's Level: L5 — EVALUATE 📚 Course Outcome: CO1 📖 Topic 7 — IoT Levels & Deployment Templates

✏  Answer Key Points:

Level 1: Single node, all analysis + storage local. Example: standalone datalogger.

Level 2: Single node, storage in cloud but analysis local.

Level 3: Single node, both storage and analysis in cloud. Example: smart thermostat.

Level 4: Multiple nodes + observer nodes. Observer monitors, does not control. Example: multi-room sensors.

Level 5: Multiple nodes + coordinator. Coordinator collects from all end nodes (WSN pattern). Example: campus weather network.

Level 6: Multiple independent nodes + centralized controller that commands all nodes. Example: smart grid.

Summary table: Level | Nodes | Analysis location | Storage | Key feature.

Justification for campus energy monitoring: Level 6 is most appropriate — multiple independent building energy meters (nodes) reporting to a central energy management controller that can issue commands (reduce HVAC load on peak demand). Cloud analysis provides historical trends. Central controller enables load balancing across buildings.

Alternative: Level 5 if only monitoring (no control) — justify based on requirement.

MODULE 2 — QUESTION BANK

IoT & M2M | SDN & NFV | Sensors | Actuators & MEMS | Smart Objects | WSNs | Protocols | Communication Criteria

| Q1 ¦ L1 | Q2 ¦ L4 | Q3 ¦ L2 | Q4 ¦ L1 | Q5 ¦ L2 | Q6 ¦ L3 |
| --- | --- | --- | --- | --- | --- |
| REMEMBER | ANALYSE | UNDERSTAND | REMEMBER | UNDERSTAND | APPLY |

SECTION A — 3 Mark Questions  (Short Answer)

Q1 [3 Marks] Define M2M. State the role of an M2M gateway using an example to illustrate how it connects an M2M area network to an external network. 🎯 Bloom's Level: L1 — REMEMBER 📚 Course Outcome: CO4 📖 Topic 1 — M2M

✏  Answer Key Points:

M2M (Machine-to-Machine): networking of machines/devices for remote monitoring, control, and data exchange WITHOUT human involvement.

M2M Gateway role: translates native non-IP M2M protocols (ZigBee, Modbus, PLC) ↔ Internet Protocol (IP) for the outside world. Acts as a proxy — makes each M2M node appear as a virtualised IP node.

Example: Smart metering — electricity meters communicate via Modbus on the M2M area network → M2M Gateway translates to IP → sends readings to utility company server over Internet.

Q2 [3 Marks] Distinguish between SDN and NFV using THREE points of difference. State whether they are dependent on each other and justify your answer. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO4 📖 Topics 2 & 3 — SDN & NFV

✏  Answer Key Points:

SDN: separates control plane (decisions) from data plane (forwarding); centralises control in software. NFV: virtualises network functions (firewall, NAT, router) as software (VNFs) on standard servers.

3 differences: (1) SDN virtualises control; NFV virtualises network appliance functions. (2) SDN uses OpenFlow protocol; NFV uses ETSI architecture (VNF + NFVI + MANO). (3) SDN acts on packet forwarding; NFV replaces dedicated hardware boxes.

Dependence: NOT dependent — each works independently. But they are COMPLEMENTARY: SDN can manage the network on which NFV-hosted VNFs run, creating an agile, scalable IoT infrastructure.

Q3 [3 Marks] Classify sensors using any THREE criteria. Provide one sensor example and one IoT application for each criterion. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO4 📖 Topic 4 — Sensors

✏  Answer Key Points:

Classification (1) by Energy Type: Active (requires external power — ultrasonic distance sensor in parking systems) vs Passive (generates own signal — PIR motion sensor in burglar alarms).

Classification (2) by Output Type: Analog (thermistor — outputs variable resistance for HVAC control) vs Digital (DHT22 — outputs binary-encoded temperature/humidity for smart agriculture).

Classification (3) by Physical Quantity Measured: Temperature (DS18B20 — cold chain monitoring), Pressure (BMP280 — weather station), Gas (MQ-2 — LPG leak detector in kitchen safety systems).

Note: sensors = INPUT devices in the IoT sense–decide–act cycle.

Q4 [3 Marks] Define a Smart Object. List its FOUR components. Describe how combining these components creates the 'smart' in a smart object. 🎯 Bloom's Level: L1 — REMEMBER 📚 Course Outcome: CO4 📖 Topic 5 — Smart Objects

✏  Answer Key Points:

Smart Object: a physical object embedded with a processor, sensor/actuator, communication unit, and power source — enabling it to sense, process, and communicate autonomously.

4 components: (1) Sensor/Actuator — perceives environment or acts on it. (2) Microprocessor/MCU — processes sensor data locally (edge computing). (3) Communication unit — connects to network (ZigBee, WiFi, BLE). (4) Power source — battery, mains, or energy harvesting.

The 'smart' comes from NETWORKING these components — an isolated sensor is not smart; a sensor that processes locally and communicates with other objects to take collective action IS smart.

Q5 [3 Marks] Explain 6LoWPAN. Describe the specific problem it solves and the mechanisms it uses to solve it. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO5 📖 Topic 7 — Communication Protocols

✏  Answer Key Points:

6LoWPAN (IPv6 over Low-Power Wireless Personal Area Networks): an IETF adaptation layer that enables IPv6 packets to traverse IEEE 802.15.4 (ZigBee) networks.

Problem: IPv6 minimum MTU = 1280 bytes. IEEE 802.15.4 maximum frame = 127 bytes. Standard IPv6 packets CANNOT fit in a 802.15.4 frame.

Mechanisms: (1) Header Compression — compresses IPv6 header from 40 bytes → as few as 2 bytes using shared context. (2) Fragmentation & Reassembly — splits large IPv6 datagrams into multiple 802.15.4 frames; reassembles at destination. Result: end-to-end IPv6 connectivity on constrained sensor nodes.

Q6 [3 Marks] Select and justify FOUR communication criteria you would evaluate when planning a WSN deployment for a smart agriculture field monitoring system. 🎯 Bloom's Level: L3 — APPLY 📚 Course Outcome: CO5 📖 Topic 8 — Communication Criteria

✏  Answer Key Points:

(1) Range: fields can be hundreds of metres to kilometres wide. Select sub-GHz technology (LoRa/LoRaWAN) for km-range coverage vs ZigBee for 100 m max.

(2) Power Consumption: field sensors are battery-powered with no mains access. Design for deep sleep with 30-minute sampling intervals → target 2–5 year battery life.

(3) Topology: Mesh topology preferred — if one node fails (flood, physical damage), data re-routes through neighbours. No single point of failure.

(4) Data Rate: soil moisture, temperature, and humidity readings are small payloads (<50 bytes). Low data rate (LoRa <50 Kb/s) is sufficient and allows long range + low power.

Other valid criteria: Frequency band (sub-GHz for better soil/foliage penetration), Overhead/Payload.

| Q1 ¦ L2 | Q2 ¦ L3 | Q3 ¦ L4 | Q4 ¦ L2 | Q5 ¦ L4 | Q6 ¦ L5 |
| --- | --- | --- | --- | --- | --- |
| UNDERSTAND | APPLY | ANALYSE | UNDERSTAND | ANALYSE | EVALUATE |

SECTION B — 9 Mark Questions  (Long Answer)

Q1 [9 Marks] Explain the architecture of an M2M system with a labelled diagram. Compare and contrast IoT and M2M using a table with at least FIVE points of difference. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO4 📖 Topic 1 — M2M

✏  Answer Key Points:

M2M Architecture — 3 components: (1) M2M Area Network: devices/sensors communicate using non-IP protocols (ZigBee, Modbus, PLC, 6LoWPAN). (2) Communication Network: Internet/cellular (IP-based) connecting M2M area network to application domain via M2M Gateway. (3) Application Domain: servers, dashboards, management software processing M2M data.

M2M Gateway: proxy translator — converts non-IP ↔ IP; makes each M2M node visible as a virtualised IP endpoint to the application domain.

M2M application domains: Utility metering, Healthcare monitoring, Vehicle telematics, Consumer electronics, Industrial automation.

IoT vs M2M table — 5 differences: (1) Protocol — M2M: proprietary/closed; IoT: open standards (IP/REST/MQTT). (2) Device type — M2M: machines only; IoT: any 'thing' including everyday objects. (3) Data storage — M2M: enterprise/local; IoT: cloud-scale. (4) Intelligence — M2M: limited automation; IoT: AI/analytics/predictive. (5) Scale — M2M: thousands of devices; IoT: billions. Also valid: (6) Communication — M2M: point-to-point; IoT: many-to-many. (7) Evolution — M2M is predecessor; IoT evolved from M2M.

Diagram: draw 3 labelled boxes (M2M Area Network | Gateway | Communication Network | Application Domain).

Q2 [9 Marks] Explain the architecture of SDN with its three layers and three key elements (Centralised Controller, Programmable APIs, OpenFlow). Apply SDN concepts to demonstrate how it solves the scalability and management challenges of large-scale IoT networks. 🎯 Bloom's Level: L3 — APPLY 📚 Course Outcome: CO4 📖 Topic 2 — SDN

✏  Answer Key Points:

Why SDN: limitations of conventional networks — (1) proprietary hardware with complex protocols, (2) per-device manual configuration (impossible at IoT scale), (3) limited scalability for VM-to-VM cloud traffic generated by IoT.

SDN Architecture — 3 layers: (1) Infrastructure/Data Plane — simple packet-forwarding switches/routers with no embedded control logic. (2) Control Plane — centralised SDN Controller with global network view; programs all switches. (3) Application Plane — network apps (traffic engineering, security policies, load balancing) interact via Northbound APIs.

Key Elements: (1) Centralised Controller: global view, installs flow rules in all switches. (2) APIs: Northbound (REST/JSON to apps); Southbound (OpenFlow to devices). (3) OpenFlow: open protocol; each flow entry has Match Fields + Counters + Instructions; controller adds/modifies/deletes flow entries.

OpenFlow operation: packet arrives at switch → check flow table → match found → forward per rule; no match → send to controller → controller installs new rule.

Application to IoT: (1) Scalability — controller programs thousands of IoT gateways from one place; no per-device configuration. (2) Dynamic policy — when new IoT device joins, controller installs its traffic rules instantly. (3) Traffic isolation — controller separates IoT traffic from enterprise traffic using virtual slices.

Diagram: Application Plane ↔ [N-API] ↔ Controller ↔ [S-API/OpenFlow] ↔ Switches.

Q3 [9 Marks] Analyse NFV architecture (VNF, NFVI, MANO) using the virtualised home gateway as a practical example. Compare SDN and NFV and determine whether they are competing or complementary technologies. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO4 📖 Topic 3 — NFV

✏  Answer Key Points:

NFV definition: replaces dedicated hardware network appliances with software functions (VNFs) running on standard COTS (Commercial Off-The-Shelf) servers.

NFV Architecture (ETSI): (1) VNF — Virtual Network Function; software equivalent of a hardware appliance (e.g., virtual firewall, virtual NAT, virtual router). (2) NFVI — NFV Infrastructure; physical compute + storage + network + hypervisor/container layer on which VNFs run. (3) MANO — Management & Orchestration; manages VNF lifecycle (instantiate, scale, terminate), allocates NFVI resources, manages service chains.

Practical example — Virtualised Home Gateway: Traditional: physical CPE box performs DHCP, NAT, Firewall, VoIP, IPTV. With NFV: all these functions run as VNFs in the telco cloud. Home just has a simple modem. Upgrade = software update, not hardware truck-roll.

Benefits for IoT: Cost (COTS servers cheaper than appliances), Rapid deployment (deploy new IoT gateway type in software), Elastic scaling (spin up more VNF instances when IoT traffic spikes), Multi-tenancy (isolate multiple IoT tenants on shared infrastructure).

SDN vs NFV table — 5 rows: (1) Focus: SDN=network control; NFV=network functions. (2) What it replaces: SDN=control plane hardware; NFV=dedicated appliances. (3) Key tech: SDN=OpenFlow; NFV=hypervisors/containers. (4) Where: SDN=switches; NFV=servers. (5) Standard: SDN=ONF; NFV=ETSI.

Conclusion: COMPLEMENTARY — SDN controls how packets are routed through the network; NFV determines what functions process those packets. Together they create a fully programmable, virtualised IoT infrastructure.

Q4 [9 Marks] Classify sensors using SIX criteria with examples. List and describe EIGHT types of IoT sensors with one application each. Explain FOUR key sensor performance parameters and justify their importance for IoT design. 🎯 Bloom's Level: L2 — UNDERSTAND 📚 Course Outcome: CO4 📖 Topic 4 — Sensors

✏  Answer Key Points:

Sensor definition: converts a physical quantity into a digital signal processable by a microcontroller. IoT role: INPUT device in the sense–decide–act cycle.

6 Classification criteria: (1) Energy — active (external power: ultrasonic) vs passive (self-generating: thermocouple). (2) Output — analog (thermistor) vs digital (DHT22). (3) Physical quantity — temperature, pressure, humidity, etc. (4) Detection method — contact (RTD thermometer) vs non-contact (infrared thermometer). (5) Transduction principle — resistive, capacitive, piezoelectric, optical. (6) Application domain — industrial, medical, environmental, consumer.

8 sensor types: Temperature (DHT22 — HVAC), Humidity (SHT31 — greenhouse), Pressure (BMP280 — weather station), Motion/Accelerometer (ADXL345 — wearable fitness), Light/LDR (BH1750 — smart lighting), Proximity (HC-SR04 — parking sensor), Gas (MQ-2 — LPG leak detection), Acoustic (MEMS mic — sound level monitoring).

Mnemonic: T-H-P-M-L-P-G-A = 'The Happy People Make Life Pretty Good Always'.

4 Performance parameters: (1) Accuracy — how close reading is to true value; critical for medical sensors. (2) Resolution — smallest detectable change; critical for precision industrial monitoring. (3) Response Time — how fast sensor reacts; critical for safety systems (gas leak = must detect in seconds). (4) Power Consumption — critical for battery-powered WSN nodes where low power = longer deployment life.

Conclusion: sensor selection is a tradeoff — higher accuracy usually means more power and cost.

Q5 [9 Marks] Explain WSN architecture with a labelled diagram. Analyse the differences between FFD and RFD nodes. Examine the SIX design constraints of WSN nodes and explain how data aggregation and self-organisation address these constraints. 🎯 Bloom's Level: L4 — ANALYSE 📚 Course Outcome: CO4 📖 Topic 6 — WSNs

✏  Answer Key Points:

WSN definition: network of wirelessly connected sensor nodes (motes) that cooperatively monitor physical/environmental conditions and relay data to a gateway/server.

WSN Architecture: Sensor Nodes (leaf nodes, collect and transmit data) → Cluster Heads (local aggregators) → Routers (relay nodes) → Gateway/Coordinator (bridges to Internet) → Backend Server.

FFD vs RFD table: FFD (Full Featured Device) — acts as router or coordinator, communicates with all node types, has more memory/processing, can form the network backbone. RFD (Reduced Function Device) — end device only, communicates only with its parent FFD, minimal resources, optimised for battery life, cannot route for others.

6 Design constraints: (1) Limited energy — battery-powered, cannot replace easily; (2) Limited processing — MHz-class MCU; (3) Limited memory — KB-scale RAM/Flash; (4) Limited communication range — 10–100 m per hop; (5) Limited bandwidth — 250 Kb/s (ZigBee); (6) Physical size — must be small enough for deployment.

How Data Aggregation addresses constraints: Cluster Head collects raw readings from nearby nodes → computes aggregate (average/min/max) → sends ONE value upstream instead of N values → drastically reduces transmissions → saves energy, reduces bandwidth consumption, reduces latency. This is where fog/mist computing operates.

Self-Organisation: nodes automatically discover neighbours, elect cluster heads, and form routing paths when new nodes join or existing nodes fail — no manual configuration. Essential because manual config of thousands of field nodes is impractical.

Conclusion: individual WSN nodes are weak; their collective strength compensates through redundancy, aggregation, and self-organisation.

Q6 [9 Marks] Evaluate the WSN protocol stack by examining IEEE 802.15.4, ZigBee, and 6LoWPAN at each layer. Assess FIVE communication criteria and recommend the most suitable wireless technology for a smart hospital patient monitoring system, justifying each criterion. 🎯 Bloom's Level: L5 — EVALUATE 📚 Course Outcome: CO5 📖 Topics 7 & 8 — Protocols & Communication Criteria

✏  Answer Key Points:

WSN Protocol Stack (5 layers): PHY (802.15.4) → MAC (802.15.4 CSMA-CA) → Network (6LoWPAN adaptation + RPL routing) → Transport (UDP — lightweight, no TCP overhead) → Application (CoAP/MQTT).

Why UDP not TCP: constrained nodes cannot handle TCP connection overhead, retransmission delays, and state maintenance.

IEEE 802.15.4: PHY + MAC for LR-WPANs. Specs: 2.4 GHz, 250 Kb/s data rate, 10–100 m range, 127-byte max frame, CSMA-CA access. Foundation for ZigBee and 6LoWPAN.

ZigBee: adds Network + Security + Application layers on top of 802.15.4. Mesh topology, up to 65,000 nodes, reactive routing (broadcasts route request, selects lowest hop-count path). Applications: smart home, building automation, healthcare.

6LoWPAN: adaptation layer enabling IPv6 over 802.15.4. Compresses 40-byte IPv6 header → 2 bytes. Fragments 1280-byte IPv6 MTU into 127-byte frames. Result: each patient monitoring node gets a unique global IPv6 address — doctors can query any sensor directly.

RPL: IPv6 routing protocol for LLNs. Creates a DODAG tree rooted at gateway. Handles lossy links and low-power constraints.

5 Communication Criteria for Smart Hospital: (1) Range — hospital floors are 30–50 m; ZigBee 100 m sufficient. (2) Power — wearable patient sensors must run on coin cells for a week; ZigBee sleep modes support this. (3) Latency — patient alarms need <1 second delivery; ZigBee multi-hop latency is acceptable for monitoring (not surgery control). (4) Topology — mesh preferred; if one ward node fails, data reroutes through adjacent nodes — critical for patient safety. (5) Data Rate — ECG streams ~250 bytes/reading; ZigBee 250 Kb/s is sufficient. (6) Overhead — 6LoWPAN compressed headers keep payload efficient within 127-byte frames.

Recommendation: ZigBee (IEEE 802.15.4 + 6LoWPAN) is the optimal technology for smart hospital patient monitoring — it meets range, power, topology, and latency requirements at low cost.

QUESTION BANK — BLOOM'S TAXONOMY MATRIX

| Q# | Mod | Sec | Marks | Bloom's Level | CO | Topic |
| --- | --- | --- | --- | --- | --- | --- |
| Q1 | 1 | A | 3 | L1 — Remember | CO1 | Introduction to IoT — Definitions & Characteristics |
| Q2 | 1 | A | 3 | L2 — Understand | CO1 | IoT Architectural View |
| Q3 | 1 | A | 3 | L4 — Analyse | CO2 | Physical Design — Network Layer (IPv4/IPv6) |
| Q4 | 1 | A | 3 | L2 — Understand | CO3 | Communication Models — Publish-Subscribe |
| Q5 | 1 | A | 3 | L3 — Apply | CO2 | Logical Design — Functional Blocks |
| Q6 | 1 | A | 3 | L4 — Analyse | CO3 | Physical Design — Transport Layer (TCP/UDP) |
| Q1 | 1 | B | 9 | L2 — Understand | CO2 | Physical Design — Full Protocol Stack |
| Q2 | 1 | B | 9 | L2 — Understand | CO3 | Communication Models — All 4 Models |
| Q3 | 1 | B | 9 | L3 — Apply | CO2 | Logical Design — Smart Irrigation Mapping |
| Q4 | 1 | B | 9 | L4 — Analyse | CO3 | Enabling Technologies — WSN + Cloud + Big Data |
| Q5 | 1 | B | 9 | L4 — Analyse | CO3 | Communication APIs — REST vs WebSocket |
| Q6 | 1 | B | 9 | L5 — Evaluate | CO1 | IoT Levels 1–6 — Campus Energy System |
| Q1 | 2 | A | 3 | L1 — Remember | CO4 | M2M — Definition & Gateway Role |
| Q2 | 2 | A | 3 | L4 — Analyse | CO4 | SDN vs NFV — 3 Differences |
| Q3 | 2 | A | 3 | L2 — Understand | CO4 | Sensors — Classification by 3 Criteria |
| Q4 | 2 | A | 3 | L1 — Remember | CO4 | Smart Objects — Definition & 4 Components |
| Q5 | 2 | A | 3 | L2 — Understand | CO5 | 6LoWPAN — Problem & Mechanism |
| Q6 | 2 | A | 3 | L3 — Apply | CO5 | Communication Criteria — Smart Agriculture |
| Q1 | 2 | B | 9 | L2 — Understand | CO4 | M2M Architecture + IoT vs M2M Table |
| Q2 | 2 | B | 9 | L3 — Apply | CO4 | SDN Architecture + IoT Scalability Application |
| Q3 | 2 | B | 9 | L4 — Analyse | CO4 | NFV Architecture + SDN vs NFV Comparison |
| Q4 | 2 | B | 9 | L2 — Understand | CO4 | Sensors — 6 Criteria + 8 Types + Parameters |
| Q5 | 2 | B | 9 | L4 — Analyse | CO4 | WSN — Architecture + FFD/RFD + Constraints |
| Q6 | 2 | B | 9 | L5 — Evaluate | CO5 | Protocol Stack + Criteria — Hospital Recommendation |

Bloom's Level Distribution:  L1 (REMEMBER) = 3   L2 (UNDERSTAND) = 8   L3 (APPLY) = 4   L4 (ANALYSE) = 7   L5 (EVALUATE) = 2

Total: 24 Questions (12 per Module) | 12 × 3-Mark + 12 × 9-Mark | Bloom's Levels L1–L5 | COs: CO1–CO5 | PEMRT525 | KTU S5 | JECC